Collision Checklist

碰撞事故緊急處理程序檢查表

| Ship Side-Emergency Response船端-緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| Duty officer shall:當值船副應： |  |  |
| 1. | Reduce relative speed, avoid the collision of the accommodation area and the engine room as much as possible and minimize the damage to vessel. 減低相對速度，儘可能避免住艙機艙的碰撞，使船體損害減到最小。 | ☐ |
| 2. | Call master immediately if Captain is not on the bridge and standby engine room staffs. Sound general alarm. Note collision time and press Man overboard button on ECDIS to record position. Record the angle and heading at the time of collision. 立刻呼叫船長，如果其不在駕駛台。機艙人員備便。鳴放全船警報器。記下碰撞時間與使用”人員落水”功能以記錄碰撞地點。記下碰撞當時的碰撞角與Heading。 | ☐ |
| 2. | Verify which part of ownship collided with other vessels by visual and ECDIS tracks etc., check any possibility / happening of explosion/fire/sinking. 利用目視與電子海圖等航儀，確認本船與他船的碰撞部位，檢查是否有爆炸/起火/沉沒的情勢與可能。 | ☐ |
| 3. | If ownship stuck in opposite vessel’s midship section or stern, stop main engine immediately and check speed over ground of both vessels are the same.(do not use emergency astern to separate both ships unless no flooding danger is warranted.) 如果已經插入對方船舯或船尾，立即停車並檢查兩船的對地速度，是否一致。(不可使用緊急倒車將兩船分離，除非能確保兩船不會有大量進水危險)。 | ☐ |
| 4. | If ownship had been collided in midship section or stern, check speed and course over ground of both vessels to keep the same as possible.(contact opposite vessel to coordinate both vessels actions and intention to avoid further expansion of collision losses). 如果本船船舯或船艉被撞，確認兩船的對地速度與航向，是否一致?(聯絡對方船隻，協調兩船的意圖與行動，避免碰撞損失進一步擴大)。 | ☐ |
| 5. | Broadcast to all crew to announce in “Collision” muster and roll call to make sure any personnel injury or fatality. 廣播全船就”撞船”部署表並點名確認是否有人員傷亡。 | ☐ |
| 6. | Before Master come on bridge, duty officer can continue using VHF to alert the ships in the vicinity, inform VTS or Coastal state authority 船長上駕駛台前，當值船副可繼續：利用VHF廣播警告附近來往船隻，戒備或請求支援。 |  |
| 7. | Display the related lights and sound signal according to COLREG rules. 依照COLREG RULE 顯示相關燈號及聲號。 | ☐ |
| 8. | Make notes of the time, position and prevailing weather, sea condition of the collision for company internal reference which should be correct and in details. 記錄事故發生之時間、船位及當時天氣海況等相關資料記錄在個人筆記本，供公司內部參考之用，務必詳盡與正確。 | ☐ |
| 9. | Keep related records of bell book, course recorder and VDR (immediately download if possible), CCTV, etc. in good order. 將相關記錄器如俥鐘紀錄器 / 航向記錄器 / VDR(如果可能，立即download) / CCTV等相關法定文件與記錄，確實保存。 | ☐ |
| 10. | Inform collided part of ownship to C/O to check damage condition and deploy damage control station if no personal safety concerns. 如無人員傷亡的顧慮，將發生與他船碰撞之部位記下，以通知大副查看損害狀況及進行損害管制。 |  |
| Take action should by Master船長應採取之行動： |  |  |
| 1. | He shall ascertain own ship and other ship: collided / damaged position and take following emergency measures: Verify that watertight and automatic fire doors remain closed and undamaged. 應立即確認本輪與對方輪發生碰撞部位與損害發生位置，並採取下列緊急措施： 確認水密及自動防火門保持關閉且沒有損壞。 | ☐ |
| 2. | Deal with immediate danger - take the ship out of dangerous situation first. 處理立即會產生之危險-先將船舶脫離危險： |  |
|  | - If vessel traffic is dense, alert passing vessels by all available means like VHF, whistle and AIS status. 若碰撞地點船舶密集，應利用任何可行之方式(霧號/VHF/AIS等)及時警告他船。 | ☐ |
|  | - If any shallow water or obstruction nearby the collision position might endanger the safety of ownship, master should maneuver own ship out of the danger area if this action will not cause excess pressure and water ingress in damage ship’s hull. 碰撞地點有任何危及船舶安全之淺灘或礙航物，應在不會造成本船受損部位的大量水壓與進水可能下，運轉本船脫離危險。 | ☐ |
|  | - Instruct C/O / C/E and their damage control team to thoroughly inspect whole ship damage condition and make necessary emergent repair. 下令大副及輪機長立即徹底檢查全船受損狀况，並作緊急修補。 | ☐ |
| 3. | Muster Damage Control Station, confirm all heads are counted. If anyone missing or injury due to the collision, immediately deploy search and rescue station or medical first aid. Direct damage control team to ascertain collided and damaged condition(including underwater void space and any tank broken) with immediate effect to ship’s structure, stability and buoyancy, any oil leaking and pollution, any fire or explosion, then apply the emergency measures to control the damage and casualty, report and seek company’s timely assistance; immediately organize fire-fighting and medical team to deal with the fire and explosion, seek shore side’s emergent assistance, If necessary. 召集損害管制部署(Damage Control Station)，確認本輪各部署組人員就位、是否有人員因碰撞事故傷亡與失蹤，如有人員傷亡或失蹤，應隨即實施急救與人員搜索；指揮損害管制組前往確認碰撞點與因碰撞導致之損害部位(水下空艙、壓水艙或油艙是否破損)、評估及確認水線下船體受損程度與各艙間進水程度，對於本輪目前水密性與穩定度之影響、是否有油料因破損溢出、有無因碰撞造成之貨物爆炸與失火；對船舶水密、浮力及穩定度，作迅速有效的緊急處理及管制，與公司有關部門連絡，要求提供適切的援助。對爆炸失火應立即組成滅火組及醫療組，有必要時須請岸上緊急支援。 | ☐ |
| 4. | According to SOPEP, from the information provided by damage control team, appraise the extent of the damage and immediately proceed the leaking-block, confirm other tanks watertight to avoid the damage spreading; any oil spill due to breakage of the tank, stop the pollution according to HSE PR-08 Emergency Plan: Pollution Prevention; and prevent fire and explosion from oil spill; any spill overboard, report to parties concerned as per SOPEP’s procedure and format. 依據船舶油污染應急計畫(SOPEP)，以損害管制組回報之資訊，評估損害程度並指揮損害管制組進行堵漏、確保其他艙間水密性，以防止損害擴大；有油料因破損溢出時，依照HSE PR-08 Emergency Plan: Pollution Prevention進行油污染防阻，並避免因溢油產生之爆炸與火災；若有任何油污漏洩入海，須照SOPEP程序通報有關單位。 | ☐ |
| 5. | Once oil spill overboard若油污漏洩入海 |  |
|  | - Make risk assessment for oil spill overboard & confirm the oil pollution condition and take photos. 執行油污漏洩入海風險評估，確認油污污染狀況並拍攝現場照片。 | ☐ |
|  | - Muster all crew for the oil pollution control. 召集船員執行防油污污染部署。 | ☐ |
|  | - Dispatched an initial oil spills report to TPE / Maritech & local agent and coastal state related Dept. 發送初步油汙汙染報告至海技部及與當地代理行與港口國相關單位。 | ☐ |
|  | - PIC of Legal & INS Dept. to inform P & I Club and Hull insurance to arrange oil pollution clear operation. 法保PIC通知P&I Club 履行保險契約準備清理油汙汙染作業。 | ☐ |
|  | - Inform Agent to arrange for any emergency assistance 通知代理，準備任何緊急情況之協助。 | ☐ |
| 6. | Contact opponent ship and confirm both ships basic data (including ship’s name, port of registry, IMO No., last port, destination, owner, management Co., etc.), initial damage condition( photo and record), if any danger of immediate sinking which need immediate rescue. If any ship suffered serious damage which might sink, another ship after ascertaining himself no immediate danger shall render her rescue to other ship. 與對方船聯繫確認雙方船舶基本資料(船名、註冊港、IMO編號、上一港、目的港、船東與船舶管理人資料等)、初步受損情況(拍攝照片並記錄)、有無立即沈沒危險與是否極需救援，若有一船遭受嚴重損害，有沈沒之虞時，另一船應在確認自身無立即危險時，施予人命救援。 | ☐ |
| 7. | In compliance with SMM PR-08, any accident happened, notify Maritech Division immediately referring to “Emergency Contact List” and send ER for timely assistance. 依安全管理手冊PR-08規定，當事件發生時，船長應參考“緊急聯絡電話表＂立即通知海技部及發送ER要求適時援助。 | ☐ |
| 8. | At early stage of the emergency, Master shall notify costal state or flag state. The notification shall include accident type and assistance required. 緊急事件初期，船長應通知最近的沿岸國及船旗國當局，通報內容包括事件類型及需要何種協助。 | ☐ |
| 9. | Execute risk assessment & ensure the following: 執行風險評估並確認下列事項： |  |
|  | - Muster & deploy all crew in damage control station. 是否已令所有船員依照規定進入損害控管部屬。 | ☐ |
|  | - Send damage control team to examine around the ship, Switch on deck lighting at night. 已指派損害控管隊去檢查船舶周遭情況，夜間開啟甲板燈。 | ☐ |
|  | - Sound & confirm all tanks and bilge wells condition 已測量並確認所有油艙與汙水艙狀況。 | ☐ |
|  | - Confirm situation of engine. 已確認主機情況。 | ☐ |
|  | - Check condition of all navigation equipment. 已檢查所有航海儀器之情況。 | ☐ |
|  | - Download VDR, CCTV & ECDIS, kept related record, bell book/Log book/Chart /Course record/Sounding record, etc. 已下載VDR、CCTV及ECDIS並保留任何有關的紀錄如航行日誌、海圖、航項紀錄、量水紀錄等。 | ☐ |
|  | - Ship’s stress force and stability assessment. 已執行船體壓力與穩度評估。 | ☐ |
|  | - Correct AIS status to “Not Under Command” to warn other ships to keep clear. 已更正AIS的航行狀態為”NUC”，以警告附近船舶保持距離。 | ☐ |
|  | - Offer assistance to other ship by VHF. 已VHF聯絡他船，是否需要協助與救助。 | ☐ |
|  | - Check does hull damage caused oil spill overboard, activate SOPEP. 確認是否因為船殼受損而造成油污污染，並啟動SOPEP程序。 | ☐ |
|  | - Has injured or overboarded crew and/or in opposite ship? Is it necessary to rescue or first aid (CPR)? Any Fire happened? 確認本輪與他船是否有人員受傷或落海，有無需要執行救助或急救(包括CPR)? 和是否有任何火警發生? | ☐ |
|  | - Sent ER, Contacted PIC and Coast Guard for further assistance, prepare lifeboat station if necessary. 確認已發送ER報、通知PIC以及相關單位請求更多幫助，有需要時，準備棄船部署。 | ☐ |
|  | - Sent Claim letter to opposite ship and made Sea protest and ISM report, took photos and diagram about the collision. 確認已寄送求償信給對方船並製作ISM相關報表，製作關於此碰撞事件的相關照片報告與圖表。 | ☐ |
|  | - Stability should be assessed when taking any action. 在採取任何動作之前是否都已做船舶的穩定度評估。 | ☐ |
| After the collision, Master shall pay attention to collect the evidences and make the reports as the following: 碰撞事故發生後，船長應注意收集之証據及繕製之報告如下： |  |  |
| 1. | As per HSE manual PR-08, any accident, required report forms are FM-0803-2, FM-0803-7, statement of fact and sea protest. 依據公司安全管理手冊所規定之意外事故表格繕製FM-0803-2、FM-0803-7、以及STATEMENT OF FACT、SEA PROTEST呈報公司； | ☐ |
| 2. | Shipboard personnel shall have coherent statement through good communication; only the fact to be recorded, do not make inference without any ground. If time is allowable, Captain shall obtain the signature from liable parties’ commitment in liability of the collision. 船上相關人員對碰撞事故應有溝通，有一致之陳述，且船上一切記載限發生之事實記錄，不可作任何推斷或認定，如時間場合許可，應儘量取得對方資料，乃至碰撞責任承諾書之簽註。 | ☐ |
| 3. | Master shall ensure all the fixes on using charts, no forgery. 使用之海圖船長務必確定海圖上之船位，避免塗改。 | ☐ |
| 4. | Telegraph or other RPM output recording. 車鐘記錄器或其他主機引擎輸出有關運轉之記錄器 | ☐ |
| 5. | Confirm that bridge clock time match with engine room’s and other auto recording. 駕駛台上船鐘與機艙是否正確一致及其他自動記錄器之確認。 | ☐ |
| 6. | Course recorder’s time and course at the moment of collision and any weather forecast. 航向記錄器所指示之碰撞時間，航向指示，氣象指示。 | ☐ |
| 7. | Take good care of CCTV & VDR’s recording( as VDR will continue recording for 12 hours, after Master contacting with the authority concerned, stop the recording and take out CF Card for later investigation) 妥善保存CCTV及VDR之記錄(因VDR會連續紀錄12HRS，在船長與主管機關聯繫後，再將記錄器停止，並取出CF CARD以供後續調查使用)。 | ☐ |
| 8. | Photo or CCTV recording till the accident. 事故經過的拍攝影片及照相； | ☐ |
| 9. | Discard any unnecessary draft/notes to avoid trouble or confusion. 不要的紙頭如計算稿紙/交待記錄等最好丟棄，避免造成或受到困擾； | ☐ |
| 10. | The statement in deck log and engine log shall be coherent (coordinate with the Company to record only the favorable part). 在甲板DECK LOG 及ENG LOG記錄應協調一致(事先與公司協調記載有利部份)。 | ☐ |
| 11. | The description of collision accident shall include all witnesses and duty persons, statement of fact shall attach photos as much as possible, the information shall include: 碰撞事故過程之描述，包括所有目擊及當事人之報告，STATEMENT OF FACT儘可能附上事故現場照片，收集呈報公司之資料証據包括： |  |
|  | - The actual time of collision/see if it is matched with engine time; record any difference. 碰撞之確實時間/注意與機艙時間是否一致，任何誤差均須註明。 | ☐ |
|  | - Ship’s position/course/speed/rpm and course recorder reading at time of collision. 碰撞當時之船位/航向/航速/轉速及碰撞時航向記錄器所顯示之記錄。 | ☐ |
|  | - Engine/steering maneuvering/time and condition after collision. 碰撞後用車/用舵/時間及狀況。 | ☐ |
|  | - Estimated prevailing visibility/the time of signal heard or seen and its relative position. 預估能見度/註明看到或聽到信號時間及相關船位。 | ☐ |
|  | - Opponent ship’s maneuvering situation/bearing/distance before collision. 碰撞前見到它船運轉情形/方位/距離。 | ☐ |
|  | - Angle of blow and ship’s speed at the time of collision. 碰撞角度(Angle of blow)及船速。 | ☐ |
|  | - The prevailing weather/wind direction, wind speed/direction of wave and height/swell direction and height/visibility/received weather forecast. 當時天氣狀況/風向、風速/浪之方向與高度/湧之方向與高度/能見度/收受之天氣預報。 | ☐ |
|  | - The condition of tide and current. 潮、流之情形。 | ☐ |
|  | - Bridge composition before collision or immediately on bridge upon collision. 碰撞前在駕駛台上或立即上駕駛台之人員配置。 | ☐ |
|  | - Positioning fix system. 使用之定位系統。 | ☐ |
|  | - The time of first observing opponent ship/the observing method/bearing and distance/prevailing situation/displayed course/actual time. 首次觀測對方之船舶/利用的方法/距離與方位/觀察到的亮度與型式/當時的狀況/顯示的航向/時間。 | ☐ |
|  | - Own ship’s course/speed/action taken at the time of first observing opponent ship. 首次觀測對方船舶時本船之航向/航速/採取的行動。 | ☐ |
|  | - Constant and regular observation/time/distance/bearing. 持續的觀測/時間/距離/方位。 | ☐ |
|  | - The initial visual contact with opponent ship/time/distance and method/the light observation/displayed course. 最初目視船舶之情況/時間/距離與方法/觀測的灯光/當時之狀況/顯示的航向。 | ☐ |
|  | - Multiple drawing to opponent ship (the formal drawing on ARPA radar). 多段式對他船之繪圖(避碰雷達上正式繪圖)。 | ☐ |
|  | - Both ships recording of response action from the beginning till collision (including operation of M/E). 雙方船舶從開始至碰撞發生之時間內反應動作之記錄(含主機操作)。 | ☐ |
|  | - When released fog signal or signal heard and characteristics of operation. 何時施放及聽到之音響信號和操作特性。 | ☐ |
|  | - The communication details of both ships before and after collision. 碰撞前後雙方船舶彼此間之聯絡細節。 | ☐ |
|  | - The other passing ship’s name in the vicinity at time of collision. 碰撞發生時鄰近他船的船名。 | ☐ |
|  | - The damage condition of ship and cargo on both ships, with attached photos, if possible. 雙方船貨損壞情況，儘可能附照片。 | ☐ |
|  | - Opponent ship’s name/nationality/owner/charterer/last port/destination. 他船船名/國籍/船東/傭船人/啟航港/目的港。 | ☐ |
|  | - Opponent ship’s insurance Co. /P & I, if need any rescue/repair. 他船保險公司/P & I，是否須救助/修理。 | ☐ |
|  | - Deck, engine bell book, telegraph record. 甲板、機艙之車鐘記錄簿，車鐘記錄圖紙。 | ☐ |
|  | - Course recorder record. 航向記錄。 | ☐ |
|  | - Used chart at the time. 當時使用海圖。 | ☐ |
|  | - Other records relating to maneuverability, compass error adjustment. 其他有關操船能力，羅經誤差校準等記錄。 | ☐ |
|  | - Master’s sea protest shall be endorsed (only state the part favorable to the company). 船長之海事報告(Sea Protest)，須加以簽證(只記錄對我有利之部份)。 | ☐ |
|  | - If nay salvage, shall record the details of salvor’s operation and own ship’s coordinating action in order for rewards basis; if arrangement of rescue and repair, fitness for sailing will be impaired, seaworthiness certificate shall be verified and secured by Class to issue “fit to proceed” before sailing to sea. 假如有SALVAGE時，應將SALVOR所作所為詳作記錄，以及本船所配合進行動作列入記錄，以作為日後記算酬勞金之依據，若有安排修理救助，適航性必受損，在未獲得SEAWORTHINESS CERTIFICATE前移動航行將失去保險之COVER，故一定要取得適航証明(由船級協會CLASS SOCIETY SURVEYOR簽發)，完成FIT TO PROCEED如此才能確保保險有效。 | ☐ |
|  | - If arranged surveyor for damage inspection, shall prepare GA and shell expansion drawing; 碰撞後如有安排驗船師SURVEYOR登輪作損壞檢定，應備妥GA及SHELL EXPANSION圖。 | ☐ |
|  | - After collision shall exchange Claim Notice soonest and remark “ACKNOWLEDGED RECEIPT ONLY WITHOUT PREJUDICE TO LIABILITY ". 碰撞後應儘速交換CLAIM NOTICE，收到對方CLAIM NOTICE必須加註 "ACKNOWLEDGED RECEIPT ONLY WITHOUT PREJUDICE TO LIABILITY "。 | ☐ |
|  | - The likely boarding parties after collision; 碰撞事故發生後可能上船之相關人員分 : |  |
|  | - CLASS SURVEYOR: to assess ship’s seaworthiness: it’s better to be. 用於判定船體適航性、最好由公司決定是否僱請。 | ☐ |
|  | - INSURANCE SURVEYOR: to evaluate the damage, it’s separated to hull surveyor and P & I surveyor; no assessment for liability. 作損傷鑑定、可分船體 SURVEYOR 及 P & I SURVEYOR、不作責任判斷用。 | ☐ |
|  | - Lawyer: shall identify for whom he represents. 律師 (LAWYOR, SOLICITOR, ATTORNEY, ADVOCATE均是)：須認清代表何方。 | ☐ |
|  | - Salvage Superintendent or Salvage Surveyor, etc.； 救助負責人或檢查員等。 | ☐ |
|  | - Coast Guard or Port authority investigator. 海岸巡防隊或港務局/海事處調查人員。 | ☐ |

| Shore Side-Emergency Response岸端緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目內容 |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報課級主管、DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  | ☐ |
| 1. | Maritech Emergency response team must confirm the following as soon as receiving report from Captain: 海技緊急應變小組第一時間確認船端回報下列事項： | ☐ |
|  | - The condition of ship’s damage from the collision. 船體撞擊受損情況。 | ☐ |
|  | - Have damage been controlled? 是否受損已被控制？ | ☐ |
|  | - Have all machine, equipments and tanks condition been checked & confirmed? 所有機器、設備及tank情況是否已檢查並確認? | ☐ |
|  | - Whether it is necessary to arrange under water survey? 是否需要安排水下作業檢查? | ☐ |
|  | - Whether it is necessary to notify the Class surveyor for on-board inspection是否需要通知Class驗船師上船檢查? | ☐ |
| 2. | Any personal injury? Any oil pollution? Container leaking? Fire? If need shore side urgent assistance? 是否有人員受傷? 是否有油污染?是否有貨櫃漏洩?是否發生火災?是否需岸端緊急援助? | ☐ |
| 3. | Have exchanged information with opponent vessel? The damage condition of other vessel? Did she need urgent assistance? 是否已與他船交換資訊?他船受損情況?是否需要我船援助? | ☐ |
| 4. | Have photos at scene (including damage condition) been taken & ready to send back? VDR, CCTV & ECDIS been downloaded? 現場照片(包括碰撞受傷情況)是否已拍攝並準備寄回? VDR、CCTV及ECDIS是否已下載? | ☐ |
| 5. | Have the risk assessment for overall emergency response & ship’s stability been effectively executed? 對所有緊急處置及船舶穩定性的風險評估是否已有效執行？ | ☐ |
| 6. | Have notified the nearest coasting country & flag state? 是否已通知沿岸最近管轄國及船旗國？ | ☐ |
| 7. | Have sent the following initial accident reports according to SMM PR-0802 in safety situation: 是否已照安全管理手冊PR-0802程序，在安全無虞下，寄回下列初步事件報告： | ☐ |
|  | - Statement of fact(in Chinese & English) with photos | ☐ |
|  | - Sea Protest | ☐ |
|  | - Claim Letter | ☐ |
|  | - FM-0803-2, FM-0803-7 | ☐ |
|  | - Drawing of collision(including angle of blow) | ☐ |
|  | - Report to flag state | ☐ |
|  | - Copy of log book, bell book, telegraph record, course recorder, etc. | ☐ |
| 8. | PIC continues closely contacting with ship’s captain & provides necessary emergent assistance until the fleet vessel is safe & the case is over. 海技PIC持續與船端緊密連絡並協助緊急處置，直到屬輪平安事件結束為止。 | ☐ |