Touch Bottom Accident (Oil Spill overboard included)

觸底事故緊急處理程序檢查表

| Ship Side-Emergency Response船端-緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1. | When feeling the ship abnormal vibrating, but the speed does not reduce to zero, the ship could be touching bottom, then reduce speed & inform the Master immediately. 當船舶發生異常震動，船速未歸零，則船可能觸底；將船舶減速並通知船長。 | ☐ |
| 2. | Check echo sounder and make record of local time & place. 檢查測深儀並紀錄當時時間與地點。 | ☐ |
| 3. | Record ship position and mark it on chart. 紀錄當時船位並標示在海圖上。 | ☐ |
| 4. | Check chart depth & sea bed around the touch bottom position. 檢查海圖上區域之水深及底質。 | ☐ |
| 5. | Check if any other immediate dangerous situation (like fire or flooding). 確認是否有其他立即危險(如起火或進水)。 | ☐ |
| As soon as the Master on bridge 船長上駕駛台後： |  |  |
| 1. | He shall ascertain own ship damaged position and take following emergency measures: 應立即確認本輪損害發生位置，並採取下列緊急措施： | ☐ |
| 2. | Injury, immediately deploy search and rescue station or medical first aid. Direct damage control team to ascertain the touch bottom point and damage condition (including underwater part and any tank broken and emerged) with immediate effect to ship’s structure, stability and buoyancy, any oil leaking and pollution, any fire or explosion, then apply the emergency measures to control the damage and casualty, report and seek company’s timely assistance; immediately organize fire-fighting and medical team to deal with the fire and explosion, seek shore side’s emergent assistance if necessary. 召集損害管制部署(Damage Control Station)，確認本輪各部署組人員就位情形、人員是否傷亡與失蹤，如有人員傷亡或失蹤，應隨即實施急救與人員搜索；指揮損害管制組前往確認觸底點與可能之損害部位（船體結構、壓水艙或油艙因此破損）、評估及確認水線下船體受損程度與各艙間進水程度對於本輪目前水密性與穩定度之影響、是否有油料因破損溢出、有無因碰撞造成之貨物爆炸與失火；對船舶水密、浮力及穩定度作迅速有效的緊急處理及管制，必要時迅與公司有關部門連絡以提供適切的援助。對爆炸失火應立即組成滅火組及醫療組，有必要時須請岸上緊急支援。 | ☐ |
| 3. | After Master judged the ship is touching, ensure M/E has already stopped at safe place. 船長判斷船舶已觸底後，確認主機已停車在安全位置。 | ☐ |
| 4. | Sound general emergency alarm and broadcast this case to all crew at once 廣播全船警報並通告全體船員本輪觸底情況。 | ☐ |
| 5. | Muster all crew in ship damage control station to proceed following measures. 應立刻召集全體船員就損害管制部署，並準備下列事項。 | ☐ |
| 6. | In compliance with SMM PR-08, any accident happened, notify Maritech Division immediately referring to “Emergency Contact List” and send ER for providing timely assistance. 依安全管理手冊PR-08規定，當事件發生時，船長應參考“緊急聯絡電話表＂立即通知海技部及發ER，以利提供適時的援助。 | ☐ |
| 7. | At early stage of the emergency, Master shall notify costal state or flag state, the notification shall include the type of the accident and what kind of assistance required. 緊急事件初期，船長應通知最近的沿岸國及船旗國當局，通報內容包括事件類型及需要何種協助。 | ☐ |
| 8. | Execute risk assessment & ensure the following: 執行風險評估並確認下列事項： |  |
|  | - Mustered & deployed all crew in damage control station. 是否已令所有船員依照規定進入損害控管部屬。 | ☐ |
|  | - Closed watertight doors and automatic fire door. 水密及自動防火門已關閉。 | ☐ |
|  | - Sent damage control team and examined around the ship, Switched on all deck lighting at night. 已指派損害控管隊去檢查船舶周遭情況，夜間開啟甲板燈。 | ☐ |
|  | - Sent 3/O sounded depth around the ship, 2/O confirmed the tide condition and object on the chart. 指派三副測量並確認六面水深，二副確認觸底當時的潮水與海圖附近礙航塢。 | ☐ |
|  | - Assigned the damage control team and inspected the ship’s hull & structure around the ship blocked the leakage and made immediate repair if any. 指派船舶損害控制小組檢查船體與結構有無受損；如果有任何漏洩，堵漏並做立即必要的修理。 | ☐ |
|  | - Sounded & confirmed all tanks and bilge wells condition 已測量並確認所有油艙與汙水艙狀況。 |  |
|  | - Confirmed the situation of engine. 已確認主機情況。 | ☐ |
|  | - Checked the condition of all navigation equipment. 已檢查所有航海儀器之情況。 | ☐ |
|  | - Downloaded VDR, CCTV, kept related record such as bell book/telegraph record/Log book/Chart/Course record/Sounding record, etc. 已下載VDR、CCTV及ECDIS並保留任何有關的紀錄如車鐘簿/車鐘記錄/航行日誌、海圖、航向紀錄、量水紀錄等。 | ☐ |
|  | - Ship’s stress force and stability assessment. 已執行船體壓力與穩度評估。 | ☐ |
|  | - Sent radio warnings to ship’s in the vicinity to keep clear. 已以無線電通知附近可見船舶保持距離。 | ☐ |
|  | - Checked if hull damage causing oil spill overboard, activated SOPEP. 確認是否因為船殼受損而造成油污染，並啟動SOPEP程序。 | ☐ |
|  | - The injuries or overboard of any crew and any Fire happened. Is it necessary to execute rescue, first aid including CPR? 確認本輪是否有人員受傷或落海和是否有任何火警發生。有無需要執行救助或急救(包括CPR)? | ☐ |
|  | - Sent ER, Contacted with PIC and Coast Guard for further assistance, prepared the abandon ship procedure (in case if necessary). 確認已發送ER報、通知PIC以及相關單位請求進一步協幫助，如果有需要情況下則準備棄船部署。 | ☐ |
|  | - Ship stability should be assessed when taking any action. 在採取任何動作之前是否已做船舶的穩定度評估。 | ☐ |
|  | - Determined which way shallow water lies 確認附近淺攤區域與及方位。 | ☐ |
|  | - Determined the nature of the seabed. 確認附近地點之海底底質；視沙泥地或礁石地採取不同的對策。 | ☐ |
|  | - Obtained information on local current and tides, particularly details of the rise and fall of the tide. 計算當地潮流與潮差，確認高低潮時間。 | ☐ |
|  | - Record tide, ship position, course, local time to find reason of touch bottom 將潮水、船位、航向、當時時間標記在海圖上，確認分析觸底原因。 | ☐ |
| Once oil spill overboard若油污漏洩入海 |  |  |
| 1. | According to SOPEP, from the information provided by damage control team, appraise the extent of the damage and immediately proceed the leaking-block, confirm other tanks watertight to avoid the damage spreading; any oil spill due to the breakage of the tank, stop the pollution according to SMM PR-08 Emergency Plan: Pollution Prevention; and prevent fire and explosion from oil spill; any spill overboard, report to parties concerned as per SOPEP’s procedure and format. 依據船舶油污染應急計畫(SOPEP)，以損害管制組回報之資訊，評估損害程度並指揮損害管制組進行堵漏、確保其他艙間水密性以防止損害擴大；有油料因破損溢出時，依照SMM PR-08 Emergency Plan: Pollution Prevention進行油污染防阻，並避免因溢油產生之爆炸與火災；若有任何油污漏洩入海，須照SOPEP程序通報有關單位。 |  |
|  | - Make risk assessment for oil spill overboard & confirm the oil pollution condition and take photos. 執行油污漏洩入海風險評估，確認油污污染狀況並拍攝現場照片。 | ☐ |
|  | - Muster all crew for the oil pollution control. 召集船員執行防油污污染部署。 | ☐ |
|  | - Dispatched an initial oil spills report to TPE / Maritech & local related Dept. 發送初步油汙汙染報告至海技課及與當地相關單位。 | ☐ |
|  | - PIC of Legal & INS Dept. to be informed to P & I Club and Hull insurance to arrange oil pollution clear operation. 法保PIC通知P&I Club 履行保險契約準備清理油汙汙染作業。 | ☐ |
|  | - Inform Agent to arrange for any emergency assistance 通知代理，準備任何緊急情況之協助。 | ☐ |
| After the case, Master shall pay some special attention to collect the evidences and make the reports as the following: 事故發生後，船長應特別注意收集之証據及繕製之報告如下: |  |  |
| 1. | As per SMM PR-08, any accident, the required report forms are FM-0803-6, FM-0803-7, statement of fact & sea protest. 依據公司安全管理手冊所規定之意外事故所規定表格繕製事故報告(FM-0803-6)、意外事故調查報告(FM-0803-7)以及STATEMENT OF FACT、SEA PROTEST呈報公司； | ☐ |
| 2. | All charting work including the fixes before the accident. 所有持續到事故當時的駕駛台海圖作業，包括事故前繪於海圖上的船位； | ☐ |
| 3. | Master shall ensure all the fixes on using charts, no forgery. 船長務必確定海圖上之船位，避免塗改； | ☐ |
| 4. | Telegraph or other RPM output recording. 車鐘記錄器或其他主機引擎輸出有関運轉之記錄器； | ☐ |
| 5. | Confirm that bridge clock time match with engine room’s and other auto recording. 駕駛台上船鐘與機艙是否正確一致及其他自動記錄器之確認； | ☐ |
| 6. | Course recorder’s time and course at the moment of grounding and any weather forecast. 航向記錄器所指示之失火淺時間，航向指示，氣象指示； | ☐ |
| 7. | Take good care of VDR & CCTV’s recording( as VDR will continue recording for 12 hours, after Master contacting with the authority concerned, stop the recording and take out CF Card for later investigation) 妥善保存VDR、CCTV之記錄(因VDR會連續紀錄12HRS，在船長與主管機關聯繫後，再將記錄器停止，並取出CF CARD以供後續調查使用)。 | ☐ |
| 8. | The position fixed by radar operator. 雷達操作者所做的船位記錄； | ☐ |
| 9. | Video or photos for the accident. 事故經過的拍攝影片及照相； | ☐ |
| 10. | Toss any unnecessary draft/notes to avoid trouble or confusion. 不要的紙頭如計算稿紙/交待記錄等最好丟棄，避免造成或受到困擾； | ☐ |
| 11. | The statement in deck log and engine log shall be coherent(coordinate with the Company to record only the favorable part). 在甲板DECK LOG 及ENG LOG記錄應協調一致； | ☐ |
|  | The description of touch bottom accident shall include all witnesses and duty persons, statement of fact shall attach photos as much as possible, the information shall include: 觸底事故過程之描述，包括所有目擊及當事人之報告，STATEMENT OF FACT儘可能附上事故現場照片，收集呈報公司之資料証據包括： |  |
|  | - The actual time of touch bottom/see if it is matched with engine time; record any difference; 觸底之確實時間/注意與機艙時間是否一致，任何誤差均須註明； | ☐ |
|  | - Ship’s position/course/speed/rpm and course recorder reading at time of touch bottom; 觸底當時之船位/航向/航速/轉速及觸底時航向記錄器所顯示之記錄； | ☐ |
|  | - Engine/steering maneuvering/time and condition after touch bottom; 觸底後用車/用舵/時間及狀況； | ☐ |
|  | - Estimated prevailing visibility/the time of signal heard or seen and its relative position; 預估能見度/註明看到或聽到信號時間及相關船位； | ☐ |
|  | - The prevailing weather/wind direction, wind speed/direction of wave and height/swell direction and height/visibility/received weather forecast; 當時天氣狀況/風向、風速/浪之方向與高度/湧之方向與高度/能見度/收受之天氣預報； | ☐ |
|  | - The condition of tide and current; 潮、流之情形； | ☐ |
|  | - Bridge composition before touch bottom or immediately on bridge upon touch bottom; 觸底前在駕駛台上或立即上駕駛台之人員配置； | ☐ |
|  | - The other passing ship’s name in the vicinity at time of touch bottom; 觸底發生時鄰近他船的船名； | ☐ |
|  | - The damage condition of ship and cargo on both ships, with attached photos, if possible; 船貨損壞情況，儘可能附照片； | ☐ |
|  | - Deck, engine bell book, telegraph record; 甲板，機艙之車鐘記錄簿，車鐘記錄圖紙； | ☐ |
|  | - Course recorder record; 航向記錄； | ☐ |
|  | - Used chart at the time; 當時使用海圖； | ☐ |
|  | - Other records relating to maneuverability, compass error adjustment; 其他有関操船能力，羅經誤差校準等記錄。 | ☐ |
|  | - Master’s sea protest shall be endorsed(only state the part favorable to the company); 船長之海事報告(Sea Protest)，須加以簽證； | ☐ |
|  | - If any salvage, shall record the details of salvor’s operation and own ship’s coordinating action in order for rewards basis; if arrangement of rescue and repair, fitness for sailing will be impaired, seaworthiness certificate shall be verified and secured by Class to issue “fit to proceed” before sailing to sea. 假如有SALVAGE時，應將SALVOR所作所為詳作記錄，以及本船所配合進行動作列入記錄，以作為日後記算酬勞金之依據，若有安排修理救助，適航性必受損，在未獲得SEAWORTHINESS CERTIFICATE前移動航行將失去保險之COVER，故一定要取得適航証明(由船級協會CLASS SOCIETY SURVEYOR簽發)，完成FIT TO PROCEED如此才能確保保險有效。 | ☐ |
|  | - If arranged surveyor for damage inspection, shall prepare GA and shell expansion drawing; 觸底後如有安排驗船師SURVEYOR登輪作損壞檢定，應備妥GA及SHELL EXPANSION圖。 | ☐ |
|  | - The likely boarding parties after touch bottom; 觸底事故發生後可能上船之相關人員分 : | ☐ |
|  | - CLASS SURVEYOR : to assess ship’s seaworthiness: it’s better to be arranged by the Company. 用於判定船體適航性、最好由公司決定是否僱請； | ☐ |
|  | - INSURANCE SURVEYOR : to evaluate the damage, it’s separated to hull surveyor and P & I surveyor; no assessment for liability. 作損傷鑑定、可分船體 SURVEYOR 及 P & I SURVEYOR、不作責任判斷用； | ☐ |
|  | - Lawyer: shall identify for whom he represents. 律師 (LAWYOR, SOLICITOR, ATTORNEY, ADVOCATE均是)：須認清代表何方； | ☐ |
|  | - Coast Guard or Port authority investigator. 海岸巡防隊或港務局/海事處調查人員。 | ☐ |
|  | - Report to flag state in 24 HR. 事發24小時內向船旗國回報。 | ☐ |
|  | - All reports must be checked by company. 所有報告內容都須經過公司審議。 | ☐ |

| Shore Side-Emergency Response岸端緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目內容 |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  | ☐ |
| 1. | PIC must confirm the following as soon as receiving report from Captain: PIC第一時間確認船端回報下列事項： | ☐ |
|  | - the condition of ship’s damage from the touch bottom 船體觸底受損情況 | ☐ |
|  | - Have damage been controlled? 是否受損已被控制？ | ☐ |
|  | - Ship leakage blocked & repair situation? 船上堵漏修理情況？ | ☐ |
|  | - Have all machine, equipments and tanks condition been checked & confirmed? 所有機器、設備及tank情況是否已檢查並確認？ | ☐ |
| 2. | Based on the category of the accident, the related staff and the person in charge of the ship are to be selected and summoned. For example, selecting the Head of the Crew department for an accident of crew injury and Head of the Engineering division for an accident related to engine trouble. 基於事故種類，有關人員和負責船舶人員將被選擇並且召集。例如，當發生船員受傷事故時，選擇船員課最高長官，以及當機器故障時，則召集工務部最高長官。 | ☐ |
| 3. | The personnel selected and summoned must treat the emergency as a top priority and under the command of the Designated Person, should duly take action against the accident. 被選擇及召集人員必須把緊急情況當作第一優先，並聽命於指定管理人員，應該按時對事故採取行動。 | ☐ |
| 4. | The leader of the “Emergency Response Team” is to make conclusion and to determine countermeasures and inform the Master as soon as possible. 緊急事故處理小組的領導人做結論，並且確定對策，盡快通知船長。 | ☐ |
| 5. | Any personal injury? Any oil pollution? Container leaking? Fire? If need shore side urgent assistance? 是否有人員受傷？是否有油污染？是否有貨櫃漏洩？是否發生火災？是否需岸端緊急援助？ | ☐ |
| 6. | Have photos at scene (including damage condition) been taken & ready to send back? VDR, CCTV & ECDIS been downloaded? 現場照片(包括觸底受傷情況)是否已拍攝並準備寄回？VDR、CCTV及ECDIS是否已download? | ☐ |
| 7. | Have the risk assessment for overall emergency response & ship’s stability been effectively executed? 對所有緊急處置及船舶穩定性的風險評估是否已有效執行？ | ☐ |
| 8. | Have notified the nearest coasting country & flag state? 是否已通知沿岸最近管轄國及船旗國？ | ☐ |
| 9. | Have sent the following initial accident reports according to ISM PR-0802 in safety situation: 是否已照ISM PR-08程序，在安全無虞下，寄回下列初步事件報告： | ☐ |
|  | - Statement of fact(in Chinese & English) with photos | ☐ |
|  | - Sea Protest | ☐ |
|  | - Claim Letter | ☐ |
|  | - FM-0803-6, FM-0803-7 | ☐ |
|  | - drawing of collision(including angle of blow) | ☐ |
|  | - Report to flag state | ☐ |
|  | - Copy of log book, bell book, telegraph, course recorder… etc. | ☐ |
| 10. | PIC continues closely contacting with ship’s captain & provides necessary emergent assistance until the fleet vessel is safe & the case is over. 海技PIC持續與船端緊密連絡並協助緊急處置，直到屬輪平安事件結束為止。 | ☐ |
| 11. | PIC must go to the scene to investigate the accident as soon as possible, to check & confirm the persons, the thing, the time, & place, if all the same with the report from ship’s side. 後續海技PIC須儘快至現場調查事件發生情況，須針對人事時地物確認詳細情況是否與船上所報相同。 | ☐ |
| 12. | Interview with all the persons concerned for the accident to confirm the details of the accident. 與事件相關人員當面訪談以確認事件經過。 | ☐ |
| 13. | The finished report must be approved by the company before sending out to the parties concerned like flag state. 相關報告製作完成後，在發給相關單位如船旗國前，須經公司主管批核。 | ☐ |