Engine room on fire Emergency checklist

機艙失火緊急事故處理檢查表

| Ship Side-Emergency Response船端-緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| Whoever discovering the fire in Engine Room, push down the nearest button to active the fire alarm, notify OOW on bridge or at deck office immediately. Take effective actions to control the fire if it is safe to do so. 任何人發現機艙火災，按下最近的火警按鈕，立即通知駕駛台或甲板當值船副，如狀況安全可行下採取有效行動控制火源。 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1. | Received notification from engine duty or smoke & fire detecting alarm from engine room, immediately blow the fire alarm & broadcast for fire-fighting station and inform the Master. 接獲機艙當值人員通報或機艙冒煙或失火偵測警報，立即拉火警警報並廣播就滅火佈署及通知船長。 | ☐ |
| 2. | Check if any other traffic or immediate danger in the vicinity. 確認附近是否有其他交通或立即危險。 | ☐ |
| 3. | Close all air units for active Eng. Rm fire dampers by using the remote control in NAV & RADIO EQ. RM. 關閉位於駕駛台NAV & RADIO EQ. RM內控制箱遙控裝置的所有機艙通風閥門。 | ☐ |
| 4. | Check chart depth & object & other information around the ship. 檢查海圖上附近區域之水深、物件與其他必要資訊。 | ☐ |
| 5. | Record the time & ship position on chart. 記錄當時時間及船位並標示在海圖上。 | ☐ |
| 6. | Notify E/R for standing by engine. 通知機艙備車。 | ☐ |
| As soon as the Master on bridge 船長上駕駛台後： |  |  |
| 1. | Immediately ensure smoke or fire from which part of the engine room & the extent & seriousness of the smoke & fire, then broadcast & summon the crew for “engine room fire-fighting station”. 立即確認煙或火起自機艙何處及其嚴重程度及範圍，之後廣播與召集全員就”機艙滅火部署”。 | ☐ |
| 2. | Inform C/E reduce to lowest speed, start the emergency generator and maneuver to safe place for anchoring. 通知輪機長減到最低速，啟動緊急發電機並將船操到安全地方下錨。 | ☐ |
| 3. | Immediately ask C/O to assist C/E to organize & deploy the fire-detecting team with full fireman outfit & portable extinguisher into engine room to detect the source of fire. If the fire is small, use the portable extinguisher to put out the fire at scene. 立即請大副協助輪機長組織及部署探火隊，穿著全套防火裝備及手提滅火器進入機艙偵探火源，如為小火，立即用手提滅火器就地滅火。 | ☐ |
| 4. | If found dense smoke all over the engine room & the crew unable to approach, then immediately all evacuate from the engine room. 如發現機艙濃煙密佈，人員無法靠近，則全員立即自機艙撤出。 | ☐ |
| 5. | After summoning all crew & roll call, immediately deploy engine room CO2 fire-fighting station preparing to put out the fire. 船長召集全員點名後，立即照附錄機艙CO2滅火程序，就機艙CO2滅火部署。 | ☐ |
| 6. | CO2 release must be ordered by Master after completed the final roll call，ensure all evacuated from engine room and all ventilations in E/R are closed. CO2必須在所有人員撤出機艙，完成點名及關閉機艙所有水密門，艙蓋，通風，燃油閥及停燃油泵後，由船長下令釋放。 | ☐ |
| 7. | C/E lead the 1st fire-fighting team to fire control station, open the valve of pilot cylinder, release the associated bottles of CO2 to engine room to put out the fire. 由C/E率隊在滅火控制站打開引導鋼瓶閥，釋放相關瓶數的CO2進機艙滅火。 | ☐ |
| 8. | Another supporting team, led by 2/E with C/O’s assistance, immediately go to CO2 room stand-by, supposed 1st team in fire control station room failed, then they have to manually open the valve of CO2 one by one to release the CO2 in the hold. 另一隊支援組由大管率隊，C/O協助至船尾CO2 room stand-by，若在fire control station釋放失敗，則這組立即以手動一個一個打開安全插梢，將指定CO2陸續釋放到大艙。 | ☐ |
| 9. | Ensure all designated C/O have been released to engine room. 確認所有指定的CO2氣瓶已釋放至機艙。 | ☐ |
| 10. | Notify Maritech Division immediately according to “Emergency Contact List” and send ER & photos for necessary assistance & support. 參照“緊急聯絡電話表＂立即將情況通知海技部及發ER及照片，以利提供適時的協助及支援。 | ☐ |
| 11. | (IF NECESSARY) At early stage of the emergency, Master shall notify costal state or flag state, the notification shall include the type of the accident and what kind of assistance required. (需要的情況下)緊急事件初期，船長應通知最近的沿岸國及船旗國當局，通報內容包括事件類型及需要何種協助。 | ☐ |
| 12. | Crew is posted by turn outside of the engine room & continuously monitor the situation, cool down the top wall of engine room by water mist, if necessary. 船員輪流佈署在機艙外，持續監控狀況並在必要時，噴水霧至機艙壁以降溫。 | ☐ |
| 13. | The engine room is kept totally hermetically closed with CO2 inside for at least 7 hours. 機艙持續保持完全密封蓋住，CO2在內，最少7小時。 | ☐ |
| 14. | Execute risk assessment & ensure the following: 執行風險評估並確認下列事項： |  |
|  | - if fire-fighting team already deployed according to the fire-fighting muster list with appropriate fireman outfit & all fire-fighting equipments in order under C/O direction. 是否滅火隊已照滅火部署表穿著適當防火衣及攜所有相關滅火裝備在大副指揮下，準備滅火。 | ☐ |
|  | - Briefing to the fire-fighting team the procedure & precautions for fire-fighting. 已對滅火隊員做簡報，告知滅火步驟及注意事項。 | ☐ |
|  | - Commander & fire-fighting team keep distance from the fired container at safe place able to protect themselves during fire-fighting. 指揮官及滅火隊在滅火過程與起火櫃保持距離，立於能保護自身安全的位置。 | ☐ |
|  | - Send radio warnings to all ships in the vicinity to keep clear 無線電通知附近所有船舶保持距離。 | ☐ |
|  | - Downloaded VDR, CCTV & ECDIS, kept related record such as bell book/Log book/Chart /Course recorder/Sounding record, etc. 已下載VDR、CCTV及ECDIS並保留任何有關的紀錄如航行日誌、海圖、航向紀錄、量水紀錄等。 | ☐ |
|  | - Ship’s stress force and stability assessment 已執行船體應力與穩度評估。 | ☐ |
|  | - Check if container leaking cause oil spill overboard, if does, activate SOPEP at once. 確認是否有貨櫃漏洩造成油污染，如有，立即啟動SOPEP程序。 | ☐ |
|  | - Any injury happened due to fire-fighting. Is it necessary to execute rescue, first aid including CPR? 確認本輪是否因滅火導致人員受傷。有無需要執行救助或急救(包括CPR)。 | ☐ |
|  | - Sent ER, Contact with PIC and coasting country for further assistance, prepare the abandon ship procedure (in case if necessary) 確認已發送ER報、通知PIC及沿海國等相關單位請求更多幫助，如果有需要情況下則準備棄船部署。 | ☐ |
|  | - Stability should be assessed when taking any action. 在採取任何動作之前是否都已做船舶的穩定度評估。 | ☐ |
|  | - Chang Trim, Reduce the draught of the ship & make even keel & pump out water if necessary 有必要情況下，調整Trim、減低船舶吃水或Even keel，打水以減低損傷。 | ☐ |
|  | - Find the cause of container fire 查明機艙失火的原因。 | ☐ |
|  | - Record the progress of the fire-fighting hourly & report to the company. 每小時記錄滅火進度並報告公司 | ☐ |
|  | - Continuously contact with the company & agent of next port for necessary assistance & arrangement (i.e. shore fire-fighting brigade or shelter anchorage). 持續與公司及下一港的代理保持連絡以獲得必要的援助及安排(如岸上救火隊或避難錨地)。 | ☐ |
|  | - If the situation of fire becomes serious & out of control, affecting seamen’s life onboard and necessary to “abandon ship”, after cautious assessment & discussing with C/E & C/O, send the “distress signal” first, announce/broadcasting “abandon ship” summon all the crew for abandon ship station in order according to “abandon ship muster station list” 若火勢變得嚴重且失控，威脅到船員生命，需要”棄船”；經過審慎評估與輪機長，大副商議後，先發送遇難信號，立即廣播全船就”棄船部署”，召集所有船員照棄船部署表就位及按秩序執行。 | ☐ |
| 15. | - After at least 7 hours, send the fire detecting team with full fireman outfit & portable extinguisher in engine room to check the situation if all the fire has been extinguished and put out the remaining fire by portable extinguisher, if necessary, restore the generator, M/E and resume the course proceeding to the next port, then report the details to the company. 最少經7小時後，派探火隊著全套滅火衣及手提滅火器去機艙檢查，看是否所有的火已滅掉並以手提滅火器撲滅餘火，啟動發電機，主機及回復原航向駛往下一港，並將詳情及所需援助報公司。 | ☐ |
| Once oil spill overboard若油污漏洩入海 |  |  |
| 1. | According to SOPEP, from the information provided by damage control team, appraise the extent of the damage and immediately proceed the leaking-block, confirm other tanks watertight to avoid the damage spreading; any oil spill due to the breakage of the tank, stop the pollution according to SMM PR-08 Emergency Plan: Pollution Prevention; and prevent fire and explosion from oil spill; any spill overboard, report to parties concerned as per SOPEP’s procedure and format. 依據船舶油污染應急計畫(SOPEP)，以損害管制組回報之資訊，評估損害程度並指揮損害管制組進行堵漏、確保其他艙間水密性以防止損害擴大；有油料因破損溢出時，依照安全管理手冊PR-08 Emergency Plan: Pollution Prevention進行油污染防阻，並避免因溢油產生之爆炸與火災；若有任何油污漏洩入海，須照SOPEP程序通報有關單位。 | ☐ |
| 2. | Make risk assessment for oil spill overboard & confirm the oil pollution condition and take photos. 執行油污漏洩入海風險評估，確認油污污染狀況並拍攝現場照片。 | ☐ |
| 3. | Muster all crew for the oil pollution control. 召集船員執行防油污污染部署。 | ☐ |
| 4. | Dispatched an initial oil spills report to TPE / Maritech & local related Dept. 發送初步油汙汙染報告至海技部及與當地相關單位。 | ☐ |
| 5. | PIC of Legal & INS Dept. to be informed to P & I Club and Hull insurance to arrange oil pollution clear operation. 法保PIC通知P&I Club 履行保險契約準備清理油汙汙染作業。 | ☐ |
| 6. | Inform Agent to arrange for any emergency assistance 通知代理，準備任何緊急情況之協助。 | ☐ |
| After the emergency action, Master shall pay some special attention to collect the evidences and make the reports as the following: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 1. | As per SMM PR-08, any accident, the required report forms are, FM-0803-1, FM-0803-7, and statement of fact & sea protest. 依據公司安全管理手冊所規定之意外事故所規定表格繕製意外事故報告(FM-0803-1)、意外事故調查報告(FM-0803-7)以及Statement of Fact、Sea Protest呈報公司； | ☐ |
| 2. | Take good care of VDR & CCTV’s recording( as VDR will continue recording for 12 hours, after Master contacting with the authority concerned, stop the recording and take out CF Card for later investigation) 妥善保存VDR、CCTV之記錄(因VDR會連續紀錄12HRS，在船長與主管機關聯繫後，再將記錄器停止，並取出CF CARD以供後續調查使用)。 | ☐ |
| 3. | Video or photos for the accident. 事故經過的拍攝影片及照相； | ☐ |
| 4. | The description of fire incident shall include all witnesses and duty persons, statement of fact shall attach photos as many as possible, the information shall include: 火災事故過程之描述，包括所有目擊及當事人之報告，STATEMENT OF FACT儘可能附上事故現場照片，收集呈報公司之資料証據包括： | ☐ |
|  | - Ship’s position/course/speed/rpm and course recorder reading at time; 當時之船位/航向/航速/轉速及當時航向記錄器所顯示之記錄； | ☐ |
|  | - Engine/steering maneuvering/time and condition; 當時的用車/用舵/時間及狀況； | ☐ |
|  | - The prevailing weather/wind direction, wind speed/direction of wave and height/swell direction and height/visibility/received weather forecast; 當時天氣狀況/風向、風速/浪之方向與高度/湧之方向與高度/能見度/收受之天氣預報； | ☐ |
|  | - Positioning fix system; 使用之定位系統； | ☐ |
|  | - Ships recording of response action at time (including operation of M/E); 發現起火時，反應動作之記錄(含主機操作)； | ☐ |
|  | - The damage condition of associated ship’s hull due to fire with attached photos, if possible; 相鄰船體因火災受損壞情況，儘可能附照片； | ☐ |
|  | - The damage condition of ship and cargo on both ships, with attached photos, if possible; 有船貨損壞情況，儘可能附照片； | ☐ |
|  | - Master’s sea protest shall be endorsed; 船長之海事報告(Sea Protest)，須加以簽證； | ☐ |
|  | - If arranged surveyor for damage inspection, shall prepare GA and shell expansion drawing; 如有安排驗船師SURVEYOR登輪作損壞檢定，應備妥GA及SHELL EXPANSION圖。 | ☐ |
|  | - CLASS SURVEYOR : to assess ship’s seaworthiness: it’s better to be arranged by the Company. 用於判定船體適航性、最好由公司決定是否僱請。 | ☐ |
|  | - INSURANCE SURVEYOR: to evaluate the damage, it’s separated to hull surveyor and P & I surveyor; no assessment for liability. 作損傷鑑定、可分船體 SURVEYOR 及 P & I SURVEYOR、不作責任判斷用； | ☐ |
|  | - Lawyer: shall identify for whom he represents. 律師 (LAWYOR, SOLICITOR, ATTORNEY, ADVOCATE均是)：須認清代表何方； | ☐ |
|  | - Salvage Superintendent or Salvage Surveyor等； 救助負責人或檢查員； | ☐ |
|  | - Coast Guard or Port authority investigator. 海岸巡防隊或港務局/海事處調查人員。照公司指示提供文件及證據。 | ☐ |
|  | - Report to flag of country in 24 HR. 事發24小時內向船旗國回報。 | ☐ |
|  | - .All external report must check by company. 所有對外報告內容都須經過公司審議 | ☐ |

| Shore Side-Emergency Response 岸端緊急處置 |  |  |
| --- | --- | --- |
| Checking Items 檢查項目內容 |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  | ☐ |
| 1. | PIC must confirm the following as soon as receiving report from Master: PIC第一時間確認船端回報下列事項： | ☐ |
|  | - The fire position? 火災發生位置？ | ☐ |
|  | - Have damage been controlled? 是否受損已被控制？ | ☐ |
|  | - The draft of fore and astern? 艏艉現在吃水？ | ☐ |
|  | - Have all machine, equipments, cargo holds and tanks condition been checked & confirmed? 所有機器、設備、貨艙及tank情況是否已檢查並確認？ | ☐ |
| 2. | Any personal injury? Any oil pollution? Container leaking? Fire? If need shore side urgent assistance? 是否有人員受傷？ 是否有油污染？是否有貨櫃漏洩？是否發生火災？是否需岸端緊急援助？ | ☐ |
| 3. | Have photos at scene been taken & ready to send back? VDR, CCTV & ECDIS been downloaded? 現場照片是否已拍攝並準備寄回？VDR、CCTV及ECDIS是否已下載? | ☐ |
| 5. | Have the risk assessment for overall emergency response & ship’s stability been effectively executed? 對所有緊急處置及船舶穩定性的風險評估是否已有效執行？ | ☐ |
| 6. | Have notified the nearest coasting country & flag state? 是否已通知沿岸最近管轄國及船旗國？ | ☐ |
| 7. | Have sent the following initial accident reports according to SMM PR-0802 in safety situation: 是否已照安全管理手冊PR-0802程序，在安全無虞下，寄回下列初步事件報告： | ☐ |
|  | - Statement of fact(in Chinese & English) with photos | ☐ |
|  | - Sea Protest | ☐ |
|  | - FM-0803-1, FM-0803-7 | ☐ |
|  | - Report to flag state | ☐ |
|  | - Tank & cargo hold sounding record | ☐ |
|  | - Copy of log book, bell book, telegraph, course recorder, weather chart… etc.. | ☐ |
| 8. | PIC continues closely contacting with ship’s captain & provides necessary emergent assistance until the fleet vessel is safe & finish repair & investigate case over. 海技PIC持續與船端緊密連絡並協助緊急處置，直到屬輪安全後。以及後續的維修與調查事件結束為止。 | ☐ |
| 9. | After PIC investigates the condition of scene, he must check person, thing, time, place, object if the same with record of ship side. 後續海技PIC至現場調查事件發生情況，須針對人事時地物是否與船上紀錄相同。 | ☐ |
| 10. | Interview with all persons related to the accident. 與事件相關人員的當面洽談。 | ☐ |
| 11. | The finished report must be approved by the company before sending out to the parties concerned like flag state. 相關報告製作完成後，在發給相關單位如船旗國前，須經公司主管批核。 | ☐ |