Mooring Rope Fouling with Propeller Accident

纜繩事故緊急處理程序檢查表

| Ship Side-Emergency Response 船端-緊急處置 |  |  |
| --- | --- | --- |
| Checking Items 檢查項目 |  |  |
| The action of on scene officer: 現場船副應： |  |  |
| 1. | Find any abnormal condition when heaving the mooring rope, stop the operation immediately and report to the bridge. 當發現收回纜繩有異常情況時，停止絞纜，並於第一時間通報駕駛台。 | ☐ |
| 2. | If line still tight, keep away from the mooring rope for avoiding any damage condition. 若纜繩有持續吃力之情況，命令有關人員遠離，以防危險事故發生。 | ☐ |
| 3. | After reporting the details to the bridge, follow the master’s instruction. 通報駕駛台詳細情況後，依船長命令執行處理。 | ☐ |
| When receiving the massage of fouling with the propeller or B/T, Master should immediately stop the operation of maneuvering engine and rudder & check the following: 當駕駛台收到纜繩攪入的訊息後，船長應立即停止動車動舵。並確認下列事項： |  |  |
| 1. | Order the duty officer to record all related process and time. 吩咐駕台船副記錄詳細所有相關事項。 | ☐ |
| 2. | If the direction of ship’s motion will endanger the situation. 船舶現在運行方向與是否有立即危險。 | ☐ |
| 3. | Order the fore station standing by both anchors. 吩咐船頭雙錨備便以防緊急需要。 | ☐ |
| 4. | Ask pilot for tug assistance and discuss next countermeasures. 請求領港呼叫拖輪給予需要協助並商討適當對策。 | ☐ |
| 5. | Check the position of the mooring rope fouling with the propeller. 確認纜繩絞入事件發生位置。 | ☐ |
| 6. | Check the condition of the mooring rope fouling with propeller or B/T. 纜繩絞入之情況。 | ☐ |
| 7. | Check the condition of weather, the direction and speed of wind. 確認天氣情況，風向及風速。 | ☐ |
| 8. | Forbid operating engine in any condition for protecting the propeller. 在任何情況下禁止動車以防傷及車葉。 | ☐ |
| 9. | Continue mooring operation with “stop, parallel, alongside” upon ship’s berthing. 若正在進港：繼續帶纜，試圖將船舶平停靠於碼頭上。 | ☐ |
| 10. | Ask pilot for tug assistance, try to moor back to berth upon ship’s departure from port. 若正在離港：告知領港，以拖輪協助，將船帶回原碼頭上。 | ☐ |
| 11. | After making sure ship safety, contact Maritech PIC and agent for diver’s help. 在保證船舶安全情形下，聯絡岸端PIC與代理給予相關支援。 | ☐ |
| the action of bridge officer: 駕駛台當職船副應： |  |  |
| 1. | Record all related process and time. 詳實記錄所有事件的處理步驟與時間。 | ☐ |
| 2. | Check bell book & record tugs name, pilot name and pilot signature. 確認車鐘簿是否有領港簽名、拖船名稱。 | ☐ |
| 3. | Record the ship position and local time. 記錄當時船位與時間。 | ☐ |
| 4. | Record the course of ship 記錄當時船航向。 | ☐ |
| 5. | Check the echo sounder depth 檢視測深儀深度。 | ☐ |
| 6. | Check the telegraph recording time and in good condition 檢視車鐘記錄器運作與時間是否一切正常。 | ☐ |
| 7. | Notify E/R that the mooring rope fouling with propeller, stop operate engine in any condition. 通知機艙纜繩攪入，切勿動車。 | ☐ |
| After ship mooring at berth, Master should check items as follows: 當船舶繫回碼頭後，在事件發生初期，船長應注意下列事項： |  |  |
| 1. | If already contacted with the Maritech PIC and local agent. 是否已聯絡岸端PIC與當地代理。 | ☐ |
| 2. | Send C/O to the scene, check the condition and take photo. 指派大副前往評估現場狀況與拍攝現場照片。 | ☐ |
| 3. | Keep tugs for steadying ship’s position if necessary. 如情況需要保留拖船，協助穩定船身。 | ☐ |
| 4. | Notify E/R for checking all condition of main engine (any oil spill) and stop operation. 通知機艙適當檢查主機情況(艉軸是否有漏油現象)，並嚴禁動俥。 | ☐ |
| 5. | Check if any emergency condition about MOB, crew injury, etc. 確認是否有人員受傷或落海。有無需要執行救助或急救(包括CPR)。 | ☐ |
| 6. | Check if already sent ER. 確認已發送ER報。 | ☐ |
| 7. | Download VDR/CCTV data. 確認VDR/CCTV 可以下載。 | ☐ |
| 8. | Ship stability should be assessed when taking any action. 在採取任何動作之前是否都已做船舶的穩定度評估。 | ☐ |
| After the emergency action, Master shall pay some special attention to collect the evidences and make the reports as the following: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 1. | As per SMM PR-08, any accident, the required report forms are FM-0803-06, FM-0803-07, statement of fact & sea protest. 依據公司安全管理手冊所規定之意外事故所規定表格繕製意外事故報告(FM-0803-6)、意外事故調查報告(FM-0803-7)以及STATEMENT OF FACT、SEA PROTEST呈報公司； | ☐ |
| 2. | All bridge and fore/astern station work includes the fixes and mooring rope operation condition before the accident. 所有持續到纜繩攪入當時的駕駛台與現場作業，包括事故前繪於海圖上的船位、纜繩操作順序等； | ☐ |
| 3. | Master shall ensure all the fixes on using charts, no forgery. 船長務必確定海圖上之船位，避免塗改； | ☐ |
| 4. | Telegraph or other RPM output recording. 車鐘記錄器或其他主機引擎輸出有関運轉之記錄器； | ☐ |
| 5. | Confirm that bridge clock time matches with engine room’s and other auto recording. 駕駛台上船鐘與機艙是否正確一致及其他自動記錄器之確認； | ☐ |
| 6. | Take good care of VDR & CCTV’s recording( as VDR will continue recording for 12 hours, after Master contacting with the authority concerned, stop the recording and take out CF Card for later investigation) 妥善保存VDR、CCTV之記錄(因VDR會連續紀錄12HRS，在船長與主管機關聯繫後，再將記錄器停止，並取出CF CARD以供後續調查使用)。 | ☐ |
| 7. | The position fixed by radar operator. 雷達操作者所做的船位記錄； | ☐ |
| 8. | Video or photos for the accident. 事故經過的拍攝影片及照相； | ☐ |
| 9. | The statement in deck log and engine log shall be coherent (coordinate with the Company to record only the favorable part). 在甲板DECK LOG 及ENG LOG記錄應協調一致； | ☐ |
| 10. | The description of Mooring Rope Fouling accident shall include all witnesses and duty persons, statement of fact shall attach photos as much as possible, the information shall include: 纜繩絞纏事故過程之描述，包括所有目擊及當事人之報告，STATEMENT OF FACT儘 可能附上事故現場照片，收集呈報公司之資料証據包括： | ☐ |
|  | - Ship’s position/course/speed/rpm and course recorder reading at time. 當時之船位/航向/航速/轉速及航向記錄器所顯示之記錄； | ☐ |
|  | - Engine/steering maneuvering/time and condition after case; 事件發生後用車/用舵/時間及狀況； | ☐ |
|  | - Estimated prevailing visibility/the time of signal heard or seen and its relative position; 預估能見度/註明看到或聽到事故時間及相關船位； | ☐ |
|  | - The prevailing weather/wind direction, wind speed/direction of wave and height/swell direction and height/visibility/received weather forecast; 當時天氣狀況/風向、風速/浪之方向與高度/湧之方向與高度/能見度/收受之天氣預報； | ☐ |
|  | - The condition of tide and current; 潮、流之情形； | ☐ |
|  | - Bridge composition before the accident or immediately on bridge 在駕駛台上或立即上駕駛台之人員配置； | ☐ |
|  | - Ships recording of response action from the beginning till Mooring Rope Fouling with Propeller 船舶從開始至事件發生之時間內反應動作之記錄(含主機操作)； | ☐ |
|  | - The damage condition of ship and cargo with attached photos, if possible; 船貨損壞情況，儘可能附照片； | ☐ |
|  | - Deck, engine bell book, telegraph record; 甲板，機艙之車鐘記錄簿，車鐘記錄圖紙； | ☐ |
|  | - Used chart and fixed position at the time; 當時使用海圖與定位記錄。 | ☐ |
|  | - Master’s sea protest shall be endorsed(only state the part favorable to the company); 船長之海事報告(Sea Protest)，須加以簽證； | ☐ |
|  | - If any salvage, shall record the details of salvor’s operation and own ship’s coordinating action in order for rewards basis; if arrangement of rescue and repair, fitness for sailing will be impaired, seaworthiness certificate shall be verified and secured by Class to issue “fit to proceed” before sailing to sea. 假如有SALVAGE時，應將SALVOR所作所為詳作記錄，以及本船所配合進行動作列入記錄，以作為日後記算酬勞金之依據，若有安排修理救助，適航性必受損，在未獲得SEAWORTHINESS CERTIFICATE前移動航行將失去保險之COVER，故一定要取得適航証明(由船級協會CLASS SOCIETY SURVEYOR簽發)，完成FIT TO PROCEED如此才能確保保險有效。 | ☐ |
|  | - If arranged surveyor for damage inspection, shall prepare GA and shell expansion drawing; 如有安排驗船師SURVEYOR登輪作損壞檢定，應備妥GA及SHELL EXPANSION圖。 | ☐ |
|  | - The likely boarding parties after mooring rope fouling; 纜繩絞纏事故發生後可能上船之相關人員分。 | ☐ |
|  | - CLASS SURVEYOR: to assess ship’s seaworthiness: it’s better to be arranged by the Company. 用於判定船體適航性、最好由公司決定是否僱請； | ☐ |
|  | - INSURANCE SURVEYOR: to evaluate the damage, it’s separated to hull surveyor and P & I surveyor; no assessment for liability. | ☐ |
|  | 作損傷鑑定、可分船體 SURVEYOR 及 P & I SURVEYOR、不作責任判斷用； |  |
|  | - Salvage Superintendent or Salvage Surveyor等； 救助負責人或檢查員； | ☐ |
|  | - Coast Guard or Port authority investigator. 海岸巡防隊或港務局/海事處調查人員。 | ☐ |
|  | - Report to flag of country in 24 HR. 事發24小時內向船旗國回報。 | ☐ |
|  | - .All external report must check by company. 所有對外報告內容都須經過公司審議。 | ☐ |

| Shore Side-Emergency Response岸端緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目內容 |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  | ☐ |
| 1. | PIC must confirm the following as soon as receiving report from Master: PIC第一時間確認船端回報下列事項： | ☐ |
|  | - Notify Master for stopping to use engine immediately. 提醒船長切勿動車。 | ☐ |
|  | - The condition of Mooring Rope Fouling? 纜繩攪入情況？ | ☐ |
|  | - Ship mooring at berth safely? 船是否安全靠上碼頭？ |  |
|  | - Have all machine, equipment and tanks condition been checked & confirmed? 所有機器、設備及tank情況是否已檢查並確認？ |  |
|  | - Any emergency condition like crew injury for any urgent help. 是否有人員受傷或其他緊急需要協助之事故？ | ☐ |
| 2. | Contact M&R PIC and local agent, arrange the diver for service. 聯絡工務PIC與當地代理，協助安排潛水夫與後續處理事項。 | ☐ |
| 3. | Have photos at scene (including damage condition) been taken & ready to send back? VDR, CCTV & ECDIS been downloaded? 提醒船長拍攝現場照片並準備寄回？VDR、CCTV及ECDIS是否已下載? | ☐ |
| 4. | Have the risk assessment for overall emergency response & ship’s stability been effectively executed? 對所有緊急處置及船舶穩定性的風險評估是否已有效執行？ | ☐ |
| 5. | Have notified the nearest coasting country & flag state? 是否已通知沿岸最近管轄國及船旗國？ | ☐ |
| 6. | Have sent the following initial accident reports according to SMM PR-0802 in safety situation: 是否已照安全管理手冊PR-08程序，在安全無虞下，寄回下列初步事件報告： | ☐ |
|  | - Statement of fact(in Chinese & English) with photos | ☐ |
|  | - Sea Protest | ☐ |
|  | - Claim Letter | ☐ |
|  | - FM-0803-6, FM-0803-7 | ☐ |
|  | - Report to flag state | ☐ |
|  | - Copy of log book, bell book, telegraph record, course recorder, etc. | ☐ |
| 7. | PIC continues closely contacting with ship’s captain & provides necessary emergent assistance until the fleet vessel is safe & the case is over. 海技PIC持續與船端緊密連絡並協助緊急處置，直到屬輪平安事件結束為止。 | ☐ |
| 8. | PIC must go to the scene to investigate the accident as soon as possible, to check & confirm the persons, the thing, the time, & place, if all the same with the reports from ship’s side. 後續海技PIC須儘快至現場調查事件發生情況，須針對人事時地物確認詳細情況是否與船上所報相同。 | ☐ |
| 9. | Interview with all the persons related to the accident to confirm the details. 與事件相關人員當面訪談以確認事件經過。 | ☐ |
| 10. | The finished report must be approved by the company before sending out to the parties concerned like flag state. 相關報告製作完成後，在發給相關單位如船旗國前，須經公司主管批核。 | ☐ |