Main Engine Breakdown Emergency Checklist

主機故障緊急事故處理檢查表

| Generally, the main engine will be shutting down automatically due to: Lubricating oil pressure failure, Excess bearing temperature, Engine over-speed, Control air failure and etc. Immediately attention and treatment to the cause of shut down must be made so as to have a quick restore of the engine function. If the engine repair work have outside the capability of ship-hands, the master should report to the company to ask for shore assistance. 主機故障一般是滑油低壓、軸承溫度過高、主機超速、空氣控制故障等因素造成。應立即注意和處理故障原因，以便能迅速恢復這主機的功能，如果主機修理超過船上的能力，船長應該向公司報告要求岸上幫助。 |  |  |
| --- | --- | --- |
| Ship Side-Emergency Response船端-緊急處置 |  |  |
| Bridge Emergency Response駕駛台緊急處置 |  |  |
| Checking Items檢查項目 |  |  |
| In Harbour: vessel may suffer an engine breakdown after engine repairs. 在港區:船隻可能因臨時修理與進塢，造成主機異常。 |  |  |
| 1. | Before sailing, test main engine telegraphs ahead and astern with engineer. 開航前，與輪機員確實測試主機進倒車，功能正常。 | ☐ |
| 2. | Before sailing, avoid vessel planning route inside inland waterway shortly after departure. 避免開航後，本船航線需要立即通過內陸或繁忙水域。 | ☐ |
| 3 | Noticed main engine breakdown signs, inform captain immediately with walky-talky. 當發現主機故障，立即以對講機通知船長。 | ☐ |
| 4. | Confirm the cause of breakdown is Compress air low pressure or starting RPM failure immediately or any notifications from E/R. 立刻確認是否啟動空氣失壓，點火失敗，或機艙通知的其他原因。 | ☐ |
| 5. | Is engine RPM readily available after reset or restart again? 主機重設或重新啟動，是否立即可用？ | ☐ |
| 6. | If main engine is not available in short interval, inform Fore and Aft station to stand by for emergency let-go anchor or take additional tug to assistance, host NUC lights or shapes. 若主機無法立刻回復正常，通知船頭船艉人員備便緊急下錨，或增帶拖船協助，懸掛操縱失靈號燈號標。 | ☐ |
| 7. | Confirm whether vessels in vicinity will cause immediate danger or not 確認附近船舶，不會造成立即的危險。 | ☐ |
| 8. | Check water depth and distance from shore now, current direction and other navigational hazards. 檢查海圖上附近區域之水深、離岸距離、流水方向與其他航行險阻。 | ☐ |
| 9. | Send emergency or safety massage to VTS by VHF 16, ask for right of way or tug service and required anchorage for repairing. Don’t discuss any term of service type with tug boat operator, do not agree to any wordings of Lloyd's Open Form (LOF) which is “no cue no pay”. 發送VHF警急或安全信文給海岸管制台，要求通航的優先權或拖船協助，或臨時修理的錨地。請勿同意拖船此次服務為救難的性質，如被VHF錄音，會要求天價的金額。 | ☐ |
| 10. | Note the date/time and ship position on the chart and use remained speed to steady the vessel and keep away from danger，proceed to anchorage if any. 在海圖上記錄當時時間和船位，利用本船餘速，穩定航向，遠離危險，準備下錨待命。 | ☐ |
| 11. | Notify the engine room to stand by bow thruster to assist heading control. 通知機艙前俥供電，必要時調整船位，以利掌控漂流方向。 | ☐ |
| 12. | Ask chief mate to assign deck crews to guard against pirates. 請大副指派甲板人員值防盜班。 | ☐ |
| In coastal area 在近岸航行區: |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1. | Received main engine breakdown notification, call captain immediately. 當接獲機艙當值人員通報主機故障，立即通知船長上駕駛台。 | ☐ |
| 2. | Confirm whether vessels in vicinity will cause immediate danger. 確認附近船舶，是否造成立即的危險。 | ☐ |
| 3. | Send emergency or safety massage to all vessels by VHF 16, host NUC lights or shapes. 發送VHF警急或安全信文給海面其他船隻，懸掛操縱失靈號燈號標。 | ☐ |
| 4. | Check water depth and distance from shore now, current direction and other navigational hazards. 檢查海圖上附近區域之水深、離岸距離、流水方向與其他航行險阻。 | ☐ |
| 5. | Note the date/time and ship position on the chart and use remained speed alter course to open sea and keep away from danger. 在海圖上記錄當時時間和船位，利用本船餘速，轉向開闊水域，使船舶遠離危險。 | ☐ |
| 6. | Notify the engine room to stand by bow thruster, if necessary. 通知機艙前俥供電，必要時調整船位，以利掌控漂流方向。 | ☐ |
| 7 | Broadcasts main engine failure and other immediate danger to all crews if necessary. 如有必要全船廣播，主機故障與其他立即危險，全船人員注意警戒待命。 | ☐ |
| As soon as the Master on bridge 船長上駕駛台後： |  |  |
| 1. | Confirm the navigational hazards in vicinity and safe drifting area immediately. 立即確認周圍環境與安全的漂流區域。 | ☐ |
| 2. | Judge vessel final stop position after all speed had lost. 決定船隻最後停止的位置何在？ | ☐ |
| 3. | Decide which side is suitable weather side (upwind or up-current) to drift from final stop position. 決定漂流時，何舷受風，才能遠離危險。 | ☐ |
| 4. | Use last steerage or Bow thruster to adjust ship’s weather side. 利用船隻餘速，調整船首向正確受風，如果不行，使用船首推進器協助掉頭。 | ☐ |
| 5. | Estimate drifting speed and course if danger is nearby. 估計漂流的方向與速度，如果臨近危險。 | ☐ |
| 6. | Walk back 1 and 1/2 shackles of weather side anchor into water to slow down the drifting and further drifting to shallow water if water depth is not enough. 如有必要，用錨機緩慢將上風錨一節半鬆至水中，用來減緩漂流速度與預防擱淺。 | ☐ |
| 7. | If water depth and sea conditions are suitable, dual anchors should be ready to let go at any time. 如果水深、海況條件合適，雙錨備便隨時準備下錨。 | ☐ |
| 8. | Ask Chief Engineer to estimate the approximate time needed for repairs to evaluate outside support (such as Tug boat) we needed. 詢問輪機長預估大約需要修理的時間與情形，以決定是否需要外部救援(如拖船)。 | ☐ |
| 9. | Ask chief mate to assign deck crews to guard against pirates. 請大副指派甲板人員值防盜班。 | ☐ |
| 10. | Do not obstruct the engine room emergency repair, continue to monitor progress of the emergency repair. 以不妨礙機艙搶修的原則下，持續保持了解搶修進度。 | ☐ |
| 11. | Has the ship’s safety assessment been done before taking any action? 在採取任何動作之前，是否都已做好船舶的安全性評估。 | ☐ |
| 12. | If the result of assessment requires onshore support or seriously affect to the ship's schedule, Master is required to send an ER. Refer to the "Emergency contact telephone list" and according to the safety management manual PR-08. Notify to Marine Technology Division, Operation Divisions, and Engineering Division etc. immediately for the related works arrangement. 如評估需要岸上支援或嚴重影響船期，船長需發送ER。並依安全管理手冊PR-08規定，參考“緊急聯絡電話表＂立即將情況通知海技部、作業部、工務部，以利船期調度等岸端協助及支援。 | ☐ |
| 13. | Downloaded VDR, CCTV & ECDIS, kept related record such as bell book/Log book/Chart/Course recorder/Sounding record, etc. 下載VDR、CCTV及ECDIS並保留任何有關的記錄如俥鐘記錄、航行日誌、海圖、航向記錄、水深記錄等。 | ☐ |
| 14. | After initial assessment of the situation captain should notify port state authorities if necessary, the notification shall include type of incident and what assistance is needed. 在初步緊急處理後，如有必要，船長應通知最近的港口國海事當局，通報內容包括事件類型及需要何種協助。 | ☐ |
| 15. | Continuous contact with the company and the agent in the next port to help grasp the ETA. 持續與公司代理行聯絡，報告最新的ETA資訊。 | ☐ |
| 16. | Notify the nearby agents and prepare for assistance in any emergency. 通知沿岸代理，本輪所需任何緊急情況之協助。 | ☐ |
| After the emergency action, Master shall pay special attention to collect the evidence and make the reports as the followings: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 17. | As per SMM PR-08, any accident must do the required reports as follows: 依據公司安全管理手冊所規定之意外事故所規定表格繕製下列報告: |  |
|  | - Statement of fact (in Chinese & English) with photos. | ☐ |
|  | - Sea Protest. | ☐ |
|  | - FM-0803-4, FM-0803-7. | ☐ |
|  | - Report to flag state. | ☐ |
|  | - Tank & cargo hold sounding record. | ☐ |
|  | - Copy of log book, bell book, telegraph, course recorder, weather chart etc.. | ☐ |
| 18. | All external reports must be reviewed by Company before sending out. 所有對外報告在寄出去前，都須經過公司審核。 | ☐ |

| Engine Room Emergency Response機艙緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| 1. | Find out the cause according to the problem (refer to the instruction manual or engine remote alarms reference), and carry out emergency repairs. 根據故障問題 (參考主機說明書或內部的警報修理指南) 找出原因，進行搶修。 | ☐ |
| 2. | Do risk assessment and pay attention to personnel safety during emergency repairs. 搶修過程中執行風險評估及注意人員安全。 | ☐ |
| 3. | Evaluate the time required for repairs and notify the captain. 評估修理所需時間並通知船長。 | ☐ |
| 4. | Ensure the main power supply is normal. 確保主電力供應正常。 | ☐ |
| 5. | After engine trouble solved notify the bridge for test running and resume sailing when everything is normal. 故障排除後通知駕駛台試車，一切正常後恢復航行。 | ☐ |
| 6. | Submit the incident report and attach a video or photos description. 提交事故處理報告並附上影片或照相說明。 | ☐ |

| Shore Side-Emergency Response 岸端緊急處置 |  |  |
| --- | --- | --- |
| Checking Items 檢查項目 |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up. 海技PIC接獲船端報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門： |  | ☐ |
| 1. | Any personal injury? Any oil pollution? Container leaking? Fire? If need shore side urgent assistance (such as Tug boat)? 是否有人員受傷? 是否有油污染?是否有貨櫃漏洩?是否發生火災?是否需岸端緊急援助(如拖船)? | ☐ |
| 2. | Have photos at scene been taken & ready to send back? VDR & ECDIS been downloaded? 現場照片是否已拍攝並準備寄回?VDR、CCTV及ECDIS是否已下載? | ☐ |
| 3. | Have the risk assessment for overall emergency response & ship’s stability been effectively executed? 對所有緊急處置及船舶穩定性的風險評估是否已有效執行? | ☐ |
| 4. | Have notified the nearest coasting country & flag state? 是否已通知沿岸最近管轄國及船旗國? | ☐ |
| 5. | Completed the initial report which required from SSM department, include following items: 完成海技部船管課事故初始報告，包含以下項目: |  |
| 6. | - Date發生日期 | ☐ |
|  | - Time發生時間 | ☐ |
|  | - Position船舶位置 | ☐ |
|  | - Ship Status 船舶目前狀態 | ☐ |
|  | - Description of the incident 事故狀況說明 | ☐ |
|  | - Current handling progress and other department’s contact detail. 目前處理進度＆其他部門聯繫狀況說明 | ☐ |
| 7. | Have sent the following initial accident reports according to SMM PR-0802 in safety situation: 是否已照安全管理手冊PR-0802程序，在安全無虞下，寄回下列初步事件報告： |  |
|  | - Statement of fact(in Chinese & English) with photos | ☐ |
|  | - Sea Protest | ☐ |
|  | - FM-0803-4, FM-0803-7 | ☐ |
|  | - Report to flag state | ☐ |
|  | - Tank & cargo hold sounding record | ☐ |
|  | - Copy of log book, bell book, telegraph, course recorder, weather chart… etc.. | ☐ |
| 8. | PIC continues closely contacting with ship’s captain & provides necessary emergent assistance until the fleet vessel is safe & finish repair & investigate case over. 海技PIC持續與船端緊密連絡並協助緊急處置，直到屬輪安全後。以及後續的維修與調查事件結束為止。 | ☐ |
| 9. | The finished report must be approved by the company before sending out to the parties concerned like flag state. 相關報告製作完成後，在發給相關單位如船旗國前，須經公司主管批核。 | ☐ |
| 10 | Remind the ship to apply for emergency spares parts, if needed. 提醒如果需要，須向工務部提出緊急配件申請。 | ☐ |