Electricity failure Emergency Checklist

電力故障緊急處理程序檢查表

| Electricity failure usually caused by generator breakdown because of no lubrication oil, cooling water, fuel oil, or over-speed, overload, etc. Partial electric failure caused by defects of electric insulation. The priority in total electric failure is to restore by using standby and emergency generators. When one generator set fails, to reduce the electric load, chief engineer should turn off high-power demand equipment during the berthing/unberthing, such as reefer and bow thruster power supply. 主電力故障通常是因為滑油、冷卻水、燃油不夠，或超速，過負荷等，引起發電機故障。部分停電通常是由於絕緣的缺陷所引起。在全部停電時，優先使用備用和緊急發電機恢復電力。當部份發電機組故障時，降低電力負載，在靠/離港期間輪機長應該考慮關掉大功率設備供，例如冷凍櫃供電、停用前車等。 |  |  |
| --- | --- | --- |
| Ship Side-Emergency Response船端-緊急處置 |  |  |
| Bridge Emergency Response駕駛台緊急處置 |  |  |
| Checking Items檢查項目 |  |  |
| In Harbor area, there may have danger of main engine loses power due to power trip (black out). 在港區，因為突然的跳電，可能有船舶主機失去動力的險情。 |  |  |
| 1 | When OOW saw the sign of main power failure or received E/R notice, take immediate action to keep the ship away from danger. Show NUC lights and shapes if vessel traffic is many. 當船副發現主電源故障，或接到機艙通知時，立即採取行動，使船舶遠離危險。如果來往船隻眾多，應立即懸掛操縱失靈的號燈號標。 | ☐ |
| 2 | Confirm emergency power supply has been restored to gyro compass, steering stand and ECDIS, RADAR and VHF communication equipment etc. 立刻確認電羅經，舵機供電和電子海圖、雷達與VHF通訊設備，是否已經恢復緊急電力供應。 | ☐ |
| 3 | If main engine is not available in short interval, inform Fore and Aft station to stand by for emergency let-go anchor or take more tugs to assistance. 若主機無法立刻恢復正常，通知船頭人員準備緊急下錨，或加帶拖船協助。 | ☐ |
| 4 | Confirm whether vessels in vicinity will cause immediate danger or not, use sound signals as necessary. 確認附近船舶，不會造成立即的危險，必要時使用汽笛信號。 | ☐ |
| 5 | Steering ownship in outer limits of traffic lane if possible to avoid collision area. 盡可能沿著航道外側航行，以避免可能的碰撞區域。 | ☐ |
| 6 | Note the date/time and ship position on the chart and use remained speed to steady the vessel and keep away from danger，proceed to anchorage if any. 在海圖上記錄當時時間和船位，利用本船餘速，穩定航向，遠離危險，準備下錨待命。 | ☐ |
| 7 | Inform VTS or port authority with VHF - Channel 16., as appropriate. 視情況通過 VHF - 16 頻道通知信號台或港務局。 | ☐ |
| In coastal area, there may have danger of main engine loses power due to power trip (black out). 在近岸航行區，可能因突然的跳電，造成船舶主機失去動力的險情。 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1 | When OOW saw the sign of main power failure or received E/R notice, call captain immediately. 當船副發現主電源故障，或接到機艙通知時，立即通知船長上駕駛台。 | ☐ |
| 2 | Confirm whether vessels in vicinity will cause immediate danger. Show NUC lights and shapes if vessel traffic is many. 確認附近船舶，是否造成立即的危險。如果來往船隻眾多，應立即懸掛操縱失靈的號燈號標。 | ☐ |
| 3 | Confirm emergency power supply has been restored to gyro compass, steering stand and ECDIS, RADAR and VHF communication equipment etc. 立刻確認電羅經，舵機供電和電子海圖、雷達與VHF通訊設備，是否已經恢復緊急電力供應。 | ☐ |
| 4 | Send emergency or safety massage to all vessels by VHF 16, use sound signals as necessary. 發送VHF警急或安全信文給海面其他船隻，必要時使用汽笛信號。 | ☐ |
| 5 | Check water depth and distance from shore now, current direction and other navigational hazards. 檢查海圖上附近區域之水深、離岸距離、流水方向與其他航行險阻。 | ☐ |
| 6 | Note the date/time and ship position on the chart and use remained speed alter course to open sea and keep away from danger. 在海圖上記錄當時時間和船位，利用本船餘速，轉向開闊水域，使船舶遠離危險。 | ☐ |
| 7 | Sound general alarm, broadcasts to notify the main power failure, all crews pay attention to standby, if necessary. 如有必要鳴放警鈴，全船廣播，通知主電力故障，全船人員注意警戒待命。 | ☐ |
| As soon as the Master on bridge 船長上駕駛台後： |  |  |
| 1. | Confirm the navigational hazards in vicinity and safe drifting area immediately. 立即確認周圍環境與安全的漂流區域。 | ☐ |
| 2 | Judge vessel final stop position before main power not yet restore. 在主電力尚未恢復前，判斷船隻最後停止的位置？ | ☐ |
| 3 | Decide which side is suitable weather side (upwind or up-current) to drift from final stop position. 決定漂流時，何舷受風，才能遠離危險。 | ☐ |
| 4 | Use last steerage to adjust ship’s weather side (when the emergency power supplied). 利用船隻餘速，調整船首向正確受風(當緊急電力供應時)。 | ☐ |
| 5 | Estimate drifting speed and course if danger is nearby. 估計漂流的方向與速度，如果臨近危險。 | ☐ |
| 6 | If water depth and sea conditions are suitable, both anchors should be ready to let go at any time. 如果水深、海況條件合適，雙錨備便隨時準備下錨。 | ☐ |
| 7 | Ask Chief Engineer to estimate the approximate time needed for repairs to evaluate whether outside support is needed (Tug Boat assistant). 詢問輪機長預估大約需要修理的時間與情形，以決定是否需要外部的救援(如拖輪)。 | ☐ |
| 8 | Do not interrupt electricity emergency repair, continue to monitor progress of the emergency repair. 以不妨礙電源搶修的原則下，持續保持搶修進度的了解。 | ☐ |
| 9 | Has the ship’s safety assessment been done before taking any danger action? 在採取任何危險動作之前，是否都已做好船舶的安全性評估。 | ☐ |
| 10 | If the result of assessment requires onshore support or seriously affect to the ship's schedule, Master is required to send an ER. Refer to the "Emergency contact telephone list" and according to the safety management manual PR-08. Notify to Marine Technology Division, Operation Divisions, and Engineering Division etc. immediately for the related works and any supporting arrangement. 如評估需要岸上支援或嚴重影響船期，船長需發送ER。依照公司安全管理手冊PR-08規定，參考“緊急聯絡電話表＂立即將情況通知海技部、作業部、工務部，以利船期調度等後續作業與是否需要岸端協助及支援。 | ☐ |
| 11 | After main electric power restored, downloaded VDR, CCTV & ECDIS, kept related record such as bell book/Log book/Chart/Course recorder/Sounding record, etc. 當電力恢復後，下載VDR、CCTV及ECDIS並保留任何有關的記錄如俥鐘記錄、航行日誌、海圖、航向記錄、水深記錄等。 | ☐ |
| 12 | Continuous contact with the company and the agent in the next port to restore coastal schedule. 持續與公司代理行聯絡，以趕上失去的船期。 | ☐ |
| After the emergency action, Master shall pay special attention to collect the evidence and make the reports as the followings: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 13 | As per SMM PR-08, any accident must do the required reports as follows: 依據公司安全管理手冊所規定之意外事故所規定表格繕製下列報告: |  |
|  | - Statement of fact (in Chinese & English) with photos. | ☐ |
|  | - Sea Protest. | ☐ |
|  | - FM-0803-4, FM-0803-7. | ☐ |
|  | - Report to flag state. | ☐ |
|  | - Tank & cargo hold sounding record. | ☐ |
|  | - Copy of log book, bell book, telegraph, course recorder, weather chart etc.. | ☐ |
| 14 | All external reports must be reviewed by Company before sending out. 所有對外報告在寄出去前，都須經過公司審核。 | ☐ |

| Engine Room Emergency Response機艙緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| 1 | Carry out emergency measures according to the blackout emergency procedure to restore electric power supply. 根據停電應急程序，採取應急措施以迅速恢復電力供應。 | ☐ |
| 2 | Do risk assessment and pay attention to personnel safety during emergency repairs. 搶修過程中執行風險評估及注意人員安全。 | ☐ |
| 3 | Evaluate the time required for repairs and notify the Bridge. 評估修理所需時間並通知駕駛台。 | ☐ |
| 4 | After trouble solved notify the bridge for main engine test running and resume sailing when everything is normal. 故障排除後，通知駕駛台主機試車，一切正常後恢復航行。 | ☐ |
| 5 | Submit the incident report and attach video or photos description. 提交事故處理報告，並附上影片或照相說明。 | ☐ |
| 6 | Assess repairing parts required for malfunctioned generator set, and submit an emergency spare parts application to the engineering division spare parts department. 評估故障的發電機機組所需的配件，向工務部配件課提出緊急配件申請。 | ☐ |

| Shore Side-Emergency Response 岸端-緊急處置 |  |  |
| --- | --- | --- |
| Checking Items 檢查項目 |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  |  |
| PIC must confirm the followings as soon as receiving report from Master: PIC第一時間確認船端回報下列事項： |  |  |
| 1 | Any personal injury? oil pollution? If Captain need shore side urgent assistance?(Tug boat assistant) 是否有人員受傷? 是否有油污染?是否船上需要岸端緊急援助?(如拖輪協助) | ☐ |
| 2 | Have photos at scene been taken & ready to send back? VDR, CCTV & ECDIS been downloaded? 現場照片是否已拍攝？並準備寄回? VDR、CCTV及ECDIS是否已下載? | ☐ |
| 3 | Have notified the nearest coastal country & flag state? 是否有通知沿岸管轄國及船旗國? | ☐ |
| 4 | Have the risk assessment for overall emergency & ship’s response been effectively executed? 對所有緊急及船方處置的風險評估，已有效執行? | ☐ |
| 5 | Completed the initial report required by SSM department, include following items: 完成海技部船管課事故初始報告，包含以下項目: |  |
|  | (1) Date發生日期 | ☐ |
|  | (2) Time發生時間 | ☐ |
|  | (3) Position船舶位置 | ☐ |
|  | (4) Ship Status 船舶目前狀態 | ☐ |
|  | (5) Description of the incident 事故狀況說明 | ☐ |
|  | (6) Current handling progress and other department’s contact detail. 目前處理進度＆其他部門聯繫狀況說明 |  |
| 6 | When vessel is safe, Master Have sent the following initial accident reports according to SMM PR-0802: 在安全無虞時，船長是否已照安全管理手冊PR-0802程序，寄回下列初步事件報告： |  |
|  | (1) Statement of fact (in Chinese & English) with photos | ☐ |
|  | (2) Sea Protest | ☐ |
|  | (3) FM-0803-4, FM-0803-7 | ☐ |
|  | (4) Report to flag state | ☐ |
|  | (5) Tank & cargo hold sounding record | ☐ |
|  | (6) Copy of log book, VDR, ECDIS, weather chart etc.. | ☐ |
| 7 | MARTECH PIC contact with ship’s captain continuously & provide necessary emergent assistance, such as follow up the emergency spare parts delivery progress, until vessel is safe, repair & investigation finished. 海技PIC持續與船端緊密連絡，並協助緊急處置，如確認緊急配件送船進度，直到屬輪安全，以及後續維修與調查結束。 | ☐ |
| 8 | The finished report must be approved by company before sending to outside parties like flag state. 相關報告製作完成後，在發給外部單位，如船旗國前，須經公司主管批核。 | ☐ |