MAN OVERBOARD

人員落水緊急處理程序檢查表

| Ship Side-Emergency Response船端-緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| At any time, any personnel on board who observes a person falling overboard, must immediately report to the Duty Officer and the Master and must immediately take the following actions: 任何時候，任何在船人員一旦發現有人員落海時，應立即通報當值船副和船長，並立即採取下列措施： |  |  |
| 1. | Shout loudly to attract the attention of those nearby for possible rescue action. 大聲喊叫，以引起附近他人注意，採取任何可能的救援行動。 | ☐ |
| 2. | Throw overboard life-buoys, and if possible, attached with self-igniting light and smoke signal 將救生圈拋入海中，如有可能，連同自燃燈和煙霧信號一起拋入海中。 | ☐ |
| 3. | Keep closely your eyes on the person in the water 眼睛緊盯著落海人員。 | ☐ |
| Duty officer shall: When ship could be grounding, stop M/E or reduce speed and inform Master immediately. 當值船副應：當發現人員落水情況時，立即釋放該測救生圈及煙霧信號。 |  |  |
| 1. | Sound the distress emergency alarm for “Rescue Boat Station” 以通用警報器發出”救難艇佈署”的遇險。 | ☐ |
| 2. | Take the position of the lifebuoy and plot it on the chart. 觀測救生圈所在位置，並將之標示在海圖上。 | ☐ |
| 3. | Activate MOB mode of ECDIS. 啟動電子海圖之人員落水模式。 | ☐ |
| Maneuvering of the ship when approaching people overboard “Immediate action”: The person overboard is noticed from the bridge and action is taken immediately. “Delayed action”: The person is reported to the bridge by an eyewitness and action is initiated with some delay. “Person-missing action”: The person is reported to the bridge as missing. Different means of maneuvering of the ship are to be taken when approaching people overboard due to the change of maneuverability in ambient circumstances and different timing for taking actions. 接近落水人員的操船方法：駕駛室人員發現落水者，立即採取行動，稱「立即行動」；發現落水者，由目擊者報告駕駛台，經過一段時間後採取行動，稱「延遲行動」；發現落水者，報告駕駛台時發現目標丟失，稱「人員失蹤」；。由於外界環境影響下的操縱性能的變化和採取行動的時間不同，接近落水人員應採取不同的操船方法。 |  |  |
| 1. | Put the rudder hard over to the side of the person who has fallen overboard to avoid the danger of his being struck by the propeller, and to start to perform (1) Single turn for immediate action or person in sight (2) Williamson turn person for delayed action or person not in sight. 向人員落海的舷側操滿舵，以避免俥業打傷落水人員，並開始採取：(1)立即行動或人員在視線內，採取單旋迴法。(2)延遲行動或人員已不在視線內，採取威廉遜氏迴轉法。 | ☐ |
| 1.1 | Applicable to “immediate action” for the man overboard always in sight, fastest and most effective way of approaching the target, not applicable to “delayed action” or “person-missing action”. 單旋迴法(Single turn)：本法最適用於”立即行動”，是落水人員始終在視線中，船舶接近落水人員最快、最有效的操縱方法，但不適用於”延遲行動”和”人員失蹤”。 | ☐ |
|  | Stop, rudder hard over to the side of the casualty. 停車，向落水者一舷操滿舵。 | ☐ |
|  | Accelerate after the person overboard passes the stern. 落水者過船艉後，進車加速。 | ☐ |
|  | when heading 20º from the target, rudder to midship position, slow down and stop as appropriate, turning towards the upwind side of the target through inertia, heading steady, approaching the target. 當船艏轉至距落水者差20度時，正舵，減速，適時停車，利用慣性轉至對準落水者上風側，把定，接近落水者。 | ☐ |
|  | if it is hard to locate the target, rudder to midship position after deviation from the original course by 250º slow down and search for the target; once the target is found, stopping; maneuver to be initiated immediately and approach to the upwind side of the target. 落水者位置難於確認時，應在船首向轉過 250 度時，正舵，邊減速邊努力搜尋落水者，發現後立即停車駛向落水者上風側。 | ☐ |
|  |  | ☐ |
| 1.2 | Williamson turn：this turn can make good original track line, good in reduced visibility or at night, applicable to “delayed action”. 威廉遜迴轉：本法能準確地把船轉回到原航跡線上，在夜間或能見度不良時是一種有效的方法，最適用於”延遲行動”。 | ☐ |
|  | stop, rudder hard over to the side of the casualty. 停車，向落水者一舷操滿舵。 | ☐ |
|  | accelerate after the person overboard passes the stern. 落水者過船尾後，進車加速。 | ☐ |
|  | after deviation from the original course by 60º rudder hard over to the opposite side. 當船首轉過60度時，回舵並操另一舷滿舵。 | ☐ |
|  | when heading 20ºshort of opposite course, rudder to midship position and ship to be turned to opposite course; searching while proceeding, slow down as appropriate and stop when the target is found, approaching the target. 當船首轉到與原航向之反航向差20度時，正舵，待轉到原航向的反航向時把定，邊搜索邊前進，發現落水者後適時減速停車，駛近落水者。 | ☐ |
|  |  | ☐ |

| As soon as the Master on bridge: 船長上駕駛台後： |  |  |
| --- | --- | --- |
| 1. | Upon receipt of the report from the Duty Officer, the Master must immediately go to the bridge and give the order for a “Rescue Boat Station” and must take the following measures: 船長接獲當值船副的報告後，應立即到駕駛台下達”救難艇佈署”的命令，並採取下列措施： | ☐ |
| 2. | Identify the location of the person falling overboard. 確認人員落海的位置。 | ☐ |
| 3. | Increase the number of look-out members. 加強瞭望人員。 | ☐ |
| 4. | Carry out the Single turn or Williamson turn to maneuver the ship close to the lifebuoy in the water. 以單旋回或或威廉遜氏迴轉法操船駛近水中救生圈。 | ☐ |
| 5. | Call for the attention of ships in the vicinity by using VHF Channel 16. 使用VHF，CH16上呼叫鄰近船舶注意。 | ☐ |
| 6. | Sufficient lookouts are to be arranged to ensure that the recovery team will be able to communicate readily with the Bridge team and the location and condition of people in distress are to be reported to the Bridge and recovery team to avoid the recovery target being run down or crushed. 足夠的人員，負責瞭望和聯絡，確保駕駛室和現場營救人員隨時保持聯繫，並向駕駛人員和現場營救人員不斷報告落水人員的位置和狀況，避免船舶碰撞或擠壓營救目標。 | ☐ |
| 7. | The ship is to be upwind to keep sufficiently off the wind to minimize swinging and to create a lee for those in distress so that it is easier for them to approach the recovery ship. 舶駛向遇險人員的上風處，為其提供避風保護，並儘量減少船舶的受風面積，減少船舶的搖擺幅度，使落水人員更容易靠近營救船舶。 | ☐ |
| 8. | In heavy weather, the use of oil for reducing the effect of the sea may be considered. Experience has shown that vegetable oils and animal oils, including fish oils, are most suitable for quelling waves. Lubricating oils may be used. However, fuel oil is not to be used, except as a last resort, as it is harmful to persons in the water. 惡劣天氣時，可考慮使用油來減少海況的影響，經驗表明，植物油和動物油最適合用於鎮浪，也可以使用滑油，除非無其他辦法，否則不得使用燃油，因為燃油對水中人員有害。 | ☐ |
| 9. | In compliance with SMM PR-08, any accident happened, notify Maritech immediately referring to “Emergency Contact List” and send ER for providing timely assistance. 依SMM PR-08規定，當事件發生時，船長應參考“緊急聯絡電話表＂立即通知海技課及發ER，以利提供適時的援助。 | ☐ |
| 10. | At early stage of the emergency, Master shall notify costal state or flag state, the notification shall include the type of the accident and what kind of assistance required. 緊急事件初期，船長應通知最近的沿岸國及船旗國當局，通報內容包括事件類型及需要何種協助。 | ☐ |
| Planning for recovery：Recovery of persons from the water is an emergency event. The circumstances you find when you arrive at the scene will differ from incident to incident; but general planning can, and is to, be done. Effective recovery will occur, making sure that everyone understands the plan and their own place in it and full preparation is made. Due regard is to be paid to: 營救計畫的制定：營救落水人員是突發事件，不同的事故現場情勢不同，因此應制定一個總體營救計畫，確保每個人都明白這個計畫和自己的任務，並做好充分的準備，才能有效地實施營救目的。計畫應充分考慮到下列事項。 |  |  |
| 1. | Current sea conditions, number, condition, location and disposition of persons in the water. 當前的海況，落水人員的數量、狀況、位置及分佈。 | ☐ |
| 2. | Risk to persons involved in recovery operations, appropriate recovery equipment onboard. 營救人員的風險，船上合適的營救設備。 | ☐ |
| 3. | Duties and divisions of onboard persons involved in recovery operations. 船上參與營救人員的職責和分工。 | ☐ |
| 4. | How to proceed to recovery scene and to conduct sufficient risk identification. 如何接近營救現場，並做出充分的風險識別。 | ☐ |
| 5. | Assistance provided for persons in the water and preparations prior to recovery. 營救前對落水人員可提供的援助和營救前的準備。 | ☐ |
| 6. | Recovery operation region and the selection of embarkation point; 實施營救操作區域和登乘點的選擇。 | ☐ |
| 7. | Ways to bring persons to the side of the ship and get aboard the ship. 將落水人員帶至船側和營救上船的方式。 | ☐ |
| 8. | Shelter and care of persons after they are aboard. 落水人員上船後的安置和照顧。 | ☐ |
| 9. | Standing by when persons cannot be recovered. 無法實施營救時，執行對落水人員的守護。 | ☐ |
| 10. | Refer to CK-0802-1 Checklist for man overboard search and rescue. 參照CK-0802-1 Checklist for man overboard search and rescue實施。 | ☐ |
| Send person for stand-by lifeboat and Order the rescue team to prepare as following: 指派人員備便救生艇並請救難隊準備下列事項： |  |  |
| 1. | Prepare Jacob’s ladders, safety nets and heaving lines. 準備輕便繩梯、安全網和撇纜繩。 | ☐ |
| 2. | Stand-by derrick cranes and net slings for lifting the person out of water. 備便吊桿和吊網，以便將落海人員從水中吊起。 | ☐ |
| 3. | Life-saving appliances: line-throwing apparatus, lifebuoys, lifejackets, immersion suits, rescue boats and liferafts. 救生設備：如拋繩器、救生圈、救生衣、救生服、救助艇和救生筏等。 | ☐ |
| 4. | Detection aids such as high-visibility/retro-reflective material, lights, a SART and an EPIRB. 探測定位裝置：如易於觀察/反光材料、燈具、搜救雷達應答器、應急示位標等。 | ☐ |
| 5. | Communications equipment: VHF, MF/HF, handheld radiotelephone. 通信設備：如 VHF、MF/HF、手持無線電話等。 | ☐ |
| 6. | Signaling equipment: signaling lamps, searchlights, signaling flags, loud hailer; 信號設備：信號燈、探照燈、信號旗、揚聲器等。 | ☐ |
| 7. | Medical assistance: shelter, stretcher, blanket, drink, food, medicine chest. 醫療救助設備：如居所、擔架、毛毯、飲料、食品、急救藥箱等。 | ☐ |
| 8. | Lower down rescue boat if necessary. 必要時放救難艇接近落水人員。 | ☐ |
| The master is to assess the navigational hazards on scene, having due regard to the maneuverability of the recovering ship, on scene weather conditions, location, number and condition of persons in the water. Appropriate angle is to be selected based on the directions of wind and current and the ship is to be maneuvered carefully at low speed when approaching the recovery target. Following measures are to be taken to minimize risk of harm to recovery personnel and people in the water during the recovery operation. 船長應評估現場的航行風險，充分考慮到船舶本身的操作性能、營救現場的天氣和環境狀況、落水人員在水中的位置及落水人員的數量和狀況等。根據風和流的方向，選擇適當的角度，謹慎低速駕駛自己的船舶靠近營救目標。在營救過程中應採取如下措施減少對營救人員和落水人員的傷害： |  |  |
| 1. | To maneuver your own ship to prevent the recovery target being run down, crushed, capsized or swamped by the recovery ship. 謹慎操縱自己的船舶，防止營救目標被營救船虹吸、碰撞、傾覆或淹沒。 | ☐ |
| 2. | Try to keep sufficiently off the wind to reduce the ship’s roll and pitch and to create a lee. 保持船舶最少受風面，減少船舶的搖擺和傾斜，為落水人員提供一個有效的背風庇護區域。 | ☐ |
| 3. | A liferaft can be deployed at the foot of the ladder or net, to act as a transfer platform. 將救生筏佈置在舷側登乘梯或網具的底部，為被營救者提供一個中間轉移平台。 | ☐ |
| 4. | Fenders are to be rigged in the embarkation area to minimize the harm to the recovered people due to swinging against the ship’s side. 在落水人員登乘區域的舷側佈置碰墊，最大程度減少被營救人員與舷側碰撞造成的傷害。 | ☐ |
| 5. | When lifting people, safety lines are always to be used to secure the casualty in case he/she is injured and/or falls and control lines are to be rigged to the hoist and tended in an effort to minimize swinging. 在提升被營救人員時，應使用救助吊帶或安全帶，防止人員跌落或損傷，並應系有控制繩索，減少被營救者在提升過程中產生的搖擺幅度。 | ☐ |
| 6. | People who have been in the water, the injured and the incapable, are to be lifted in a horizontal or near-horizontal position if possible (for example, in a basket, or in two strops; one under the arms, the other under the knees). Recovery in a vertical position is to be avoided whenever possible as it risks cardiac arrest in hypothermic casualties. 處於水中的倖存者可能已經受傷或無行為能力，被提升時應盡可能處於水準或接近水準的狀態，可以使用兩根吊帶，一根從手臂下穿過，另一根從膝下穿過，任何時候都應避免落水人員在營救過程中呈現直立的姿勢，因為那樣做體溫過低的傷患會有心臟驟停的危險。 | ☐ |
| 7. | To maneuver your own ship to prevent the recovery target being run down, crushed, capsized or swamped by the recovery ship. 謹慎操縱自己的船舶，防止營救目標被營救船虹吸、碰撞、傾覆或淹沒。 | ☐ |
| Shelter and care of people recovered對落水人員營救後的安置和照顧: |  |  |
| 1. | The recovered people are to be transferred to a safe shelter as soon as possible and care medical supplies are to be provided by a dedicated person after they are aboard. 遇險人員登船後，應儘快轉移到安全的庇護處所，由專人負責護理和提供醫療救助。 | ☐ |
| 2. | Information on persons in distress are to be asked from those conscious survivors as far as possible since the information they know may provide more help for the recovery operation. The master and on-scene operation commander are to be informed of the information known in time. 盡可能向意識清醒的倖存者瞭解遇險人員的情況，他們介紹的情況，可能對本次營救行動提供更多的幫助。應將瞭解到的資訊及時告知船長和現場營救指揮人員。 | ☐ |
| Standing by when people cannot be recovered: When recovery cannot be attempted or completed without unduly endangering the ship due to bad weather or other reasons, the master has the right to decide to terminate the recovery operation, and care and assistance are still to be given to those in distress within the capabilities of the ship until other help arrives or weather conditions improve. The following measures are to be taken by the master to assist people in the water. 無法對營救人員的守護： 由於天氣狀況惡劣或其他原因，無法在不危及本船安全的情況下進行營救時，船長有權作出終止營救的決定，留在現場對落水人員進行守護和提供力所能及的幫助，直到其他的救助單位到達或天氣狀況得到改善。船長應採取如下措施為落水人員提供幫助。 |  |  |
| 1. | Providing updated and detailed reports on the situation to local Rescue Co-ordination Centre (RCC). 及時向所在區域的救助協調中心(RCC)提供現場最新的、詳細的情況彙報和其他相關資訊。 | ☐ |
| 2. | if possible, using throwing rockets or heaving lines to pass lines to boats, rafts to be rescued and towing the boats or rafts to a safe position. 如可能，使用火箭拋繩器或引纜等手段將纜繩拋到待救助的艇、筏上，將待救助的艇筏拖向一個相對安全水域。 | ☐ |
| 3. | Creating a lee for people in the water with your own ship. 利用自己的船舶為落水人員提供一個有效的背風庇護區域。 | ☐ |
| 4. | posting sufficient look-outs to keep people in the water in sight. 安排足夠的瞭望人員，保持落水人員一直處在視線範圍內。 | ☐ |
| 5. | Establishing communications with people in the water as far as possible to provide them with comfort. 盡可能與落水人員建立通訊聯繫，給予落水人員精神上的支持。 | ☐ |
| 6. | providing life-saving equipment to people in the water as far as possible, such as lifebuoys, liferafts, medical supplies, water and food. 盡可能向落水人員提供救生設備，如救生圈、救生筏等，以及醫療用品和其它物品如水、食品等。 | ☐ |
| 7. | assisting other recovery units in the recovery operation. 協助其他救助單位營救落水人員。 | ☐ |
| In the case if a person is found missing without witness, the following action steps must be taken: 在無目擊者的情況下，若有人員失蹤案件，必須採取下列步驟： |  |  |
| 1. | Taking note of day and time, and the ship’s position when the missing person is assumed to have gone missing and retain this evidence. 將人員失蹤的推測日期、時間和船位記錄下來，並將之保留作為證據。 | ☐ |
| 2. | Take a photograph of any material evidence relating to the disappearance of the missing person and leave the related cabin undisturbed, if possible. 拍攝與人員失蹤有關的任何物證，如果可能，相關得房間保持原狀。 | ☐ |
| 3. | Report the disappearance to the company 向公司報告人員失蹤事件。 | ☐ |
| 4. | Call for the attention and rescue operations to the nearest search and rescue organization or authorities 利用VHF Ch16，通知附近船舶注意並要求給予可能的協助。 | ☐ |
| 5. | Perform search and rescue operations as appropriate. 執行適宜的搜索救助作業。 | ☐ |
| After the MOB, Master shall pay some special attention to collect the evidences and make the reports as the following: 人員落水事故發生後，船長應注意收集之証據及繕製之報告如下： |  |  |
| 1. | As per SMM PR-08, any accident, the required report forms are FM-0803-5 (if any), FM-0803-6, FM-0803-7, statement of fact & sea protest. 依據公司安全管理手冊所規定之意外事故所規定表格繕製意外事故報告FM-0803-5(如有)、FM-0803-6、意外事故調查報告FM-0805-7以及STATEMENT OF FACT、SEA PROTEST呈報公司； | ☐ |
| 2. | All charting work including the fixes before the accident. 所有持續到人員落水當時的駕駛台海圖作業，包括事故前繪於海圖上的船位； | ☐ |
| 3. | Master shall ensure all the fixes on using charts, no forgery. 使用之海圖船長務必確定海圖上之船位，避免塗改； | ☐ |
| 4. | Telegraph or other RPM output recording. 車鐘記錄器或其他主機引擎輸出有関運轉之記錄器。 | ☐ |
| 5. | Confirm that bridge clock time match with engine room’s and other auto recording. 駕駛台上船鐘與機艙是否正確一致及其他自動記錄器之確認； | ☐ |
| 6. | Course recorder’s time and course at the moment of MOB and any weather forecast. 航向記錄器所指示之人員落水時間，航向指示，氣象指示； | ☐ |
| 7. | Take good care of VDR & CCTV’s recording( as VDR will continue recording for 12 hours, after Master contacting with the authority concerned, stop the recording and take out CF Card for later investigation) 妥善保存VDR、CCTV之記錄(因VDR會連續紀錄12HRS，在船長與主管機關聯繫後，再將記錄器停止，並取出CF CARD以供後續調查使用)。 | ☐ |
| 8. | The position fixed by radar operator. 雷達操作者所做的船位記錄。 | ☐ |
| 9. | Video or photos for the accident. 事故經過及現場的影片及照片。 | ☐ |
| 10. | The statement in deck log and engine log shall be coherent (coordinate with the Company to record only the favorable part). 在甲板DECK LOG 及ENG LOG記錄應協調一致(事先與公司協調記載有利部份)。 | ☐ |
| 11. | The description of MOB accident shall include all witnesses and duty persons, statement of fact shall attach photos as much as possible, the information shall include: 人員落水事故過程之描述，包括所有目擊及當事人之報告，STATEMENT OF FACT儘可能附上事故現場照片，收集呈報公司之資料証據包括： | ☐ |
|  | - The actual or estimated time of MOB; 人員落水之確實或推算時間 | ☐ |
|  | - Ship’s position/course/speed/rpm at time of MOB; 人員落水當時之船位/航向/航速/轉速記錄。 | ☐ |
|  | - Engine/steering maneuvering/time and condition after MOB; 人員落水後用車/用舵/時間及狀況。 | ☐ |
|  | - Estimated prevailing visibility/the time of signal heard or seen and its relative position; 當時預估能見度/註明看到或聽到信號時間及相關船位。 | ☐ |
|  | - The prevailing weather/wind direction, wind speed/direction of wave and height/swell direction and height/visibility/received weather forecast. 當時天氣狀況/風向、風速/浪之方向與高度/湧之方向與高度/能見度/收受之天氣預報。 | ☐ |
|  | - The condition of tide and current. 潮、流之情形。 | ☐ |
|  | - Bridge composition before man overboard or immediately on bridge upon MOB. 人員落水前在駕駛台上或立即上駕駛台之人員配置。 | ☐ |
|  | - Positioning fix system. 使用之定位系統。 | ☐ |
|  | - Ships recording of response action from the beginning till MOB(including operation of M/E); 船舶從開始至發現人員落水之時間內反應動作之記錄(含主機操作)。 | ☐ |
|  | - When released fog signal or signal heard and characteristics of operation. 何時施放及聽到之音響信號和操作特性。 | ☐ |
|  | - The other passing ship’s name in the vicinity at time of MOB. 事發當時鄰近他船之船名。 | ☐ |
|  | - The damage condition of ship and cargo, with attached photos, if possible. 船貨損壞情況，儘可能附照片。 | ☐ |
|  | - Deck, engine bell book, telegraph record. 甲板，機艙之車鐘記錄簿，車鐘記錄圖紙。 | ☐ |
|  | - Course recorder record. 航向記錄。 | ☐ |
|  | - Used chart at the time. | ☐ |
|  | 當時使用海圖。 |  |
|  | - Other records relating to maneuverability, compass error adjustment. 其他有関操船能力，羅經誤差校準等記錄。 | ☐ |
|  | - Master’s sea protest shall be endorsed(only state the part favorable to the company). 船長之海事報告(Sea Protest)，須加以簽證(只記錄對我有利之部份)。 | ☐ |
|  | - Download data of case form VDR/CCTV. 下載事情發生整體經過的VDR/CCTV資料 | ☐ |
| 12. | Report to flag state in 24 HR. 事發24小時內向船旗國回報。 | ☐ |
| 13. | All reports must be checked by company. 所有報告內容都須經過公司審議。 | ☐ |

| Shore Side-Emergency Response岸端緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目內容 |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  | ☐ |
| 1. | PIC must confirm the following as soon as receiving report from Captain: PIC第一時間確認船端回報下列事項： | ☐ |
|  | - How many members overboard? 船員落水人數? | ☐ |
|  | - What time MOB and action is delayed or immediate? 船員落水時間?立即發現或是延遲發現? | ☐ |
|  | - The position about MOB and ship? 船舶位置與人員落水位置? | ☐ |
|  | - The local weather and visibility? 當地天氣狀況，能見度距離。 | ☐ |
|  | - Can find or visual the member around sea? 是否能夠尋獲落水人員或目視於海上? | ☐ |
| 2. | Based on the category of the accident, the related staff and the person in charge of the ship are to be selected and summoned. For example, selecting the Head of the Crew department for an accident of crew injury and Head of the Engineering division for an accident related to engine trouble. 基於事故種類，有關人員和負責船舶人員將被選擇並且召集。例如，當發生船員受傷事故時，選擇船員課最高長官，以及當機器故障時，則召集工務部最高長官。 | ☐ |
| 3. | The personnel selected and summoned must treat the emergency as a top priority and under the command of the Designated Person, should duly take action against the accident. 被選擇及召集人員必須把緊急情況當作第一優先，並聽命於指定管理人員，應該按時對事故採取行動。 | ☐ |
| 4. | The leader of the “Emergency Response Team” is to make conclusion and to determine countermeasures and inform the Master as soon as possible. 緊急事故處理小組的領導人做結論，並且確定對策，盡快通知船長。 | ☐ |
| 5. | Have photos at scene(including MOB scene) been taken & ready to send back? VDR, CCTV & ECDIS been downloaded? 相關事故照片(包括落水現場照片)是否已拍攝並準備寄回？VDR、CCTV及ECDIS是否已下載？ | ☐ |
| 6. | Have the risk assessment for overall emergency response & rescue been effectively executed? 對所有緊急處置及人員施救措施之風險評估是否已有效執行？ | ☐ |
| 7. | Have notified the nearest coasting country & flag state & MRCC? 是否已通知沿岸最近管轄國、船旗國與搜救協調中心(MRCC)？ | ☐ |
| 8. | Have sent the following initial accident reports according to SMM PR-0802 in safety situation: 是否已照安全管理手冊PR-0802程序，在安全無虞下，寄回下列初步事件報告： | ☐ |
|  | - Statement of fact(in Chinese & English) with photos | ☐ |
|  | - Sea Protest | ☐ |
|  | - The chart of man over board position | ☐ |
|  | - FM-0803-5(if any), FM-0803-6, FM-0803-7 | ☐ |
|  | - VDR/CCTV data | ☐ |
|  | - Report to flag state | ☐ |
|  | - Copy of log book, bell book, telegraph record, course recorder, etc. | ☐ |
| 9. | PIC continues closely contacting with ship’s captain & provides necessary emergent assistance until the fleet vessel is safe & the case is over. 海技PIC持續與船端緊密連絡並協助緊急處置，直到屬輪平安事件結束為止。 | ☐ |
| 10. | After PIC investigate the condition of scene, must check person, thing, time, place, object is the same with record of ship side. 後續海技PIC至現場調查事件發生情況，須針對人事時地物是否與船上紀錄相同。 | ☐ |
| 11. | Interview with all the persons related to the accident to confirm the details. 與事件相關人員當面訪談以確認事件經過。 | ☐ |
| 12. | The finished report must be approved by the company before sending out to the parties concerned like flag state. 相關報告製作完成後，在發給相關單位如船旗國前，須經公司主管批核。 | ☐ |