Flooding Checklist

船舶浸水緊急處理程序檢查表

| Ship Side-Emergency Response船端-緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1. | After receiving massage about flooding(like bilge alarm), then inform the Master immediately. 當駕駛員收到通知發現船舶浸水情況時(如污水警報)，立即通知船長。 | ☐ |
| 2. | Check if any immediate dangerous situation in the vicinity. 確認附近是否有其他立即危險。 | ☐ |
| 3. | Check chart depth & object & other information around ship position. 檢查海圖上附近區域之水深、物件與其他必要資訊。 | ☐ |
| 4. | Record ship position and mark it on chart. 紀錄當時船位並標示在海圖上。 | ☐ |
| 5. | Check where the bilge alarm is activated. 檢查污水警報起自何處。 | ☐ |
| 6. | Notify E/R for standing by engine. 通知機艙備車。 | ☐ |
| As soon as the Master on bridge 船長上駕駛台後： |  |  |
| 1. | Ask CO to send out damage control team to check. 指派大副與損害管制隊前往現場評估。 | ☐ |
| 2. | Check hull damage about position, size and flooding condition. 確認破損位置、大小，進水情況。 | ☐ |
| 3. | Trace the cause of flooding and assess if any possible means for stopping water flooding. 追查淹水的原因並評估是否可以停止繼續浸水。 | ☐ |
| 4. | Sound general emergency alarm and broadcast this case to all crew at once. 廣播全船警報並通告全體船員本輪淹水情況。 | ☐ |
| 5. | Pump out water of flooding area by ballast pump or GS pump, if necessary. 若情況許可利用壓載系統或GS幫浦將浸水區域進行排水。 | ☐ |
| 6. | Muster all crew in ship damage control station to proceed following measures. 應立刻召集全體船員就損害管制部署，並準備下列事項。 | ☐ |
| 7. | In compliance with SMM PR-08, any accident happened, notify Maritech immediately referring to “Emergency Contact List” and send ER for providing timely assistance. 依安全管理手冊PR-08規定，當事件發生時，船長應參考“緊急聯絡電話表＂立即通知海技部及發ER，以利提供適時的援助。 | ☐ |
| 8. | (IF NECESSARY) At early stage of the emergency, Master shall notify costal state or flag state, the notification shall include the type of the accident and what kind of assistance required. (需要的情況下)緊急事件初期，船長應通知最近的沿岸國及船旗國當局，通報內容包括事件類型及需要何種協助。 | ☐ |
| 9. | Execute risk assessment & ensure the following: 執行風險評估並確認下列事項： |  |
|  | - Mustered & deployed all crew in damage control station. 是否已令所有船員依照規定進入損害控管部屬。 | ☐ |
|  | - Closed watertight doors and automatic fire door. 水密及自動防火門已關閉。 | ☐ |
|  | - Sent damage control team and examined around the ship. 已指派損害控管隊去檢查船舶周遭情況。 | ☐ |
|  | - Switched on deck lighting at night when engine stopped. 若停車時，夜間開啟甲板燈。 | ☐ |
|  | - Sounded & confirmed all tanks and bilge wells condition. 已測量並確認所有油艙與汙水艙狀況。 | ☐ |
|  | - Confirmed the condition of engine. 已確認主機情況。 | ☐ |
|  | - Checked the condition of all navigation equipments. 已檢查所有航海儀器之情況。 | ☐ |
|  | - Downloaded VDR, CCTV, kept related record such as bell book/Log book/Chart/Course record/Sounding record, etc. 已下載VDR、CCTV並保留任何有關的紀錄如航行日誌、海圖、航項紀錄、量水紀錄等。 | ☐ |
|  | - Ship’s stress force and stability assessment. 已執行船體應力與穩度評估。 | ☐ |
|  | - Sent radio warnings to ship’s in the vicinity to keep clear. 已無線電通知附近可視船舶保持距離。 | ☐ |
|  | - Check if flooding cause oil spill overboard, activate SOPEP. 確認是否因為浸水情況而造成油污污染，並啟動SOPEP程序。 | ☐ |
|  | - The injuries or overboard of any crew and any Fire happened. Is it necessary to execute rescue, first aid including CPR? 確認本輪是否有人員受傷或落海和是否有任何火警發生。有無需要執行救助或急救(包括CPR)? | ☐ |
|  | - Sent ER, Contacted with PIC and Coast Guard for further assistance, prepared the abandon ship procedure (in case if necessary). 確認已發送ER報、通知PIC以及相關單位請求更多幫助，如果有需要情況下則準備棄船部署。 | ☐ |
|  | - Stability should be assessed when taking any action. 在採取任何動作之前是否都已做船舶的穩定度評估。 | ☐ |
|  | - Chang Trim, Reduce the draught of the ship & make even keel & pump out water if necessary. 有必要情況下，調整Trim、減低船舶吃水或Even keel，打水以減低船體損傷。 | ☐ |
|  | - Find the reason about flooding. 查明可能浸水之原因。 | ☐ |
|  | - Record the height of water at flooding area. 記錄浸水高漲的速度。 | ☐ |
|  | - Record the draft every hour. 每小時觀察船舶吃水的變化。 | ☐ |
| Once oil spill overboard若油污漏洩入海 |  |  |
| 1. | According to SOPEP, from the information provided by damage control team, appraise the extent of the damage and immediately proceed the leaking-block, confirm other tanks watertight to avoid the damage spreading; any oil spill due to the breakage of the tank, stop the pollution according to HSE PR-08 Emergency Plan: Pollution Prevention; and prevent fire and explosion from oil spill; any spill overboard, report to parties concerned as per SOPEP’s procedure and format. 依據船舶油污染應急計畫(SOPEP)，以損害管制組回報之資訊，評估損害程度並指揮損害管制組進行堵漏、確保其他艙間水密性以防止損害擴大；有油料因破損溢出時，依照HSE PR-08 Emergency Plan: Pollution Prevention進行油污染防阻，並避免因溢油產生之爆炸與火災；若有任何油污漏洩入海，須照SOPEP程序通報有關單位。 | ☐ |
| 2. | Make risk assessment for oil spill overboard & confirm the oil pollution condition and take photos. 執行油污漏洩入海風險評估，確認油污污染狀況並拍攝現場照片。 | ☐ |
| 3. | Muster all crew for the oil pollution control. 召集船員執行防油污污染部署。 | ☐ |
| 4. | Dispatched an initial oil spills report to TPE / Maritech & local related Dept. 發送初步油汙汙染報告至海技部及與當地相關單位。 | ☐ |
| 5. | PIC of Legal & INS Dept. to be informed to P & I Club and Hull insurance to arrange oil pollution clear operation. 法保PIC通知P&I Club 履行保險契約準備清理油汙汙染作業。 | ☐ |
| 6. | Inform Agent to arrange for any emergency assistance. 通知代理，準備任何緊急情況之協助。 | ☐ |
| After the emergency action, Master shall pay some special attention to collect the evidences and make the reports as the following: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 1. | As per SMM PR-08, any accident, the required report forms are, FM-0803-6, FM-0803-7, and statement of fact & sea protest. 依據公司安全管理手冊所規定之意外事故所規定表格繕製意外事故報告(FM-0803-6)、意外事故調查報告(FM-0803-7)以及Statement of Fact、Sea Protest呈報公司。 | ☐ |
| 2. | Take good care of VDR & CCTV’s recording( as VDR will continue recording for 12 hours, after Master contacting with the authority concerned, stop the recording and take out CF Card for later investigation). 妥善保存VDR、CCTV之記錄(因VDR會連續紀錄12HRS，在船長與主管機關聯繫後，再將記錄器停止，並取出CF CARD以供後續調查使用)。 | ☐ |
| 3. | Video or photos for the accident. 事故經過的拍攝影片及照相。 | ☐ |
| 4. | The description of flooding shall include all witnesses and duty persons, statement of fact shall attach photos as much as possible, the information shall include: 船體浸水事故過程之描述，包括所有目擊及當事人之報告，STATEMENT OF FACT儘可能附上事故現場照片，收集呈報公司之資料証據包括： |  |
|  | - The flooding position/speed/condition. 浸水的位置/速度/重量。 | ☐ |
|  | - Ship’s position/course/speed/rpm and course recorder reading at time. 當時之船位/航向/航速/轉速及當時航向記錄器所顯示之記錄。 | ☐ |
|  | - Engine/steering maneuvering/time and condition. 當時的用車/用舵/時間及狀況。 | ☐ |
|  | - The prevailing weather/wind direction, wind speed/direction of wave and height/swell direction and height/visibility/received weather forecast. 當時天氣狀況/風向、風速/浪之方向與高度/湧之方向與高度/能見度/收受之天氣預報。 | ☐ |
|  | - Positioning fix system. 使用之定位糸統。 | ☐ |
|  | - Ships recording of response action at time (including operation of M/E). 發現浸水時，反應動作之記錄(含主機操作) 。 | ☐ |
|  | - The damage condition of ship and cargo on both ships, with attached photos, if possible. 有船貨損壞情況，儘可能附照片。 | ☐ |
|  | - Master’s sea protest shall be endorsed. 船長之海事報告(Sea Protest)，須加以簽證。 | ☐ |
|  | - If arranged surveyor for damage inspection, shall prepare GA and shell expansion drawing. 如有安排驗船師SURVEYOR登輪作損壞檢定，應備妥GA及SHELL EXPANSION圖。 | ☐ |
|  | - CLASS SURVEYOR : to assess ship’s seaworthiness: it’s better to be | ☐ |
|  | arranged by the Company. 用於判定船體適航性、最好由公司決定是否僱請。 |  |
|  | - INSURANCE SURVEYOR: to evaluate the damage, it’s separated to hull surveyor and P & I surveyor; no assessment for liability. 作損傷鑑定、可分船體 SURVEYOR 及 P & I SURVEYOR、不作責任判斷用。 | ☐ |
|  | - Lawyer: shall identify for whom he represents. 律師 (LAWYOR, SOLICITOR, ATTORNEY, ADVOCATE均是)：須認清代表何方。 | ☐ |
|  | - Salvage Superintendent or Salvage Surveyor, etc. 救助負責人或檢查員。 | ☐ |
|  | - Coast Guard or Port authority investigator. 海岸巡防隊或港務局/海事處調查人員。 | ☐ |
|  | - Report to flag of country in 24 HR. 事發24小時內向船旗國回報。 | ☐ |
|  | - .All external report must check by company. 所有對外報告內容都須經過公司審議。 | ☐ |

| Shore Side-Emergency Response岸端緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目內容 |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  | ☐ |
| 1. | PIC must confirm the following as soon as receiving report from Captain: PIC第一時間確認船端回報下列事項： |  |
|  | - The flooding position & condition. 船體浸水位置及浸水情況。 | ☐ |
|  | - Have the damage part of flooding been controlled? 是否受損漏水處已被控制？ | ☐ |
|  | - The draft of fore and astern? 艏艉現在吃水？ | ☐ |
|  | - Have all machine, equipments and tanks condition been checked & confirmed? 所有機器、設備及tank情況是否已檢查並確認？ | ☐ |
| 2. | Any personal injury? Any oil pollution? Container leaking? Fire? If need shore side urgent assistance? 是否有人員受傷？ 是否有油污染？是否有貨櫃漏洩？是否發生火災？是否需岸端緊急援助？ | ☐ |
| 3. | Have photos at scene been taken & ready to send back? VDR, CCTV & ECDIS been downloaded? 現場照片是否已拍攝並準備寄回？VDR、CCTV及ECDIS是否已下載? | ☐ |
| 5. | Have the risk assessment for overall emergency response & ship’s stability been effectively executed? 對所有緊急處置及船舶穩定性的風險評估是否已有效執行？ | ☐ |
| 6. | Have notified the nearest coasting country & flag state? 是否已通知沿岸最近管轄國及船旗國？ | ☐ |
| 7. | Have sent the following initial accident reports according to SMM PR-0802 in safety situation: 是否已照安全管理手冊PR-0802程序，在安全無虞下，寄回下列初步事件報告： |  |
|  | - Statement of fact(in Chinese & English) with photos | ☐ |
|  | - Sea Protest | ☐ |
|  | - FM-0803-6, FM-0803-7 | ☐ |
|  | - Report to flag state | ☐ |
|  | - Tank sounding record |  |
|  | - Copy of log book, bell book, telegraph record, course recorder, etc. | ☐ |
| 8. | PIC continues closely contacting with ship’s captain & provides necessary emergent assistance until the fleet vessel is safe & finish repair & investigate case over. 海技PIC持續與船端緊密連絡並協助緊急處置，直到屬輪安全後。以及後續的維修與調查事件結束為止。 | ☐ |
| 9. | After PIC investigates the condition of scene, he must check person, thing, time, place, object if the same with record of ship side. 後續海技PIC至現場調查事件發生情況，須針對人事時地物是否與船上紀錄相同。 | ☐ |
| 10. | Interview with all persons related to the accident. 與事件相關人員的當面洽談。 | ☐ |
| 11. | The finished report must be approved by the company before sending out to the parties concerned like flag state. 相關報告製作完成後，在發給相關單位如船旗國前，須經公司主管批核。 | ☐ |