Container on deck overboard/collapsed/cargo inside shifting Emergency checklist

貨櫃落海/貨櫃傾倒/櫃內貨物位移緊急事故處理程序檢查表

| Ship Side-Emergency Response船端-緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1. | Immediately inform the Master. Notify E/R for standing by engine. 立即通知船長與通知機艙備車。 | ☐ |
| 2. | Check if any other traffic or immediate danger in the vicinity. 確認附近是否有其他交通及立即危險。 | ☐ |
| 3. | Check chart depth & object & other information around ship position 檢查海圖上附近區域之水深、物件與其他必要資訊。 | ☐ |
| 4. | Record the time & ship position on chart and weather condition. 記錄當時時間/氣象及船位並標示在海圖上。 | ☐ |
| 5. | VHF broadcast “Security” massage of containers overboard to avoid endanger other vessels. 以VHF廣播貨櫃落海信文給附近船隻，以避免後續的損害糾紛。 | ☐ |
| As soon as the Master on bridge 船長上駕駛台後： |  |  |
| 1. | Assess prevailing situation immediately, alter Co. & reduce speed to ease the effect of sea & swell; ensure crew can walk/work on leeside with rolling no more than 10∘if possible. 立即評估當前情勢，轉向並減速以減緩風浪效應；確認船員可由下風側前往作業且船隻橫搖請勿超過10度，如果可能。 | ☐ |
| 2. | Notify Maritech Division immediately according to “Emergency Contact List” and send ER & photos for necessary assistance & support. 參考“緊急聯絡電話表＂立即將情況通知海技部及發ER及照片，以利公司提供適時的協助及支援 | ☐ |
| 3. | Execute risk assessment & ensure the following: 執行風險評估並確認下列事項： |  |
|  | - Does CK-0705-3 NAVIGATION IN HEAVY WEATHER CONDITION have been referenced to ensure ownship had complied one by one without fail. 是否CK-0705-3在惡劣天候航行中的各項注意事項，已經一一檢查並確認執行無誤。 | ☐ |
|  | - Before assign any crew to check containers conditions on deck: overboard/ collapsed/shifting, CK-0706-7 Checklist of work on deck in heavy weather must be effectively executed & confirmed, which include the following: 派工至甲板檢查貨櫃落海/傾側/移位前，CK-0706-7惡劣天候甲板工作檢查表必須確實執行並確認；包括下列各項： | ☐ |
|  | - Persons required to work on deck during heavy weather should wear a suitable life-jacket, proper PPE, and a portable transceiver? 惡劣天候下，至甲板工作者應該穿著救生衣，適宜的個人防護裝備及佩帶手提對講機？ | ☐ |
|  | - At least two seafarers work in pairs or more who should under command of an experienced senior officer? 船員最少應有兩人以上共同工作？所有船員皆由資深船副帶隊指揮？ | ☐ |
|  | - Had risk assessment considered following factors, including: | ☐ |
|  | 所做風險評估，是否考慮到下列因素： |  |
|  | - The necessity to work now. (i.e. can it wait until daylight, the next port, do the risks outweigh the benefits?) 現在去工作的必要性(可否等到天亮，下一港，風險是否大過利益?) |  |
|  | - Permit to work and company checklist completed. 工作許可及公司要求的檢查表是否已完成？ | ☐ |
|  | - Adjust vessel’s course and speed. 調整航向及航速。 | ☐ |
|  | - Rigging lifelines. 安裝救生索。 | ☐ |
|  | - Lifejackets with safety harnesses. 穿著繫有安全帶的救生衣。 | ☐ |
|  | - Proper PPE for the job. 適合工作的個人防護裝備。 | ☐ |
|  | - Using waterproof worksuits with reflective tape fitted 使用裝有反光帶的防水工作衣。 | ☐ |
|  | - Deck lights if required are sufficient in quantity and lumination. 所需的甲板工作燈數量與照明足夠。 | ☐ |
|  | - Visual contacted by supervisor. 可由帶隊官看見工作過程。 | ☐ |
|  | - Working in teams/pairs even workload is light to render assistance in case of sudden rolling or pounding. 成雙成組工作，以預防突然的橫搖與拱頭下，若人員受傷，可立即協助。 | ☐ |
|  | - Water-proof portable radios/walkie talkie. 防水的手提無線對講機。 | ☐ |
|  | - All equipment are approved type and well maintained. 確認所有裝備合格且保養良好。 | ☐ |
|  | - Availability of rescue & emergency medical care if things go wrong 萬一出狀況，是否有施救及緊急醫療的能力。 | ☐ |
|  | - Use of bridge searchlight to determine predominant wave direction at night. In restricted visibility or darkness radar may be used to determine the predominant wave direction 夜間使用駕駛台探照燈以決定主要浪頭的方向。在能見度受限制或黑暗中，雷達可用來決定主要浪頭的方向。 | ☐ |
|  | - Be aware that even in a regular wave pattern “rogue” waves can exist which can vary in direction and size from regular wave pattern being experienced 注意即便在規則的波浪型態，”詭浪”仍可能存在，其方向及大小與正規的波浪型態並不一致。 | ☐ |
| 4. | The master should confirm work on deck during heavy weather had been authorized and informed bridge duty officer already. 船長應該確認惡劣天候甲板工作以被授權，及已通知駕駛台當值船副。 | ☐ |
| 5. | Commander gives briefing to the working team the procedure & precautions. 指揮官對工作團隊做簡報，告知工作步驟及注意事項。 | ☐ |
| 6. | The master & duty officer must monitor & ensure the safety of all working crew until the job is over. 船長與當值船副必須監控及確認所有工作船員的安全，直到工作完成為止。 | ☐ |
| 7. | Downloaded VDR, CCTV & ECDIS, kept related record such as bell book/telegraph record/Log book/Charts /Course recorder/weather charts, etc. 已下載VDR、CCTV及ECDIS並保留任何有關的紀錄如車鐘簿、車鐘記錄、航行日誌、海圖、航向紀錄、天氣圖，等等。 | ☐ |
| 8. | Ship’s stress force and stability had assessed 已執行船體應力與穩度評估。 | ☐ |
| 9. | Any injury happened due to work on deck? Is it necessary to rescue, first aid or CPR? 確認甲板工作是否導致人員受傷？有無需要執行救助或急救(包括CPR)？ | ☐ |
| 10. | Sent ER, Contact PIC and coastal country for further assistance, prepare lifeboat station if necessary. 確認已發送ER報、通知PIC及沿海國等相關單位請求更多幫助，如果需要準備棄船部署。 | ☐ |
| 11. | Depend on weather condition, recover speed & resume original course to next port. 評估當下天候海況可行時，恢復原航向及速度，駛向下一港。 | ☐ |
| 12. | Report details of inspection result (with photos) to company & request for necessary assistance/survey arrangement if any. 向公司報告詳細檢查結果(附照片)及是否需要援助及檢驗安排。 | ☐ |
| 13. | Make following report: 製作下列報告： |  |
|  | - Statement of fact(in Chinese & English) with photos | ☐ |
|  | - Sea Protest | ☐ |
|  | - FM-0803-6 OTHER ACCIDENT-INCIDENT & NEAR MISS REPORT | ☐ |
|  | - FM-0803-7 SHIPBOARD ACCIDENT INVESTIGATION REPORT | ☐ |
|  | - Flag state required incident report(if necessary) | ☐ |
| Once oil spill overboard若油污漏洩入海 |  |  |
| 1. | According to SOPEP, from the information provided by damage control team, appraise the extent of the damage and immediately proceed the leaking-block, confirm other tanks watertight to avoid the damage spreading; any oil spill due to the breakage of the tank, stop the pollution according to SMM PR-08 Emergency Plan: Pollution Prevention; and prevent fire and explosion from oil spill; any spill overboard, report to parties concerned as per SOPEP’s procedure and format. 依據船舶油污染應急計畫(SOPEP)，以損害管制組回報之資訊，評估損害程度並指揮損害管制組進行堵漏、確保其他艙間水密性以防止損害擴大；有油料因破損溢出時，依照安全管理手冊PR-08 Emergency Plan: Pollution Prevention進行油污染防阻，並避免因溢油產生之爆炸與火災；若有任何油污漏洩入海，須照SOPEP程序通報有關單位。 | ☐ |
| 2. | Make risk assessment for oil spill overboard & confirm the oil pollution condition and take photos. 執行油污漏洩入海風險評估，確認油污污染狀況並拍攝現場照片。 | ☐ |
| 3. | Muster all crew for the oil pollution control. 召集船員執行防油污污染部署。 | ☐ |
| 4. | Dispatched an initial oil spills report to TPE / Maritech & local related Dept. 發送初步油汙汙染報告至海技部及與當地相關單位。 | ☐ |
| 5. | PIC of Legal & INS Dept. to be informed to P & I Club and Hull insurance to arrange oil pollution clear operation. 法保PIC通知P&I Club 履行保險契約準備清理油汙汙染作業。 | ☐ |
| 6. | Inform Agent to arrange for any emergency assistance 通知代理，準備任何緊急情況之協助。 | ☐ |
| After the emergency action, Master shall pay some special attention to collect the evidences and make the reports as the following: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 1. | As per SMM PR-08, any accident, the required report forms are, FM-0803-6, FM-0803-7, and statement of fact & sea protest. 依據公司安全管理手冊所規定之意外事故所規定表格繕製意外事故報告(FM-0803-6)、意外事故調查報告(FM-0803-7)以及Statement of Fact、Sea Protest呈報公司； | ☐ |
| 2. | Take good care of VDR’s, CCTV recording( as VDR will continue recording for 12 hours, after Master contacting with the authority concerned, stop the recording and take out CF Card for later investigation) 妥善保存VDR、CCTV之記錄(因VDR會連續紀錄12HRS，在船長與主管機關聯繫後，再將記錄器停止，並取出CF CARD以供後續調查使用)。 | ☐ |
| 3. | Video or photos for the accident. 事故經過的拍攝影片及照相； | ☐ |
| 4. | The description of container overboard/collapsed incident shall include all witnesses and duty persons, statement of fact shall attach photos as many as possible, the information shall include: 貨櫃落海及倒塌事故過程之描述，包括所有目擊及當事人之報告，STATEMENT OF FACT儘可能附上事故現場照片，收集呈報公司之資料証據包括： | ☐ |
|  | - Overboard/collapsed containers position/container No. /POL/POD 落海及倒塌貨櫃的位置/櫃號/裝貨港/卸貨港 | ☐ |
|  | - Ship’s position/course/speed/rpm and course recorder reading at time 當時之船位/航向/航速/轉速及當時航向記錄器所顯示之記錄； | ☐ |
|  | - Engine/steering maneuvering/time and condition 當時的用車/用舵/時間及狀況； | ☐ |
|  | - The prevailing weather/wind direction, wind speed/direction of wave and height/swell direction and height/visibility/received weather forecast; 當時天氣狀況/風向、風速/浪之方向與高度/湧之方向與高度/能見度/收受之天氣預報； | ☐ |
|  | - Positioning fix system; 使用之定位系統； | ☐ |
|  | - Ships recording of response action at time (including operation of M/E); 發現貨櫃落海/倒塌時，反應動作之記錄(含主機操作)； | ☐ |
|  | - The damage condition of ship and cargo, with attached photos, if possible; 船貨損壞情況，儘可能附照片； | ☐ |
|  | - Master’s sea protest shall be endorsed 船長之海事報告(Sea Protest)，須加以簽證 | ☐ |
|  | - If arranged surveyor for damage inspection, shall prepare ship’s drawing; all foreign parties should have company’s notification to be allowed onboad. 如有安排驗船師SURVEYOR登輪作損壞檢定，應備妥相關船圖。所有登輪人員，需有公司同意，才可上船。 | ☐ |
|  | - CLASS SURVEYOR: to assess ship’s seaworthiness: it arranged by the Company. 用於判定船體適航性、由公司決定是否僱請。 | ☐ |
|  | - INSURANCE SURVEYOR: to evaluate the damage, it’s separated to hull surveyor and P & I surveyor; no assessment for liability. 作損傷鑑定、可分船體 SURVEYOR 及 P & I SURVEYOR、不作責任判斷用； | ☐ |
|  | - Lawyer: shall identify for whom he represents. 律師 (LAWYOR, SOLICITOR, ATTORNEY, ADVOCATE均是)：須認清代表何方； | ☐ |
|  | - Salvage Superintendent or Salvage Surveyor等； 救助負責人或檢查員； | ☐ |
|  | - Coast Guard or Port authority investigator. 海岸巡防隊或港務局/海事處調查人員。照公司指示提供文件及證據。 | ☐ |
|  | - Report to flag of country in 24 HR. 事發24小時內向船旗國回報。 | ☐ |
|  | - All external report must check by company. 所有對外報告內容都須經過公司審議 | ☐ |

| 岸端緊急處置Shore Side-Emergency Response |  |  |
| --- | --- | --- |
| 檢查項目內容Checking Items |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  | ☐ |
| 1. | PIC must confirm the following as soon as receiving report from Master: PIC第一時間確認船端回報下列事項： | ☐ |
|  | - containers overboard/collapsed position/condition? 貨櫃落海/倒塌位置及狀況？ | ☐ |
|  | - container lashing condition? 貨櫃繫固狀況？ | ☐ |
|  | - Have damage been controlled? 是否受損已被控制？ | ☐ |
|  | - The draft of fore and astern? 艏艉現在吃水？ | ☐ |
|  | - GM? Rolling Period? Stability? 穩定性？ | ☐ |
|  | - Have all machine, equipments, cargo holds and tanks condition been checked & confirmed? 所有機器、設備、貨艙及tank情況是否已檢查並確認？ | ☐ |
|  | - Damaged ship course /speed/location/vessels nearby are safe to do so? 受損船隻的航向/航速/船位/附近船隻，是否安全可行？ | ☐ |
| 2. | Any personal injury? Any oil pollution? Container leaking? Fire? If need shore side urgent assistance? 是否有人員受傷？ 是否有油污染？是否有貨櫃漏洩？是否發生火災？是否需岸端緊急援助？ | ☐ |
| 3. | Have photos at scene been taken & ready to send back? VDR, CCTV & ECDIS been downloaded? 現場照片是否已拍攝並準備寄回？VDR、CCTV及ECDIS是否已下載? | ☐ |
| 5. | Have the risk assessment for overall emergency response & ship’s stability been effectively executed? 對所有緊急處置及船舶穩定性的風險評估是否已有效執行？ | ☐ |
| 6. | Have notified the nearest coasting country & flag state? 是否已通知沿岸最近管轄國及船旗國？ | ☐ |
| 7. | Have sent the following initial accident reports according to SMM PR-0802 in safety situation: 是否已照安全管理手冊PR-0802程序，在安全無虞下，寄回下列初步事件報告： | ☐ |
|  | - Statement of fact(in Chinese & English) with photos | ☐ |
|  | - Sea Protest | ☐ |
|  | - FM-0803-6, FM-0803-7 | ☐ |
|  | - Report to flag state | ☐ |
|  | - Tank & cargo hold sounding record |  |
|  | - Copy of log book, bell book, telegraph, course recorder, weather chart… etc.. | ☐ |
| 8. | PIC continues closely contacting with ship’s captain & provides necessary emergent assistance until the fleet vessel is safe & finish repair & investigate case over. 海技PIC持續與船端緊密連絡並協助緊急處置，直到屬輪安全後。以及後續的維修與調查事件結束為止。 | ☐ |
| 9. | PIC investigates at scene afterward, he must check person, thing, time, place, object if the same with record of ship side. 海技PIC事後調查事故現場，須核對人事時地物與船上紀錄是否相同。 | ☐ |
| 10. | Interview with all persons related to the accident afterward. 事後與事件相關人員的當面洽談。 | ☐ |
| 11. | The finished report must be approved by the company before sending out to the parties concerned like flag state. 相關報告製作完成後，在發給相關單位如船旗國前，須經公司主管批核。 | ☐ |