Deck Container on Fire Emergency checklist

甲板貨櫃失火緊急處置表

| Ship Side-Emergency Response船端-緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1. | Sound general alarm & Muster fire-fighting station and call Master immediately. 立即鳴放火警警報並廣播全船就滅火佈署，通知船長。 | ☐ |
| 2. | Check where the fire start and fire/smoke spreading direction on bridge. If necessary, turn to windward side first to avoid endangering the accommodation. 檢查起火地點與觀察火勢/煙霧是否向駕駛台擴散，必要時先轉向下風，避免危害住艙。 | ☐ |
| 3. | Check if any other traffic or immediate danger in the vicinity. 確認附近是否有其他船隻與立即危險。 | ☐ |
| 4. | Check chart depth & wind/current direction & other information around the ship. 檢查海圖上附近區域之水深、物件與其他必要資訊。 | ☐ |
| 5. | Notify E/R to stand by engine and start emergency firefighting procedure. 通知機艙備車與啟動緊急滅火程序。 | ☐ |
| 6. | Broadcast urgent or safety emergency or safety massage to all vessels by VHF 16 or AIS. 廣播發送VHF，或AIS緊急或安全信文給海面其他船隻。 | ☐ |
| 7. | Record the time & ship position on chart. 記錄當時時間及船位，並標示在海圖上。 | ☐ |
| As soon as the Master on bridge 船長上駕駛台後： |  |  |
| 1. | Check where leeside and sea room is available for stopping and fire-fighting? 檢查是否有下風的水域，適合停車與滅火部署? | ☐ |
| 2. | Alter Co. & reduce speed to put container on fire at leeside to avoid crew accommodation, engine room and life-saving appliances been affected. Close all watertight doors immediately. 轉向並減速，將燃燒櫃放在下風側，關閉所有對外水密門，以免船員住艙、機艙和救生設備受到影響。 | ☐ |
| 3. | Check if the container on fire and nearby area has DG cargo to confirm its characteristics & precautions according to IMDG code & MSDS. 檢查燃燒櫃與附近，是否有其他危險貨，依IMDG code及物質安全資料表，確定該貨櫃性質與附近的貨物特性，及注意防範事項。 | ☐ |
| 4. | Assign fire-fighting team with fireman outfit & equipments (CABA) to fire scene from upwind side to cool and evaluate situation. 廣播並指派滅火隊，穿著防火衣與攜帶必要滅火裝備(呼吸器)，從上風接近現場，評估情勢與冷卻火場。 | ☐ |
| 5. | Decide shipboard personnel and equipments and fire pump capacity is enough to put out the fire or just to contend it spreading. 決定船上人員和設備以及消防泵的容量，足以撲滅火災，還是僅僅防範火勢蔓延。 | ☐ |
| 6. | Notify Maritech Division immediately according to “Emergency Contact List” and send ER & photos for necessary assistance & support. 參考“緊急聯絡電話表＂立即將情況，電話報告海技部及發ER及照片，以利提供適時的協助及支援。 | ☐ |

| 7. | (IF NECESSARY) At early stage of the emergency, Master shall notify costal state or flag state, the notification shall include the type of the accident, personal injury and assistance required. (需要的情況下)緊急事件初期，船長應通知最近的沿岸國及船旗國當局，通報內容包括事件類型，人員傷亡及需要何種協助。 | ☐ |
| --- | --- | --- |
| 8. | Arrange Fire-fighting team and supporting team in turn of 6 hours work and rest shift if fire is contended which need many days hard works. If Company vessel passing by may provide extra man power to assist by company’s order. 如遇火勢需多日困難撲滅，安排消防隊和支援隊輪班，輪流間隔6小時作息。若公司的經過船隻，經由公司安排，可以提供額外的人力，來協助滅火。 | ☐ |
| 9. | If fire becomes serious & out of control Captain, after careful assessment & discuss with C/E & C/O, need to abandon ship. He should inform company PIC of this situation and send the “distress signal” to outside parties first, announce/broadcasting “abandon ship”, summon all the crew for roll call, preserve all legal documents and evidence like VDR, CCTV, Logbook, certificates, bell book and fire-fighting notebook, etc…. 如果火勢嚴重失控，船長與輪機長與大副仔細評估和討論後，決定棄船。應先通報公司，並向外界發出“求救信號”，宣布/廣播“棄船”，召集全體船員進行點名，保存所有法律文件和證據，如VDR、CCTV、硬碟航海日誌、證書、俥鐘簿和消防筆記本等。 | ☐ |
| Before fire-fighting, execute risk assessment & ensure the following by C/O & Captain: 大副滅火前，執行風險評估並請船長共同確認下列事項： |  |  |
|  | - Make sure all crew had been roll called already? Had any crew not been informed yet? 確保所有船員都已經點名到齊?是否有任何船員，還沒有得到通知? | ☐ |
|  | - Does fire-fighting team member already deploy according to the fire-fighting muster list with fireman outfit & appropriate fire-fighting equipments? 是否滅火隊已照滅火部署表，穿著適當防火衣及攜帶所有相關滅火裝備。 | ☐ |
|  | - Briefing to the fire-fighting team the procedure & precautions of fired containers. 已對滅火隊員做簡報，告知滅火步驟及該貨櫃滅火的注意事項。 | ☐ |
|  | - Inform master to alter course & reduce speed to minimize the fire and smoke spreading. 通知駕駛台轉向及減速到適當位置，以便控制火勢。 | ☐ |
|  | - Fire-fighting team should proceed to fire scene from upwind side and keep safe distance from fired container at safe place to protect themselves. 滅火隊前往起火地點與在滅火過程中，要保持在起火櫃的上風，立於保護自身安全的位置。 | ☐ |
|  | - Cool down the containers on fired & adjacent containers with fixed Firefighting Monitors spray and mist if equipped. 以消防水柱(消防水炮)與水霧，減低燃燒櫃及其周遭櫃的熱量以加強隔離效果，防止火勢蔓延。 | ☐ |
|  | - Check the use of container water mist lancer (if equipped) safety by cargo property and chemical compositions with Dangerous Cargo List or MSDS. 依照危險品艙單檢查貨物性質和可能的化學反應，檢視貨櫃內部滅火槍(如有配置)的使用安全。 | ☐ |
|  | - Try to Isolate fire area with maximum fire water spray or water monitor if equipped. This intention may have to carry out in leeside of the fire, if crew safety can be assured. 儘量將燃燒區域隔絕出來，利用最大量的消防水柱(消防水砲)長期冷卻。隔絕行動可能需要從火勢的下風處進行，如果船員安全可以確保。 | ☐ |
|  | - Calculate and access Ship’s stress force and stability safety. 執行船體應力與穩定度的安全性評估。 | ☐ |
|  | - Check if cargo hold flooded with massive fire-fighting water which need to pump out immediately. 檢查貨艙是否被大量消防水淹沒，需要立即抽出。 | ☐ |
|  | - Check if cargo hold flooded oily water which could spill overboard and activate Oily Water Separator or SOPEP at once. 確認大艙是否灌入油水或貨櫃有漏洩的油污染，如有，立即啟動油水分離器或SOPEP程序。 | ☐ |
|  | - Check any crew injured need to repatriate or first aid (CPR) help on scene? 確認本輪人員是否有受傷?有無需要執行後送，或現場急救(包括CPR)。 | ☐ |
|  | - Calculate water ingress and stability deteriorate situation before further actions. 在進一步行動之前，評估進水和穩定性惡化的情況。 | ☐ |
|  | - Adjust Trim, draught of the ship to gather bilge water to pump out if necessary 必要情況下，調整俯仰差、減低船舶吃水，以打出汙水。 | ☐ |
|  | - At Captain’s discretion to preserve human life and ship in emergency, oily water may pump out if necessary. MARPOL reg. 4. 船長為保全人命和船舶，在緊急情況下得酌情決定，必要時，可排出含油污水。 | ☐ |
|  | - Remark progress of the fire-fighting in hourly Record & report to the company. 每小時記錄滅火進度並報告公司。 | ☐ |
|  | - Continuously contact with the company & agent of next port for necessary assistance & arrangement (i.e. shore fire-fighting brigade or shelter anchorage). 持續與公司及下一港的代理，保持連絡，以獲得必要的援助及安排(如岸上救火隊或避難錨地)。 | ☐ |

| Once oil spill overboard若油污漏洩入海 |  |  |
| --- | --- | --- |
| 1. | Any oil spill due to the breakage of the tank, stop the pollution according to SMM PR-08 Emergency Plan: Pollution Prevention; and prevent fire and explosion from oil spill; any spill overboard, report to parties concerned as per SOPEP’s procedure and format. 有油料因貨艙破損溢出時，依照安全管理手冊PR-08 Emergency Plan: Pollution Prevention進行油污染防阻，並避免因溢油產生之爆炸與火災；若有任何油污漏洩入海，須照船上油污染程序，通報有關單位。 | ☐ |
| 2. | Appraise the extent of the damage and block the leaking immediately, confirm other tanks watertight to avoid the damage spreading; 依據船舶油污染應急計畫(SOPEP)，評估損害程度，並指揮損害管制組進行堵漏、確保其他艙間水密性，以防止損害擴大。 | ☐ |
| 3. | Make risk assessment for oil spill overboard & confirm the oil pollution condition and take photos. Muster crew for the oil pollution control. 執行油污漏洩入海風險評估，確認油污污染狀況，並拍攝現場照片。召集船員進行防油污污染部署。 | ☐ |
| 4. | Dispatch an initial oil spills report to TPE / Maritech & local related Dept. 發送初步油汙汙染報告至海技部，及與當地相關單位。 | ☐ |
| 5. | Company PIC Inform P & I Club and Hull insurance company to arrange oil pollution clear operation. 法保PIC通知相關保險公司，準備清理油汙汙染作業。 | ☐ |
| 6. | Inform Agent to arrange for any emergency assistance. 通知代理，準備任何緊急情況之協助。 | ☐ |
| After the emergency action, Master shall collect the evidences and prepare following reports: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 1. | As per SMM PR-08, any accident, the required report forms are, FM-0803-1, FM-0803-7, and statement of fact & sea protest. 依據公司安全管理手冊所規定之意外事故，規定表格繕製意外事故報告(FM-0803-1)、意外事故調查報告(FM-0803-7)以及事實陳述、海事報告呈報公司； | ☐ |
| 2. | Preserve VDR’s, CCTV recording (as VDR will continue recording for 12 hours, Master should decide when to stop the recording for later investigation and replace/rewrite to a new CF Card to continue recording) 妥善保存VDR、CCTV之記錄(因VDR只能連續12紀錄小時，船長決定何時將記錄器停止，並更換新的記憶卡或轉錄他卡，以供後續調查使用)。 | ☐ |
| 3. | Statement of fact shall attach photos as many as possible, description of fire incident shall include all witnesses and duty persons oral statement to identify real cause of incident and prevent re-occurrence afterward. 事實陳述為對公司內部之火災事故過程之描述(不對外公開)，應盡可能附上照片，火災事件的描述應包括所有目擊者和值班人員的口頭陳述，以查明事故的真正原因，防止日後事件再次發生。 | ☐ |
|  | - Fire containers position/container No./contents. 起火貨櫃的位置/櫃號/貨物內容。 | ☐ |
|  | - Ship’s position/course/speed/rpm and course recorder reading at time. 當時之船位/航向/航速/轉速及當時航向記錄器所顯示之記錄； | ☐ |
|  | - Engine/steering maneuvering/time and condition. 當時的用車/用舵/時間及狀況； | ☐ |
|  | - Weather/ wind direction/ speed and wave/ swell height/direction and height/visibility/received weather forecast; 當時天氣狀況/風向、風速/浪之方向與高度/湧之方向與高度/能見度/收受之天氣預報。 | ☐ |
|  | - Ships recording of response to fire (including operation of M/E); 發現起火時，船方反應動作之記錄(含主機操作)； | ☐ |
|  | - The damage condition of ship and cargo on both ships, with attached photos, if possible; 有船貨損壞情況，儘可能附照片。 | ☐ |
| 3. | Master’s sea protest should include the incident time, nature, place and any personal injury or disease only which shall be official endorsed by next port of call. Master should not include any speculation and conclusion of said incident in Sea Protest. 船長對外之海事報告(Sea Protest)，應僅包括事件時間、性質、地點和任何人身傷害或死亡，不應對事件原因與後果，妄加推測，並應由官署予以簽證。 | ☐ |
| 4. | Salvage Superintendent or Salvage Surveyor should assign by company or P&I Club. Beware of VHF communication with outside parties, don’t make any commitment to any proposal without company instructions. 救助負責人或檢查員，應由船東或船東互保協會指派，船長除非警急處置，不得自行同意，避免被VHF錄音詐騙，不可擅自隨口應承。 | ☐ |
| 5. | If company arranged surveyor for damage inspection, Master shall prepare related drawing; 如有公司安排的驗船師登輪作損壞檢定，船長應備妥全船相關船圖供參考。 | ☐ |
|  | - CLASS SURVEYOR: to assess ship’s seaworthiness, arranged by the Company. 船級協會驗船師：以判定船體適航性、由公司通知安排。 | ☐ |
|  | - INSURANCE SURVEYOR: to evaluate the damage, hull or P & I surveyor; not liability assessment. 船體驗船師作損傷鑑定、可分船體險及船東協會驗船師、不作任何責任判斷用。 | ☐ |
|  | - Lawyer: Master should identify his ID and which parties he represented? Master should not allow them come on board without prior consent from company. 律師：船長應該查明他的身份，他代表哪些當事人?未經公司事先同意，船長不應允許他們上船。 | ☐ |
|  | - Report to flag of country in 24 HR. 事發24小時內向船旗國回報。 | ☐ |
|  | - All external report must check by company. 所有對外報告內容都須經過公司審議。 | ☐ |
|  | - All relevant document and records should not hand to 3rd party without company’s prior consent. 未經公司事先同意，不得將所有相關文件和記錄交給第三者。 | ☐ |

| 岸端緊急處置Shore Side-Emergency Response |  |  |
| --- | --- | --- |
| 檢查項目Checking Items |  |  |
| After receiving telephone from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端電話報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  | ☐ |
| 1. | PIC must confirm the following when receiving oral report from Master, if possible: PIC接獲船端口頭回報，如果可能，應於第一時間確認下列事項： | ☐ |
|  | - The fire breaks out position/time and type? Ownship position and safety afterward? 船上火災發生位置/時間與種類?本船位置與後續的安全性? | ☐ |
|  | - Have fire situation been controlled? 是否火勢已被控制? | ☐ |
|  | - Draft fore / aft and stability affected? 艏艉現在前後吃水與穩定度的影響? | ☐ |
|  | - Have all machine, equipments, cargo holds and tanks condition been checked & confirmed? 所有機器、設備、貨艙及油水艙情況，是否已經檢查並確認? | ☐ |
|  | - Any personal injury? oil pollution? Container leaking? Explosion? Need Shore side provide any assistance or arrangement? 任何人員傷害?油污染?貨櫃洩漏?爆炸?需要公司提供任何協助或安排嗎? | ☐ |
| 2. | Have photos at scene been taken & ready to send back? VDR, CCTV & ECDIS been downloaded? 現場照片是否已拍攝並準備寄回?航行記錄器、CCTV及電子海圖資料是否已經下載? | ☐ |
| 3. | Have the risk assessment for overall emergency response & ship’s stability been executed? 對所有緊急處置及船舶穩定性的風險評估，是否已有效執行? | ☐ |
| 4. | Have Master notified the nearest coasting country & flag state? 是否已通知沿岸最近管轄國及船旗國? | ☐ |
| 5. | Have sent the following initial accident reports according to SMM PR-0802 in safety situation: 在船隻安全無虞下，是否已照安全管理手冊PR-0802程序，寄回下列初步事件報告： | ☐ |
|  | - Statement of fact (in Chinese & English) with photos. | ☐ |
|  | - Sea Protest. | ☐ |
|  | - FM-0803-1, FM-0803-7. | ☐ |
|  | - Report to flag state. | ☐ |
|  | - Tank & cargo hold sounding record. | ☐ |
|  | - Copy of log book, bell book, telegraph, course recorder, weather chart… etc.. | ☐ |
| 8. | PIC continues contact ship’s captain and control the related resource for the support until vessel is safe & arrange repair & final investigation. 海技船管人員持續與船端緊密連絡，及掌握所需資源給予協助，直到屬輪安全後，以及後續的維修與事件調查結束。 | ☐ |
| 9. | PIC investigation in fire scene should review personnel, time, place, cause and damage are same as Master’s report. 船管人員在火災後現場調查時，應檢查人員、時間、地點、原因和損壞情況與船方報告是否相同。 | ☐ |
| 10. | Interview with all persons related to this accident and find out root cause of fire. 與事件相關人員的當面洽談，找出事件的根本原因。 | ☐ |
| 11. | Find out container fire’s cargo property and report to top management. 岸端查明貨櫃火災的貨物性質，並向公司管理層報告。 | ☐ |
| 12. | The finished report must be approved by the company before sending out to the parties concerned like flag state. 相關報告製作完成後，在發給相關單位如船旗國前，須經公司主管批核。 | ☐ |