Hold Containers on fire Emergency checklist

貨艙貨櫃失火緊急事故處理檢查表

| Ship Side-Emergency Response船端-緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1. | Ensure smoke & fire detecting alarm from which hold & observe the situation of smoke from the hold, then immediately blow the fire alarm & broadcast for fire-fighting station and inform the Master. 確認貨艙失火偵測警報自哪一貨艙響起並觀察該艙冒煙情況，立即拉火警警報並廣播就滅火佈署及通知船長。 | ☐ |
| 2. | Check if any other traffic or immediate danger in the vicinity. 確認附近是否有其他交通或立即危險。 | ☐ |
| 3. | Check chart depth & object & other information around the ship. 檢查海圖上附近區域之水深、物件與其他必要資訊。 | ☐ |
| 4. | Record the time & ship position on chart 記錄當時時間及船位並標示在海圖上。 | ☐ |
| 5. | Notify E/R for standing by engine. 通知機艙備車。 | ☐ |
| As soon as the Master on bridge 船長上駕駛台後： |  |  |
| 1. | Carefully & cautiously assess the fire situation depending on the extent of the smoke, fire & if any serious explosion from the hold to decide the next step. 視大艙冒煙，起火及是否有嚴重爆炸的程度，審慎評估下一步採取的措施。 | ☐ |
| 2. | Check with C/O if any DG flammable cargo in the hold & confirm its characteristics & precautions according to IMDG code & MSDS. 與大副檢查大艙是否裝有易燃危險貨，依IMDG code及MSDS確定該貨的特性及注意防範事項。 | ☐ |
| 3. | Close the hold ventilation & switch off the electricity. 關閉大艙通風及電源。 | ☐ |
| 4. | Broadcast & assign CO to summon fire-fighting team with fireman outfit & equipments ready, if smoke is not serious and no explosion, then fire team enter the cargo hold with full outfit to detect the fire & assess & report the situation. 廣播並指派大副立即按部署表組織救火隊，穿著防火衣與攜帶滅火裝備，若冒煙不嚴重且無爆炸，則滅火隊著全套防火衣入艙探火，評估並報告火場情況。 | ☐ |
| 5. | If smoke on all levels in the hold, unable to trace the source of the fire, for personnel’s safety, retreat from the scene, observe for 10 minutes , the smoke emitted from hold becomes denser, then after discussion with C/E & C/O, decide at once to activate the designated CO2 into the hold to put out the fire. 若煙霧彌漫整個大艙，探火組找不到火源，為人員安全，迅速撤出火場，觀察10分鐘，若持續冒農煙且越來越濃，與輪機長及大副商議後，立即決定釋放指定數量的CO2到大艙滅火。 | ☐ |
| 6. | If dense smoke billowing & explosion from the hold, don’t bother to send the crew to detect the fire, release the designated CO2 into the hold as soon as possible to put out the fire. 若持續冒滾滾濃煙且有爆炸聲自大艙船出，則無謂麻煩船員去探火，應立即儘快釋放指定數量的CO2到大艙滅火。 | ☐ |
| 7. | Muster all crew in deck office & ensure them safe. 召集所有船員在甲板辦公室並確認全部安全。 | ☐ |
| 8. | Notify Maritech Division immediately according to “Emergency Contact List” and send ER & photos for necessary assistance & support. 參照“緊急聯絡電話表＂立即將情況通知海技部及發ER及照片，以利提供適時的協助及支援。 | ☐ |
| 9. | (IF NECESSARY) At early stage of the emergency, Master shall notify costal state or flag state, the notification shall include the type of the accident and what kind of assistance required. (需要的情況下)緊急事件初期，船長應通知最近的沿岸國及船旗國當局，通報內容包括事件類型及需要何種協助。 | ☐ |
| 10. | Crew is posted near the cargo hold & continuously monitor the condition. 船員輪流佈署在接近大艙的位置，持續監控狀況。 | ☐ |
| 11. | The cargo hold is kept totally hermetically closed with CO2 inside until next port for inspection. 貨艙持續保持完全密封蓋住，CO2在內直到下一港做檢查。 | ☐ |
| 12. | Execute risk assessment & ensure the following: 執行風險評估並確認下列事項： |  |
|  | - If fire-fighting team already deployed according to the fire-fighting muster list with appropriate fireman outfit & all fire-fighting equipments ready under C/O direction to detect the fire. 是否滅火隊已照滅火部署表穿著適當防火衣及攜所有相關滅火裝備在大副指揮下，準備探火。 | ☐ |
|  | - Briefing to the fire-fighting team the procedure & precautions for fire-fighting. 已對滅火隊員做簡報，告知滅火步驟及注意事項。 | ☐ |
|  | - Commander & fire-fighting team keep distance from the fired container at safe place able to protect themselves during detecting the fire. 指揮官及滅火隊在探火過程與起火櫃保持距離，立於能保護自身安全的位置。 | ☐ |
|  | - Send radio warnings to all ships in the vicinity to keep clear. 無線電通知附近所有船舶保持距離。 | ☐ |
|  | - Downloaded VDR, CCTV & ECDIS, kept related record such as bell book/Log book/Chart /Course recorder/Sounding record, etc. 已下載VDR、CCTV及ECDIS並保留任何有關的紀錄如航行日誌、海圖、航向紀錄、量水紀錄等。 | ☐ |
|  | - Ship’s stress force and stability assessment. 已執行船體應力與穩度評估。 | ☐ |
|  | - Check if container leaking cause oil spill overboard, if does, activate SOPEP at once. 確認是否有貨櫃漏洩造成油污染，如有，立即啟動SOPEP程序。 | ☐ |
|  | - Any injury happened due to fire-fighting. Is it necessary to execute rescue, first aid including CPR? 確認本輪是否因滅火導致人員受傷。有無需要執行救助或急救(包括CPR)。 | ☐ |
|  | - Sent ER, Contact with PIC and coasting country for further assistance, prepare the abandon ship procedure (in case if necessary) 確認已發送ER報、通知PIC及沿海國等相關單位請求更多幫助，如果有需要情況下則準備棄船部署。 | ☐ |
|  | - Stability should be assessed when taking any action. 在採取任何動作之前是否都已做船舶的穩定度評估。 | ☐ |
|  | - Chang Trim, Reduce the draught of the ship & make even keel & pump out water if necessary. 有必要情況下，調整Trim、減低船舶吃水或Even keel，打水以減低損傷。 | ☐ |
|  | - Find the cause of container fire. 查明貨櫃起火的原因。 | ☐ |
|  | - Record the progress of the fire-fighting hourly & report to the company. 每小時記錄滅火進度並報告公司。 | ☐ |
|  | - Continuously contact with the company & agent of next port for necessary assistance & arrangement (i.e. shore fire-fighting brigade or shelter anchorage). 持續與公司及下一港的代理保持連絡以獲得必要的援助及安排(如岸上救火隊或避難錨地)。 | ☐ |
|  | - If the situation of fire becomes serious & out of control, affecting seamen’s life onboard and necessary to “abandon ship”, after cautious assessment & discussing with C/E & C/O, send the “distress signal” first, announce/broadcasting “abandon ship” summon all the crew for abandon ship station in order according to “abandon ship muster station list” 若火勢變得嚴重且失控，威脅到船員生命，需要”棄船”；經過審慎評估與輪機長，大副商議後，先發送遇難信號，立即廣播全船就”棄船部署”，召集所有船員照棄船部署表就位及按秩序執行。 | ☐ |
| Once oil spill overboard若油污漏洩入海 |  |  |
| 1. | According to SOPEP, from the information provided by damage control team, appraise the extent of the damage and immediately proceed the leaking-block, confirm other tanks watertight to avoid the damage spreading; any oil spill due to the breakage of the tank, stop the pollution according to SMM PR-08 Emergency Plan: Pollution Prevention; and prevent fire and explosion from oil spill; any spill overboard, report to parties concerned as per SOPEP’s procedure and format. 依據船舶油污染應急計畫(SOPEP)，以損害管制組回報之資訊，評估損害程度並指揮損害管制組進行堵漏、確保其他艙間水密性以防止損害擴大；有油料因破損溢出時，依照安全管理手冊PR-08 Emergency Plan: Pollution Prevention進行油污染防阻，並避免因溢油產生之爆炸與火災；若有任何油污漏洩入海，須照SOPEP程序通報有關單位。 | ☐ |
| 2. | Make risk assessment for oil spill overboard & confirm the oil pollution condition and take photos. 執行油污漏洩入海風險評估，確認油污污染狀況並拍攝現場照片。 | ☐ |
| 3. | Muster all crew for the oil pollution control. 召集船員執行防油污污染部署。 | ☐ |
| 4. | Dispatched an initial oil spills report to TPE / Maritech & local related Dept. 發送初步油汙汙染報告至海技部及與當地相關單位。 | ☐ |
| 5. | PIC of Legal & INS Dept. to be informed to P & I Club and Hull insurance to arrange oil pollution clear operation. 法保PIC通知P&I Club 履行保險契約準備清理油汙汙染作業。 | ☐ |
| 6. | Inform Agent to arrange for any emergency assistance 通知代理，準備任何緊急情況之協助。 | ☐ |
| After the emergency action, Master shall pay some special attention to collect the evidences and make the reports as the following: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 1. | As per SMM PR-08, any accident, the required report forms are, FM-0803-1, FM-0803-7, and statement of fact & sea protest. 依據公司安全管理手冊所規定之意外事故所規定表格繕製意外事故報告FM-0803-1、意外事故調查報告FM-0803-7以及Statement of Fact、Sea Protest呈報公司； | ☐ |
| 2. | Take good care of VDR & CCTV’s recording( as VDR will continue recording for 12 hours, after Master contacting with the authority concerned, stop the recording and take out CF Card for later investigation) 妥善保存VDR、CCTV之記錄(因VDR會連續紀錄12HRS，在船長與主管機關聯繫後，再將記錄器停止，並取出CF CARD以供後續調查使用)。 | ☐ |
| 3. | Video or photos for the accident. 事故經過的拍攝影片及照相； | ☐ |
| 4. | The description of fire incident shall include all witnesses and duty persons, statement of fact shall attach photos as many as possible, the information shall include: 火災事故過程之描述，包括所有目擊及當事人之報告，STATEMENT OF FACT儘可能附上事故現場照片，收集呈報公司之資料証據包括： | ☐ |
|  | - Cargo hold fire location/hold No./stowed DG cargo. 貨艙起火的位置/第幾大艙/所裝危險貨 | ☐ |
|  | - Ship’s position/course/speed/rpm and course recorder reading at time. 當時之船位/航向/航速/轉速及當時航向記錄器所顯示之記錄； | ☐ |
|  | - Engine/steering maneuvering/time and condition. 當時的用車/用舵/時間及狀況； | ☐ |
|  | - The prevailing weather/wind direction, wind speed/direction of wave and height/swell direction and height/visibility/received weather forecast; 當時天氣狀況/風向、風速/浪之方向與高度/湧之方向與高度/能見度/收受之天氣預報； | ☐ |
|  | - Positioning fix system; 使用之定位系統； | ☐ |
|  | - Ships recording of response action at time (including operation of M/E); 發現起火時，反應動作之記錄(含主機操作)； |  |
|  | - The damage condition of ship and cargo on both ships, with attached photos, if possible; 有船貨損壞情況，儘可能附照片； | ☐ |
|  | - Master’s sea protest shall be endorsed; 船長之海事報告(Sea Protest)，須加以簽證； | ☐ |
|  | - If arranged surveyor for damage inspection, shall prepare GA and shell expansion drawing; 如有安排驗船師SURVEYOR登輪作損壞檢定，應備妥GA及SHELL EXPANSION圖。 | ☐ |
|  | - CLASS SURVEYOR : to assess ship’s seaworthiness: it’s better to be arranged by the Company; 用於判定船體適航性、最好由公司決定是否僱請； | ☐ |
|  | - INSURANCE SURVEYOR: to evaluate the damage, it’s separated to hull surveyor and P & I surveyor; no assessment for liability. 作損傷鑑定、可分船體 SURVEYOR 及 P & I SURVEYOR、不作責任判斷用； | ☐ |
|  | - Lawyer: shall identify for whom he represents. 律師 (LAWYOR, SOLICITOR, ATTORNEY, ADVOCATE均是)：須認清代表何方； | ☐ |
|  | - Coast Guard or Port authority investigator. 海岸巡防隊或港務局/海事處調查人員。照公司指示提供文件及證據。 | ☐ |
|  | - Report to flag of country in 24 HR. 事發24小時內向船旗國回報。 | ☐ |
|  | - All external report must check by company. 所有對外報告內容都須經過公司審議 | ☐ |

| Shore Side-Emergency Response 岸端緊急處置 |  |  |
| --- | --- | --- |
| Checking Items 檢查項目內容 |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  | ☐ |
| 1. | PIC must confirm the following as soon as receiving report from Captain: PIC第一時間確認船端回報下列事項： | ☐ |
|  | - The fire position? 火災發生位置？ | ☐ |
|  | - Have damage been controlled? 是否受損已被控制？ | ☐ |
|  | - The draft of fore and astern? 艏艉現在吃水？ | ☐ |
|  | - Have all machine, equipments, cargo holds and tanks condition been checked & confirmed? 所有機器、設備、貨艙及tank情況是否已檢查並確認？ | ☐ |
| 2. | Any personal injury? Any oil pollution? Container leaking? Fire? If need shore side urgent assistance? 是否有人員受傷？ 是否有油污染？是否有貨櫃漏洩？是否發生火災？是否需岸端緊急援助？ | ☐ |
| 3. | Have photos at scene been taken & ready to send back? VDR, CCTV & ECDIS been downloaded? 現場照片是否已拍攝並準備寄回？VDR、CCTV及ECDIS是否已下載? | ☐ |
| 5. | Have the risk assessment for overall emergency response & ship’s stability been effectively executed? 對所有緊急處置及船舶穩定性的風險評估是否已有效執行？ | ☐ |
| 6. | Have notified the nearest coasting country & flag state? 是否已通知沿岸最近管轄國及船旗國？ | ☐ |
| 7. | Have sent the following initial accident reports according to SMM PR-0802 in safety situation: 是否已照安全管理手冊PR-0802程序，在安全無虞下，寄回下列初步事件報告： | ☐ |
|  | - Statement of fact(in Chinese & English) with photos | ☐ |
|  | - Sea Protest | ☐ |
|  | - FM-0803-1, FM-0803-7 | ☐ |
|  | - Report to flag state | ☐ |
|  | - Tank & cargo hold sounding record | ☐ |
|  | - Copy of log book, bell book, telegraph, course recorder, weather chart… etc.. | ☐ |
| 8. | PIC continues closely contacting with ship’s captain & provides necessary emergent assistance until the fleet vessel is safe & finish repair & investigate case over. 海技PIC持續與船端緊密連絡並協助緊急處置，直到屬輪安全後。以及後續的維修與調查事件結束為止。 | ☐ |
| 9. | After PIC investigates the condition of scene, he must check person, thing, time, place, object if the same with record of ship side. 後續海技PIC至現場調查事件發生情況，須針對人事時地物是否與船上紀錄相同。 | ☐ |
| 10. | Interview with all persons related to the accident. 與事件相關人員的當面洽談。 | ☐ |
| 11. | The finished report must be approved by the company before sending out to the parties concerned like flag state. 相關報告製作完成後，在發給相關單位如船旗國前，須經公司主管批核。 | ☐ |