Emergency Response Checklist for Cargo Leakage (Gas/Liquid)

貨櫃洩漏(氣體/液體)應急處置檢查表

| Ship Side-Emergency Response船端-緊急處置 |  |  |
| --- | --- | --- |
| If knowing that a special container (gas/liquid) is about to load on ship, it is recommended to check this container’s condition before loading. After loading onboard, need to check it regularly as well. 如知道有特殊貨櫃即將上船(氣體/液體)，建議在該櫃上船前，就先對櫃況檢視，裝船後亦須對此櫃做巡檢。 |  |  |
| Checking Items檢查項目 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1 | After receiving message about cargo leakage, inform C/O immediately. 當班人員發現或接獲貨物洩漏通知時應立刻通知大副。 | ☐ |
| 2 | Record all the actions and timeline in detail. 詳實記錄所有事件的處理步驟與時間節點。 | ☐ |
| As soon as the C/O has been noticed 大副接獲通知後： |  |  |
| 3 | Check Stowage Plan for the container related information immediately. 立刻查看Stowage Plan該櫃的相關資訊: |  |
|  | - Using Stowage Plan to mark the leakage container’s position. 利用Stowage Plan標出『洩漏貨櫃』位置。 | ☐ |
|  | - Check it’s DG or not? 是否為危險櫃DG? | ☐ |
|  | - Report to Operation department to request the name of the goods and MSDS. 報告公司作業部，要求提供貨物名稱及MSDS。 | ☐ |
| 4 | Go to the scene to take photos and check about the following items: 到現場照相採證及了解以下情況: |  |
|  | - Take photos of leaking containers and photos of hulls or containers affected by leaking containers (if any). 『洩漏貨櫃』照片及『被洩漏物波及的船體或貨櫃』的照片(如有)。 | ☐ |
|  | - Are there any human injuries caused? If so, please refer to the crew serious injured checklist. 是否有造成人員受傷?如有請參考人員受傷檢查表。 | ☐ |
|  | - Did it cause a fire? If so, please refer to the container on fire checklist. 是否造成火災?如有請參考甲板貨櫃失火緊急事故處理檢查表。 | ☐ |
|  | - To determine whether it had damaged to other equipment or hulls? 判斷是否有影響其他設備或船體受損? | ☐ |
|  | - Set up an alert area at the scene to alert crew not to approach. 於現場設置警示區域以提醒船員注意，非必要時不要靠近。 | ☐ |
|  | - If it is DG, prepare equipment according to EMS, such as fire hose, dry powder fire extinguisher, CO2 fire extinguisher, breathing apparatus, chemical protective clothes and SOPEP equipment. 若為DG，則應按EMS準備應急設備，如: 備便皮龍帶、乾粉滅火器、二氧化碳滅火器、呼吸器、防化服等器材和SOPEP等裝備。 | ☐ |
|  | - If underway, check whether it is needed to adjust course and speed? 如為航行中，評估是否需要調整航向航速? | ☐ |
| Gas Cargo Leakage若洩漏物為氣體 |  |  |
| 5 | - Whether this gas leakage condition is a normal pressure release of the relief valve? 此氣體洩漏狀況是否為釋壓閥正常的壓力釋放? | ☐ |
|  | - What’s the color of the leaking gas? 洩漏氣體顏色? | ☐ |
|  | - On-site temperature? 現場溫度? | ☐ |
|  | - Ask DG DESK if the gas is flammable? Or toxic? 詢問DG DESK該氣體是否易燃?或有毒? | ☐ |
|  | - Ask about the relevant treatment principles, how to do emergency action? Or no urgent action is required? 詢問相關處理原則，如何做緊急處置?或不需做緊急處置? | ☐ |
|  | - If urgent action is required, should carry out a risk assessment of the safety. 如需緊急處置，需注意進行相關安全性的風險評估。 | ☐ |
|  | - Whether these leaking cargoes are still safe for transshipment? 評估是否仍可以安全續運? | ☐ |
| Liquid Cargo Leakage若洩漏物為液體 |  |  |
| 6 | - Whether the leaking cargo is marine pollutant? Whether it can be discharged into the sea? 洩漏物是否為海汙類，是否能排入海中? | ☐ |
|  | - If it is marine pollutant, please note that the nearby scupper drainage holes must be plugged and cannot be discharged into the sea. 如為海汙類須注意附近排水孔須塞起來，不能排入海。 | ☐ |
|  | - Will the leakage cause human injury? Hull erosion? 洩漏物是否會造成人員損傷?船體侵蝕? | ☐ |
|  | - Ask the shore side of the relevant treatment principles, how to do the emergency action? Or no urgent action is required? 詢問岸端相關處理原則，如何做緊急處置?或不需做緊急處置? | ☐ |
|  | - Does ship can clean on its own or require shore support? 船端可自行清潔處理或需要岸端支援? | ☐ |
|  | - If urgent action is required, must do a risk assessment for safety. 如需緊急處置，需注意進行相關安全性的風險評估。 | ☐ |
|  | - Whether it is still safe for transshipment? 評估是否仍可以安全續運? | ☐ |
| 7 | If the result of assessment requires onshore support or seriously affect to the ship's schedule, Master is required to send an ER. Refer to the "Emergency contact telephone list" and according to the safety management manual PR-08. Notify to Marine Technology Division, Operation Divisions, and Engineering Division etc. immediately for the related works arrangement. 如評估需要岸上支援或嚴重影響船期，船長需發送ER。並依安全管理手冊PR-08規定，參考“緊急聯絡電話表＂立即將情況通知海技部、作業部、工務部，以利後續安排卸岸及船期調度等岸端協助及支援。 | ☐ |
| After the emergency action, Master shall pay special attention to collect the evidence and make the reports as the followings: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 8 | As per SMM PR-08, any accident must do the required reports as follows: 依據公司安全管理手冊所規定之意外事故所規定表格繕製下列報告: |  |
|  | - Statement of Fact (Chinese and English). | ☐ |
|  | - Master Report. | ☐ |
|  | - FM-0803-6 Other accident/incident report意外事故報告 | ☐ |
|  | - FM-0803-7 Shipboard Accident Investigation Report意外事故調查報告 | ☐ |
|  | - Log book航行日誌 | ☐ |
|  | - Stowage Plan積載圖、DG Manifest、MSDS | ☐ |
|  | Keep relevant evidence to report to Company (it may be claim to cargo owner /ship operator due to additional costs incurred in cleaning / unloading the containers.) 保存相關紀錄呈報公司(因貨櫃清潔及卸下會產生額外費用後續可能會向貨主/操船方索賠)。 |  |
|  | - Photos of the contaminated areas of the deck and the associated decks. 甲板汙染面積及相關甲板受汙染照片。 | ☐ |
|  | - List of anti-fouling materials consumed on board. 船上消耗防汙材清單。 | ☐ |
|  | - Weather data, please receive additional satellite cloud images for the period around the incident happened. 氣象圖紙，請額外接收最近事發當時衛星雲圖。 | ☐ |
|  | - Leaked photos and crew cleaning and leak-blocking work photos. 洩漏照片及船員清潔及堵漏工作照。 | ☐ |
|  | - List of additional hours worked by the crew. 船員額外工時清單。 | ☐ |
| 9 | All external reports must be reviewed by Company before sending out. 所有對外報告在寄出去前，都須經過公司審核。 | ☐ |

| Shore Side-Emergency Response岸端緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| After receiving the report from the Captain, Maritech PIC immediately report to department head, VP & DP, and related departments like engine, OP & legal department for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報主管、協理及DP、法保部門、工務PIC及相關OP等部門，成立緊急應變小組。 |  | ☐ |
| 1 | PIC must confirm the followings as soon as receiving report from Master: PIC第一時間確認船端回報下列事項： |  |
|  | - The Gas/Liquid container leaked during navigation or at berth? 該氣體/液體貨櫃洩漏是在航行中或是靠泊發生? | ☐ |
|  | - Are there any human injures caused? 是否造成人員受傷? | ☐ |
|  | - Did it cause a fire? 是否造成起火? | ☐ |
|  | - Does it affect or damage to other equipment? 是否有影響其他設備或受損? | ☐ |
|  | - Are there any emergency actions required? Or no need urgent action? Or need the shore support? 是否需緊急處置?或不需做緊急處置?或需要岸端支援? | ☐ |
|  | - Before confirmed the name of the cargo and obtain the MSDS to know the nature of the cargo, remind crew don’t directly contact the leaked material, and prevent the leaked material entering the drain and bilge well. 提醒船員在確認貨物名稱及取得MSDS確認貨物性質前，人員誤直接接觸洩漏物、避免洩漏物流入排水口及汙水井。 | ☐ |
|  | - Remind Master to take photos of the scene and prepare to send it back. 提醒船長現場照片是否已拍攝並準備寄回。 | ☐ |
|  | - If the case was happened at port, whether it would affect the departure time? Estimate how many hours of delay? 若為靠泊發生，是否影響開航時間?大概Delay多久? | ☐ |
|  | - Whether Master has been reminded to inform the discharging port about the leaked cargo information. 是否已提醒船長告知卸貨港，洩漏貨物訊息。 | ☐ |
| 2 | Have the risk assessments for overall emergency response been effectively executed?對所有緊急處置的風險評估是否已有效執行？ | ☐ |
| 3 | Completed the initial report which required from SSM department, include following items: 完成海技部船管課事故初始報告，包含以下項目: |  |
|  | - Date發生日期。 | ☐ |
|  | - Time發生時間。 | ☐ |
|  | - Position船舶位置。 | ☐ |
|  | - Ship Status 船舶目前狀態。 | ☐ |
|  | - Description of the incident 事故狀況說明。 | ☐ |
|  | - Current handling progress and other department’s contact detail. 目前處理進度＆其他部門聯繫狀況說明。 | ☐ |
| 4 | Have sent the following initial accident reports according to SMM PR-0802 under safety condition: 是否已照安全管理手冊PR-0802程序，在安全無虞下，寄回下列事件報告： |  |
|  | - Statement of Fact (Chinese and English) | ☐ |
|  | - Master Report | ☐ |
|  | - FM-0803-6 Other accident/incident report意外事故報告 | ☐ |
|  | - FM-0803-7 Shipboard Accident Investigation Report意外事故調查報告 | ☐ |
|  | - Log book | ☐ |
|  | - Stowage Plan | ☐ |
|  | Keep relevant evidence to report to Company (it may be claim to cargo owner /ship operator due to additional costs incurred in cleaning / unloading the containers.) 保存相關紀錄呈報公司(因貨櫃清潔及卸下會產生額外費用後續可能會向貨主/操作方索賠)。 |  |
|  | - Photos of the contaminated areas of the deck and the associated decks. 甲板汙染面積及相關甲板受汙染照片。 | ☐ |
|  | - List of anti-fouling materials consumed on board. 船上消耗防汙材清單。 | ☐ |
|  | - Weather data, please receive additional satellite cloud images for period around the incident happened. 氣象圖紙，請額外接收最近事發當時衛星雲圖。 | ☐ |
|  | - Leaked photos and crew cleaning and leak-blocking work photos. 洩漏照片及船員清潔及堵漏工作照。 | ☐ |
|  | - List of additional hours worked by the crew. 船員額外工時清單。 | ☐ |
| 5 | PIC continues closely contacting with ship’s Master & provides necessary emergent assistance until the leakage cargo was discharge in safe & the case was completed. 海技PIC持續與船端緊密連絡並協助緊急處置，直到該櫃安全卸岸及後續處理結束為止。 | ☐ |
|  | If the hull was fouled due to liquid cargo leakage, the additional cleaning costs incurred must be prepared as follows: 若為液體洩漏造成船體汙損，需準備額外產生的清潔費用資料如下: - List of additional hours worked by the crew. 船員工時成本。 - Consumption of cleaning materials costs. 消耗清潔物料費用。 - The cost of unloading oily clean waste. 含油清潔廢棄物卸岸處理費用。 | ☐ |
| 6 | The finished report must be approved by the Company before sending out to the parties concerned (such as flag state). 相關報告製作完成後，在發給相關單位前(如船旗國)，須經公司主管批核。 | ☐ |