IMDG Container Emergency checklist

危險貨櫃事故緊急處理流程表

| Ship Side-Emergency Response(IMDG Container on fire) 船端-緊急處置(危險貨櫃失火) |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1. | Sound general alarm & Muster fire-fighting station and call Master immediately. 立即鳴放火警警報並廣播全船就滅火佈署，通知船長。 | ☐ |
| 2. | Check where the fire start and fire/smoke spreading direction on bridge. If necessary, turn to windward side first to avoid endangering the accommodation. 檢查起火地點與觀察火勢/煙霧是否向駕駛台擴散，必要時先轉向下風，避免危害住艙。 | ☐ |
| 3. | Check if any other traffic or immediate danger in the vicinity. 確認附近是否有其他船隻與立即危險。 | ☐ |
| 4. | Check chart depth & wind/current direction & other information around the ship. 檢查海圖上附近區域之水深、物件與其他必要資訊。 | ☐ |
| 5. | Notify E/R to stand by engine and start emergency firefighting procedure. 通知機艙備車與啟動緊急滅火程序。 | ☐ |
| 6. | Broadcast urgent or safety emergency or safety massage to all vessels by VHF 16 or AIS. 廣播發送VHF，或AIS緊急或安全信文給海面其他船隻。 | ☐ |
| 7. | Record the time & ship position on chart. 記錄當時時間及船位，並標示在海圖上。 | ☐ |
| As soon as the Master on bridge 船長上駕駛台後： |  |  |
| 1. | Check where leeside and sea room is available for stopping and fire-fighting? 檢查是否有下風的水域，適合停車與滅火部署? | ☐ |
| 2. | Alter Co. & reduce speed to put container on fire at leeside to avoid crew accommodation, engine room and life-saving appliances been affected. Close all watertight doors immediately. 轉向並減速，將燃燒櫃放在下風側，關閉所有對外水密門，以免船員住艙、機艙和救生設備受到影響。 | ☐ |
| 3. | Check if the container on fire and nearby area has DG cargo to confirm its characteristics & precautions according to IMDG code & MSDS. 檢查燃燒櫃與附近，是否有其他危險貨，依IMDG code及物質安全資料表，確定該貨櫃性質與附近的貨物特性，及注意防範事項。 | ☐ |
| 4. | Assign fire-fighting team with fireman outfit & equipments (CABA) to fire scene from upwind side to cool and evaluate situation. 廣播並指派滅火隊，穿著防火衣與攜帶必要滅火裝備(呼吸器)，從上風接近現場，評估情勢與冷卻火場。 | ☐ |
| 5. | Decide shipboard personnel and equipments and fire pump capacity is enough to put out the fire or just to contend it spreading. 決定船上人員和設備以及消防泵的容量，足以撲滅火災，還是僅僅防範火勢蔓延。 | ☐ |
| 6. | Notify Maritech Division immediately according to “Emergency Contact List” and send ER & photos for necessary assistance & support. 參考“緊急聯絡電話表＂立即將情況，電話報告海技部及發ER及照片，以利提供適時的協助及支援。 | ☐ |
| 7. | (IF NECESSARY) At early stage of the emergency, Master shall notify costal state or flag state, the notification shall include the type of the accident, personal injury and assistance required. (需要的情況下)緊急事件初期，船長應通知最近的沿岸國及船旗國當局，通報內容包括事件類型，人員傷亡及需要何種協助。 | ☐ |
| 8. | Arrange Fire-fighting team and supporting team in turn of 6 hours work and rest shift if fire is contended which need many days hard works. If Company vessel passing by may provide extra man power to assist by company’s order. 如遇火勢需多日困難撲滅，安排消防隊和支援隊輪班，輪流間隔6小時作息。若公司的經過船隻，經由公司安排，可以提供額外的人力，來協助滅火。 | ☐ |
| 9. | If fire becomes serious & out of control Captain, after careful assessment & discuss with C/E & C/O, need to abandon ship. He should inform company PIC of this situation and send the “distress signal” to outside parties first, announce/broadcasting “abandon ship”, summon all the crew for roll call, preserve all legal documents and evidence like VDR, CCTV, Logbook, certificates, bell book and fire-fighting notebook, etc…. 如果火勢嚴重失控，船長與輪機長與大副仔細評估和討論後，決定棄船。應先通報公司，並向外界發出“求救信號”，宣布/廣播“棄船”，召集全體船員進行點名，保存所有法律文件和證據，如VDR、CCTV、硬碟航海日誌、證書、俥鐘簿和消防筆記本等。 | ☐ |
| Before fire-fighting, execute risk assessment & ensure the following by C/O & Captain: 大副滅火前，執行風險評估並請船長共同確認下列事項： |  |  |
|  | - Make sure all crew had been roll called already? Had any crew not been informed yet? 確保所有船員都已經點名到齊?是否有任何船員，還沒有得到通知? | ☐ |
|  | - Does fire-fighting team member already deploy according to the fire-fighting muster list with fireman outfit & appropriate fire-fighting equipments? 是否滅火隊已照滅火部署表，穿著適當防火衣及攜帶所有相關滅火裝備。 | ☐ |
|  | - Briefing to the fire-fighting team the procedure & precautions of fired containers. 已對滅火隊員做簡報，告知滅火步驟及該貨櫃滅火的注意事項。 | ☐ |
|  | - Inform master to alter course & reduce speed to minimize the fire and smoke spreading. 通知駕駛台轉向及減速到適當位置，以便控制火勢。 | ☐ |
|  | - Fire-fighting team should proceed to fire scene from upwind side and keep safe distance from fired container at safe place to protect themselves. 滅火隊前往起火地點與在滅火過程中，要保持在起火櫃的上風，立於保護自身安全的位置。 | ☐ |
|  | - Cool down the containers on fired & adjacent containers with fixed Firefighting Monitors spray and mist if equipped. 以消防水柱(消防水炮)與水霧，減低燃燒櫃及其周遭櫃的熱量以加強隔離效果，防止火勢蔓延。 | ☐ |
|  | - Check the use of container water mist lancer (if equipped) safety by cargo property and chemical compositions with Dangerous Cargo List or MSDS. 依照危險品艙單檢查貨物性質和可能的化學反應，檢視貨櫃內部滅火槍(如有配置)的使用安全。 | ☐ |
|  | - Try to Isolate fire area with maximum fire water spray or water monitor if equipped. This intention may have to carry out in leeside of the fire, if crew safety can be assured. 儘量將燃燒區域隔絕出來，利用最大量的消防水柱(消防水砲)長期冷卻。隔絕行動可能需要從火勢的下風處進行，如果船員安全可以確保。 | ☐ |
|  | - Calculate and access Ship’s stress force and stability safety. 執行船體應力與穩定度的安全性評估。 | ☐ |
|  | - Check if cargo hold flooded with massive fire-fighting water which need to pump out immediately. 檢查貨艙是否被大量消防水淹沒，需要立即抽出。 | ☐ |
|  | - Check if cargo hold flooded oily water which could spill overboard and activate Oily Water Separator or SOPEP at once. 確認大艙是否灌入油水或貨櫃有漏洩的油污染，如有，立即啟動油水分離器或SOPEP程序。 | ☐ |
|  | - Check any crew injured need to repatriate or first aid (CPR) help on scene? 確認本輪人員是否有受傷?有無需要執行後送，或現場急救(包括CPR)。 | ☐ |
|  | - Calculate water ingress and stability deteriorate situation before further actions. 在進一步行動之前，評估進水和穩定性惡化的情況。 | ☐ |
|  | - Adjust Trim, draught of the ship to gather bilge water to pump out if necessary 必要情況下，調整俯仰差、減低船舶吃水，以打出汙水。 | ☐ |
|  | - At Captain’s discretion to preserve human life and ship in emergency, oily water may pump out if necessary. MARPOL reg. 4. 船長為保全人命和船舶，在緊急情況下得酌情決定，必要時，可排出含油污水。 | ☐ |
|  | - Remark progress of the fire-fighting in hourly Record & report to the company. 每小時記錄滅火進度並報告公司。 | ☐ |
|  | - Continuously contact with the company & agent of next port for necessary assistance & arrangement (i.e. shore fire-fighting brigade or shelter anchorage). 持續與公司及下一港的代理，保持連絡，以獲得必要的援助及安排(如岸上救火隊或避難錨地)。 | ☐ |

| Once oil leakage overboard若油污漏洩入海 |  |  |
| --- | --- | --- |
| 1. | Any oil spill due to the breakage of the tank, stop the pollution according to SMM PR-08 Emergency Plan: Pollution Prevention; and prevent fire and explosion from oil spill; any spill overboard, report to parties concerned as per SOPEP’s procedure and format. 有油料因貨艙破損溢出時，依照安全管理手冊PR-08 Emergency Plan: Pollution Prevention進行油污染防阻，並避免因溢油產生之爆炸與火災；若有任何油污漏洩入海，須照船上油污染程序，通報有關單位。 | ☐ |
| 2. | Appraise the extent of the damage and block the leaking immediately, confirm other tanks watertight to avoid the damage spreading; 依據船舶油污染應急計畫(SOPEP)，評估損害程度，並指揮損害管制組進行堵漏、確保其他艙間水密性，以防止損害擴大。 | ☐ |
| 3. | Make risk assessment for oil spill overboard & confirm the oil pollution condition and take photos. Muster crew for the oil pollution control. 執行油污漏洩入海風險評估，確認油污污染狀況，並拍攝現場照片。召集船員進行防油污污染部署。 | ☐ |
| 4. | Dispatch an initial oil spills report to TPE / Maritech & local related Dept. 發送初步油汙汙染報告至海技部，及與當地相關單位。 | ☐ |
| 5. | Company PIC Inform P & I Club and Hull insurance company to arrange oil pollution clear operation. 法保PIC通知相關保險公司，準備清理油汙汙染作業。 | ☐ |
| 6. | Inform Agent to arrange for any emergency assistance. 通知代理，準備任何緊急情況之協助。 | ☐ |
| After the emergency action, Master shall collect the evidences and prepare following reports: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 1. | As per SMM PR-08, any accident, the required report forms are, FM-0803-1, FM-0803-7, and statement of fact & sea protest. 依據公司安全管理手冊所規定之意外事故，規定表格繕製意外事故報告(FM-0803-1)、意外事故調查報告(FM-0803-7)以及事實陳述、海事報告呈報公司； | ☐ |
| 2. | Preserve VDR’s, CCTV recording (as VDR will continue recording for 12 hours, Master should decide when to stop the recording for later investigation and replace/rewrite to a new CF Card to continue recording) 妥善保存VDR、CCTV之記錄(因VDR只能連續12紀錄小時，船長決定何時將記錄器停止，並更換新的記憶卡或轉錄他卡，以供後續調查使用)。 | ☐ |
| 3. | Statement of fact shall attach photos as many as possible, description of fire incident shall include all witnesses and duty persons oral statement to identify real cause of incident and prevent re-occurrence afterward. 事實陳述為對公司內部之火災事故過程之描述(不對外公開)，應盡可能附上照片，火災事件的描述應包括所有目擊者和值班人員的口頭陳述，以查明事故的真正原因，防止日後事件再次發生。 | ☐ |
|  | - Fire containers position/container No./contents. 起火貨櫃的位置/櫃號/貨物內容。 | ☐ |
|  | - Ship’s position/course/speed/rpm and course recorder reading at time. 當時之船位/航向/航速/轉速及當時航向記錄器所顯示之記錄； | ☐ |
|  | - Engine/steering maneuvering/time and condition. 當時的用車/用舵/時間及狀況； | ☐ |
|  | - Weather/ wind direction/ speed and wave/ swell height/direction and height/visibility/received weather forecast; 當時天氣狀況/風向、風速/浪之方向與高度/湧之方向與高度/能見度/收受之天氣預報。 | ☐ |
|  | - Ships recording of response to fire (including operation of M/E); 發現起火時，船方反應動作之記錄(含主機操作)； | ☐ |
|  | - The damage condition of ship and cargo on both ships, with attached photos, if possible; 有船貨損壞情況，儘可能附照片。 | ☐ |
| 3. | Master’s sea protest should include the incident time, nature, place and any personal injury or disease only which shall be official endorsed by next port of call. Master should not include any speculation and conclusion of said incident in Sea Protest. 船長對外之海事報告(Sea Protest)，應僅包括事件時間、性質、地點和任何人身傷害或死亡，不應對事件原因與後果，妄加推測，並應由官署予以簽證。 | ☐ |
| 4. | Salvage Superintendent or Salvage Surveyor should assign by company or P&I Club. Beware of VHF communication with outside parties, don’t make any commitment to any proposal without company instructions. 救助負責人或檢查員，應由船東或船東互保協會指派，船長除非警急處置，不得自行同意，避免被VHF錄音詐騙，不可擅自隨口應承。 | ☐ |
| 5. | If company arranged surveyor for damage inspection, Master shall prepare related drawing; 如有公司安排的驗船師登輪作損壞檢定，船長應備妥全船相關船圖供參考。 | ☐ |
|  | - CLASS SURVEYOR: to assess ship’s seaworthiness, arranged by the Company. 船級協會驗船師：以判定船體適航性、由公司通知安排。 | ☐ |
|  | - INSURANCE SURVEYOR: to evaluate the damage, hull or P & I surveyor; not liability assessment. 船體驗船師作損傷鑑定、可分船體險及船東協會驗船師、不作任何責任判斷用。 | ☐ |
|  | - Lawyer: Master should identify his ID and which parties he represented? Master should not allow them come on board without prior consent from company. 律師：船長應該查明他的身份，他代表哪些當事人?未經公司事先同意，船長不應允許他們上船。 | ☐ |
|  | - Report to flag of country in 24 HR. 事發24小時內向船旗國回報。 | ☐ |
|  | - All external report must check by company. 所有對外報告內容都須經過公司審議。 | ☐ |
|  | - All relevant document and records should not hand to 3rd party without company’s prior consent. 未經公司事先同意，不得將所有相關文件和記錄交給第三者。 | ☐ |

| Ship Side-Emergency Response(IMDG Container Leakage) 船端-緊急處置(危險貨櫃洩漏) |  |  |
| --- | --- | --- |
| If knowing that a IMDG container (gas/liquid) is about to load on ship, it is recommended to check this container’s condition before loading. After loading onboard, need to check it regularly as well. 如知道有危險貨櫃即將上船(氣體/液體)，建議在該櫃上船前，就先對櫃況檢視，裝船後亦須對此櫃做巡檢。 |  |  |
| Checking Items檢查項目 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1 | After receiving message about cargo leakage, inform C/O immediately. 當班人員發現或接獲貨物洩漏通知時應立刻通知大副。 | ☐ |
| 2 | Record all the actions and timeline in detail. 詳實記錄所有事件的處理步驟與時間節點。 | ☐ |
| As soon as the C/O has been noticed 大副接獲通知後： |  |  |
| 3 | Check Stowage Plan for the container related information immediately. 立刻查看Stowage Plan該櫃的相關資訊: |  |
|  | - Using Stowage Plan to mark the leakage container’s position. 利用Stowage Plan標出『洩漏貨櫃』位置。 | ☐ |
|  | - Report to Operation department to request the name of the goods and MSDS. 報告公司作業部，要求提供貨物名稱及MSDS。 | ☐ |
|  |  | ☐ |
| 4 | Go to the scene to take photos and check about the following items: 到現場照相採證及了解以下情況: |  |
|  | - Take photos of leaking containers and photos of hulls or containers affected by leaking containers (if any). 『洩漏貨櫃』照片及『被洩漏物波及的船體或貨櫃』的照片(如有)。 | ☐ |
|  | - Are there any human injuries caused? If so, please refer to the crew serious injured checklist. 是否有造成人員受傷?如有請參考人員受傷檢查表。 | ☐ |
|  | - Did it cause a fire? If so, please refer to the container on fire checklist. 是否造成火災?如有請參考甲板貨櫃失火緊急事故處理檢查表。 | ☐ |
|  | - To determine whether it had damaged to other equipment or hulls? 判斷是否有影響其他設備或船體受損? | ☐ |
|  | - Set up an alert area at the scene to alert crew not to approach. 於現場設置警示區域以提醒船員注意，非必要時不要靠近。 | ☐ |
|  | - If it is DG, prepare equipment according to EMS, such as fire hose, dry powder fire extinguisher, CO2 fire extinguisher, breathing apparatus, chemical protective clothes and SOPEP equipment. 若為DG，則應按EMS準備應急設備，如: 備便皮龍帶、乾粉滅火器、二氧化碳滅火器、呼吸器、防化服等器材和SOPEP等裝備。 | ☐ |
|  | - If underway, check whether it is needed to adjust course and speed? 如為航行中，評估是否需要調整航向航速? | ☐ |
| Gas Cargo Leakage若洩漏物為氣體 |  |  |
| 5 | - Whether this gas leakage condition is a normal pressure release of the relief valve? 此氣體洩漏狀況是否為釋壓閥正常的壓力釋放? | ☐ |
|  | - What’s the color of the leaking gas? 洩漏氣體顏色? | ☐ |
|  | - On-site temperature? 現場溫度? | ☐ |
|  | - Ask DG DESK if the gas is flammable? Or toxic? 詢問DG DESK該氣體是否易燃?或有毒? | ☐ |
|  | - Ask about the relevant treatment principles, how to do emergency action? Or no urgent action is required? 詢問相關處理原則，如何做緊急處置?或不需做緊急處置? | ☐ |
|  | - If urgent action is required, should carry out a risk assessment of the safety. 如需緊急處置，需注意進行相關安全性的風險評估。 | ☐ |
|  | - Whether these leaking cargoes are still safe for transshipment? 評估是否仍可以安全續運? | ☐ |
| Liquid Cargo Leakage若洩漏物為液體 |  |  |
| 6 | - Whether the leaking cargo is marine pollutant? Whether it can be discharged into the sea? 洩漏物是否為海汙類，是否能排入海中? | ☐ |
|  | - If it is marine pollutant, please note that the nearby scupper drainage holes must be plugged and cannot be discharged into the sea. 如為海汙類須注意附近排水孔須塞起來，不能排入海。 | ☐ |
|  | - Will the leakage cause human injury? Hull erosion? 洩漏物是否會造成人員損傷?船體侵蝕? | ☐ |
|  | - Ask the shore side of the relevant treatment principles, how to do the emergency action? Or no urgent action is required? 詢問岸端相關處理原則，如何做緊急處置?或不需做緊急處置? | ☐ |
|  | - Does ship can clean on its own or require shore support? 船端可自行清潔處理或需要岸端支援? | ☐ |
|  | - If urgent action is required, must do a risk assessment for safety. 如需緊急處置，需注意進行相關安全性的風險評估。 | ☐ |
|  | - Whether it is still safe for transshipment? 評估是否仍可以安全續運? | ☐ |
| 7 | If the result of assessment requires onshore support or seriously affect to the ship's schedule, Master is required to send an ER. Refer to the "Emergency contact telephone list" and according to the safety management manual PR-08. Notify to Marine Technology Division, Operation Divisions, and Engineering Division etc. immediately for the related works arrangement. 如評估需要岸上支援或嚴重影響船期，船長需發送ER。並依安全管理手冊PR-08規定，參考“緊急聯絡電話表＂立即將情況通知海技部、作業部、工務部，以利後續安排卸岸及船期調度等岸端協助及支援。 | ☐ |
| After the emergency action, Master shall pay special attention to collect the evidence and make the reports as the followings: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 8 | As per SMM PR-08, any accident must do the required reports as follows: 依據公司安全管理手冊所規定之意外事故所規定表格繕製下列報告: |  |
|  | - Statement of Fact (Chinese and English). | ☐ |
|  | - Master Report. | ☐ |
|  | - FM-0803-6 Other accident/incident report意外事故報告 | ☐ |
|  | - FM-0803-7 Shipboard Accident Investigation Report意外事故調查報告 | ☐ |
|  | - Log book航行日誌 | ☐ |
|  | - Stowage Plan積載圖、DG Manifest、MSDS | ☐ |
|  | Keep relevant evidence to report to Company (it may be claim to cargo owner /ship operator due to additional costs incurred in cleaning / unloading the containers.) 保存相關紀錄呈報公司(因貨櫃清潔及卸下會產生額外費用後續可能會向貨主/操船方索賠)。 |  |
|  | - Photos of the contaminated areas of the deck and the associated decks. 甲板汙染面積及相關甲板受汙染照片。 | ☐ |
|  | - List of anti-fouling materials consumed on board. 船上消耗防汙材清單。 | ☐ |
|  | - Weather data, please receive additional satellite cloud images for the period around the incident happened. 氣象圖紙，請額外接收最近事發當時衛星雲圖。 | ☐ |
|  | - Leaked photos and crew cleaning and leak-blocking work photos. 洩漏照片及船員清潔及堵漏工作照。 | ☐ |
|  | - List of additional hours worked by the crew. 船員額外工時清單。 | ☐ |
| 9 | All external reports must be reviewed by Company before sending out. 所有對外報告在寄出去前，都須經過公司審核。 | ☐ |