Emergency Response Checklist for the vessel damage by gantry crane

碼頭橋式機操作不當造成船舶設備受損應急處置檢查表

| While berthing, duty crew should always pay attention to the clearance of height between the gantry crane and the top of bridge are sufficient or not? If the clearance distance was not enough, the duty crew also didn’t notice it or report to the duty officer at the first time, when gantry crane moved through the bridge without raised its boom, which would cause damage to the INM-C antenna, main mast, fore mast and hand rails etc. Improper operation could also cause damage to other equipment/hulls (such as container cell guide etc.). 靠泊時，當值人員應隨時注意橋式機吊臂高度與駕駛台淨空高度是否足夠。如間距不夠，當橋式機移動通過駕駛台，又未將其吊臂舉起，當值巡邏班亦未立即發現並或報告當班的船副以糾正，將導致觸碰INM-C天線或其他羅經甲板設備、主桅與前桅、欄杆等。操作不當也會造成其他設備/船體(如貨櫃導槽等)損毀。 |  |  |
| --- | --- | --- |
| Ship Side-Emergency Response船端-緊急處置 |  |  |
| Checking Items檢查項目 |  |  |
| Duty officer shall: 當值船副應： |  |  |
| 1 | After receiving message about the damage, inform C/O, foreman and tally immediately. 當班人員發現或接獲通知時應立刻通知大副、碼頭現場工頭及理貨。 | ☐ |
| 2 | Record all the actions and timeline in detail. 詳實記錄所有事件的處理步驟與時間節點。 | ☐ |
| As soon as the C/O been noticed 大副接獲通知後： |  |  |
| 3 | Go to the scene immediately to take photos for evidence and confirm below items: 立刻到現場照相採證及確認以下情況: |  |
|  | - Are there any human injuries caused? If so, please refer to the crew serious injured checklist. 是否造成人員受傷?如有請參考人員受傷檢查表。 | ☐ |
|  | - Did it cause a fire? 是否造成起火? | ☐ |
|  | - Whether it had damaged to other equipment? 是否有影響其他設備或受損? | ☐ |
|  | - Whether it needs to be repaired immediately? 是否需即刻進行修理? | ☐ |
|  | - Can it be repaired by crew or need shore support? 船端可自行修復或需要岸端支援? | ☐ |
|  | - Does it affect to the sailing time? Estimated delay hours? 是否影響開航時間?推估Delay時間? | ☐ |
|  | - Whether it affects seaworthiness? 是否影響適航性? | ☐ |
|  | - What’s the ship's draft / air draft / tidal range at the time of the incident? 事發當時船舶吃水/淨空高度/潮差? | ☐ |
| 4 | Collect all the information & notify to the Master, inform the foreman to make FM-0704-2 STEVEDORE (other than container) DAMAGE REPORT, sign it for compensation. 彙整通報船長並知會工頭並製作FM-0704-2 STEVEDORE (other than container) DAMAGE REPORT找工頭簽字，以利求償。 | ☐ |
| 5 | If the result of assessment requires onshore support or seriously affect to the ship's schedule, Master is required to send an ER. Refer to the "Emergency contact telephone list" and according to the safety management manual PR-08. Notify to Marine Technology Division, Operation Divisions, and Engineering Division immediately for the related works arrangement. 如評估受損嚴重需要岸上支援並嚴重影響船期，船長需發送ER。並依安全管理手冊PR-08規定，參考“緊急聯絡電話表＂立即將情況通知海技部、工務部、作業部等，以利後續安排廠商維修及船期調度等岸端協助及支援。 | ☐ |
| 6 | During repair works, must pay attention to the safety related risk assessment. 修繕作業時需注意進行安全性相關的風險評估。 | ☐ |
| After the emergency action, Master shall pay special attention to collect the evidence and make the reports as the followings: 緊急處理後，船長應注意收集之証據及繕製之報告如下: |  |  |
| 7 | As per SMM PR-08, any accident must do the required reports as follows: 依據公司安全管理手冊所規定之意外事故所規定表格繕製下列報告: |  |
|  | - Statement of fact (Chinese and English). | ☐ |
|  | - Claim letter. | ☐ |
|  | - Sea Protest. | ☐ |
|  | - FM-0704-2 Stevedore (other than container) damage report. | ☐ |
|  | - FM-0803-6 Other accident/incident report意外事故報告 | ☐ |
|  | - FM-0803-7 Shipboard Accident Investigation Report意外事故調查報告 | ☐ |
|  | - Flag State report / CCTV / Photo / Other其他資料如CCTV&照片 | ☐ |
| 8 | All external reports must be reviewed by Company before sending out. 所有對外報告在寄出去前，都須經過公司審核。 | ☐ |
| 9 | If repairs were arranged, fitness for sailing will be impaired, seaworthiness certificate shall be verified and secured by Class to issue “fit to proceed” before sailing to sea. 若有安排修理，適航性可能受損，在未獲得SEAWORTHINESS CERTIFICATE前移動航行將失去保險之COVER，故一定要取得適航証明(由船級協會CLASS SOCIETY SURVEYOR簽發)，完成FIT TO PROCEED如此才能確保保險有效。 | ☐ |

| Shore Side-Emergency Response岸端緊急處置 |  |  |
| --- | --- | --- |
| Checking Items檢查項目 |  |  |
| After receiving the report from the Master, Maritech PIC immediately report to department head, division head & DP, and related departments like Engineering, OP, legal department & local Rep (if any) for their follow-up, establish emergency response team. 海技PIC接獲船端報告，立即通報課主管、部門主管及DP、工務PIC、相關OP、法保部門、及當地代表(如有)等部門，成立緊急應變小組。 |  | ☐ |
| 1 | PIC must confirm the followings as soon as receiving report from Master: PIC第一時間確認船端回報下列事項： |  |
|  | - Are there any human injuries caused? If so, please refer to the crew serious injured checklist. 是否造成人員受傷? 如有請參考人員受傷檢查表。 | ☐ |
|  | - Did it cause a fire? 是否造成起火? | ☐ |
|  | - Whether it had damaged to other equipment? 是否有影響其他設備或受損? | ☐ |
|  | - Whether it needs to be repaired immediately? 是否需即刻進行修理? | ☐ |
|  | - Can it be repaired by crew or need shore support? 船端可自行修復或需要岸端支援? | ☐ |
|  | - Does it affect to the sailing time? Estimated delay hours? 是否影響開航時間?大概Delay多久? | ☐ |
|  | - Whether it affects seaworthiness? Does it need to arrange class survey? 是否影響適航性?安排公證檢查? | ☐ |
|  | - What the ship's draft / air draft / tidal range at the time of the incident? 事發當時船舶吃水/淨空高度/潮差? | ☐ |
|  | - Remind Master to take photos / CCTV of the scene and prepare to send it back. 提醒船長拍攝現場照片/CCTV並準備寄回。 | ☐ |
| 2 | Have the risk assessments for overall emergency response been effectively executed? 對所有緊急處置的風險評估是否已有效執行？ | ☐ |
| 3 | Completed the initial report which required from SSM department, include following items: 完成海技部船管課事故初始報告，包含以下項目: |  |
|  | - Date發生日期。 | ☐ |
|  | - Time發生時間。 | ☐ |
|  | - Position船舶位置。 | ☐ |
|  | - Ship Status船舶目前狀態。 | ☐ |
|  | - Description of the incident事故狀況說明。 | ☐ |
|  | - Current handling progress and other department’s contact detail. 目前處理進度&其他部門聯繫狀況說明。 | ☐ |
| 4 | Have sent the following initial accident reports according to SMM PR-0802 under safety condition: 是否已照安全管理手冊PR-0802程序，在安全無虞下，寄回下列事件報告： |  |
|  | - Statement of fact(Chinese and English) | ☐ |
|  | - Claim letter | ☐ |
|  | - Sea Protest | ☐ |
|  | - FM-0704-2 Stevedore (other than container) damage report | ☐ |
|  | - FM-0803-6 Other accident/incident report意外事故報告 | ☐ |
|  | - FM-0803-7 Shipboard Accident Investigation Report意外事故調查報告 | ☐ |
| 5 | PIC continues closely contacting with ship’s Master & provides necessary emergent assistance until the vessel departure in safe & the subsequence report and investigation was completed. 海技PIC持續與船端緊密連絡並協助緊急處置，直到屬輪離港及後續的維修與調查事件結束為止。 | ☐ |
| 6 | The finished report must be approved by the Company before sending out to the parties concerned (such as flag state). 相關報告製作完成後，在發給相關單位如船旗國前，須經公司主管批核。 | ☐ |